TECHNICAL DESIGN EXTRACTION NOTE

ENERPRO General Purpose 3ϕ Firing Circuit

Drawing No.: SD-237-230-12-R0  │  PCB PN: FC0G6100  │  SLAC / Stanford University

Version 2 – Full Detail including Wire Nets, Signal Buses, and ASCII Schematic

1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-237-230-12-R0 |
| PCB Part Number | FC0G6100 |
| Internal Drawing No. | E128 |
| Title | Schematic Diagram – ENERPRO General Purpose 3 Phase Firing Circuit, PCB PN FC0G6100 |
| Organization | Stanford Linear Accelerator Center (SLAC), U.S. Department of Energy, Stanford University |
| Location | Stanford, California |
| Sheet | 1 of 1 |
| Scale | Do Not Scale Drawing |
| Drawing Class | Schematic Diagram |
| Size | F (large format, landscape) |
| Drawn | F:B (2-12-85) |
| Released CA | 5-27-86 |
| Checked | W.C. Mitchell 9/29/87 |
| ENGR approval | M.B. Muet 11/30/87 |

1.1 Revision History

| Rev | Appd | Date | Description |
| --- | --- | --- | --- |
| 1A (H) | fjb | 10-17-86 | ADD CP1 AND CP2 |
| 1B (I) | fjb | 10-27-86 | ADD 50/60 HZ JACK, J10 |

2. Complete Bill of Materials (Full Corrected)

All components verified by direct image analysis of schematic parts list, cross-referenced against on-schematic designator labels.

2.1 Integrated Circuits

| Ref Des | Part Number | Description / Notes |
| --- | --- | --- |
| U1 | YE28 | IC (power/misc function, lower-right board area) |
| U2 | LM78L12ACH | Positive 12V linear voltage regulator |
| U3 | ULN2004A | 7-channel Darlington transistor array (gate pulse driver) |
| U4 | EP1011 | IC (auxiliary logic) |
| U5 | LM339N | Quad voltage comparator (phase loss & sequence detection) |
| U6-U7 | LM324N | Quad op-amp (buffer amp and phase reference stages) |
| U8 | MC14053BCP | Triple 2-channel analog multiplexer (phase switch) |
| U9 | MC14077BCP | Quad XNOR gate array (firing pulse logic) |

2.2 Diode Networks & Special Semiconductors

| Ref Des | Part Number | Description |
| --- | --- | --- |
| DN1 | MAD1109P | Diode network (14-pin DIP) |
| CR1 | (rectifier) | Diode bridge for power supply |
| CR2 | 11V zener | 11V zener diode (in U6 phase sequence circuit) |
| CR1/CR2 (BOM) | 1N4746A(18V) / 1N5241B(11V) | Zener diodes per BOM line item |
| PD1 | (see note) | Photodiode / opto detector, with PO1 pot |
| PD2 | (trimmer) | Potentiometer / adjustment point for span |
| PD3 | (trimmer) | Potentiometer / adjustment point for bias |
| PPi | 435704-8 | Phase pulse injection component |

2.3 Optocouplers / Pulse Transformers

| Ref Des | Part Number | Description |
| --- | --- | --- |
| CP1-CP2 | PCB-5 | Pulse coupler boards (added Rev 1A) |
| PM1-PM6 | EP1019 | Pulse transformer modules (6 total, gate drive) |

2.4 Resistor Networks (SIP/DIP Packages)

| Ref Des | Part Number | Value/Description |
| --- | --- | --- |
| RN1 | MDP14031530 | Matched resistor network, phase reference scaling |
| RN2 | MSP08A031240 | 120K resistor network, 8-pin SIP |
| RN3 | MSP08A034730 | 47K resistor network, 8-pin SIP |
| RN4 | MSP08A011040 | 100K resistor network, 8-pin SIP |
| RN5 | MSP08A031520 (17) | 1.5K resistor network, 8-pin SIP (Ref 17: change to 6-pin SIP) |
| RN6 | MSP08A031540 | 150K resistor network, 8-pin SIP |

2.5 Potentiometers

| Ref Des | Part Number | Description |
| --- | --- | --- |
| PO1-PO2 | 550-2404 (RED) | Potentiometer, 240Ω trim |
| PO3 | 550-2204 (GRN) | Potentiometer, 220Ω trim |

2.6 Connectors

| Ref Des | Part Number | Description |
| --- | --- | --- |
| J1-J2 / P1-P2 | 640588-1/640586-1 | 8-pin connector, gate drive output (line-to-load / load-to-line) |
| J3/P3 | 350433-1/1-480708-0 | Connector, auxiliary signals |
| J4/P4 | 640466/1-480763-0 | Connector, TI excitation |
| J5/P5 | 1-350949-0/1-480763-0 | Socket, 24-18AWG, off-board phase reference input |
| P1-P5 SOCKETS | 350689-3 (24-18AWG) | PCB-mount sockets for P1-P5 |
| J6/P6 | 103311-5/499588-4 | Multi-pin connector, 3-phase input + control signals (18+ pins) |
| J7/P7 | 640454-3/640440-3 | Connector, test reference input |
| J8/P8 | 640456-8/640440-8 | Connector, A-bridge auxiliary firing bus |
| J9/P9 | 640456-8/640440-8 | Connector, B-bridge auxiliary firing bus |
| J10/P10 | 640454-3/65474-001 | Connector, 50/60 Hz select jack (added Rev 1B) |

2.7 Capacitors

| Ref Des | Value | Part Number / Notes |
| --- | --- | --- |
| C1 | 470μF 50V | ECEA1HV471S (power supply filter) |
| C2 | 470μF 50V | ECEA1HV471S (power supply filter) |
| C3 | 22μF 16V | ECSF16E22 |
| C4 | 2.2μF 16V | ECSF16E2R2 |
| C11-C13 | MKC4 10% 63V | Phase snubber capacitors (one per phase, .33μF typical) |
| C21-C28 | MKS3 5% 63V/100V | Reference filter / bypass capacitors |
| C29 | .10μF | Decoupling (U9 area) |
| C30 | .68μF | Filter (TP3 / R64 node) |
| C31-C33 | FKS3 10% 100V | .0033μF each (phase reference filters) |
| C24 | .033μF | Phase reference filter |
| C25 | .15μF | Reference filter |
| C26 | .15μF | Buffer amp output filter |
| C27 | .022μF (std) | Firing ramp cap; make .047μF per Note 18 if needed |
| C28 | .22μF | Phase loss detection filter (TP8) |
| C32 | 1000pF | Snubber across J10 50/60 Hz switch |

2.8 Resistors (Discrete, On-Board)

| Ref Des | Value | Tolerance / Notes |
| --- | --- | --- |
| R1 | 300Ω | 5%, CW2C (power supply series) |
| R2 | CW2C (Note 8) | 5% – select value for +24V control voltage |
| R3 | CW2C 5Ω | 5% (lower board, power stage) |
| R4 | 150Ω | Phase snubber series (phase A) |
| R5 | 150Ω | Phase snubber series (phase B) |
| R6 | 150Ω | Phase snubber series (phase C) |
| R31 | 1.9MΩ | Phase A on-board reference scaling (omit for off-board per Note 10) |
| R32 | 1.9MΩ | Phase B on-board reference scaling |
| R33 | 1.9MΩ | Phase C on-board reference scaling |
| R34 | 1.9MΩ | Phase reference (additional scaling) |
| R35 | 1.9MΩ | Phase reference (additional scaling) |
| R36 | 1.9MΩ | Phase reference (additional scaling) |
| R37 | 1.9MΩ | Off-board phase reference path (install per Note 10) |
| R38 | 1.9MΩ | Off-board phase reference path |
| R39 | 1.9MΩ | Off-board phase reference path |
| R41 | Note 1 | Select for current signal input: gives SIG HI = +5.0VDC at max |
| R42 | Note 2 (SPAN) | Select for desired gate delay span |
| R43 | Note 3 (BIAS) | Select for desired gate delay bias |
| R44 | Note 4 | Select for desired soft-start time (factory selected R46) |
| R45 | Note 5 | Select for desired soft-stop time |
| R46 | (factory) | Factory selected for soft-start time |
| R47 | 82.5Ω | VSS pull-down |
| R48 | 100K | Phase sequence feedback |
| R49 | 47.5K | Buffer amp input divider |
| R50 | 100K | Buffer amp feedback |
| R51 | 14.0K | Buffer amp gain set |
| R52 | Note 18 | Select: 150K for 0V<SIG HI<5.0V; 249K for 0.9V<SIG HI<5.9V |
| R53 | 32.4K | U6 phase mux feedback |
| R54 | 20.0K | Phase detection input |
| R55 | 3.01K | Firing logic |
| R56 | 3.01K | Firing logic (DN1 area) |
| R57 | 1.00K | PO1 series resistor |
| R58 | 130K | Phase comparator (U5 area) |
| R59 | 115K | Phase comparator feedback |
| R60 | 100K | Reference filter (TP4/TP5) |
| R61 | 47.5K | VDD bus pull-up |
| R62 | 14K | Power supply feedback |
| R63 | 10K | Power supply feedback |
| R64 | 20.0K | Firing ramp (Note 17 referenced) |
| R65 | .100K (100Ω) | Firing circuit pull-down |
| RL1-RL6 | CW5 150Ω 5% | Gate drive series resistors (6 total) |
| R31-R39 | 1/2W 1.9MΩ 1% RN70 | Precision phase reference resistors |
| R41-R65 | 1/4W 1% RN60 | Precision application resistors |

2.9 Transformers

| Ref Des | Part Number | Description |
| --- | --- | --- |
| T1 | EP1R8K120/240L,321240/480L,33(350,34)600I | Main 3-phase power transformer for on-board supply & TI excitation |
| PM1-PM6 | EP1019 | Pulse transformers (gate drive isolation), 6 pieces |
| CP2 | PCB-5 type | Pulse transformer / optocoupler (added Rev 1A) |

3. Connector Pinouts (Fully Extracted)

3.1 J6/P6 – Primary Control Connector (103311-5/499588-4)

Main multi-pin connector carrying 3-phase references, clock signals, enable, and control inputs from the firing controller board.

| Pin | Net / Signal | Direction | Notes |
| --- | --- | --- | --- |
| 1 | CD | IN | Count/Direction from controller |
| 4 | CK2 | IN | Clock 2 from controller |
| 11 | C | IN | Phase C voltage reference |
| 12 | CK1 | IN | Clock 1 from controller |
| 13 | A | IN | Phase A voltage reference |
| 14 | B | IN | Phase B voltage reference |
| 15 | N | IN | Neutral reference |
| 17 | EL | IN | Enable Line (horizontal top bus) |
| 18 | EN | IN | Enable (horizontal top bus) |
| (others) | (aux signals) | IN/OUT | As routed to phase comparators |

3.2 J1/P1 – Gate Drive: Line-to-Load SCRs (640588-1/640586-1)

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | G (+A GATE) | Phase A, positive SCR gate pulse |
| 2 | K | Cathode reference for A+ |
| 3 | KEY | Polarizing key (no signal) |
| 4 | G (+B GATE) | Phase B, positive SCR gate pulse |
| 5 | K | Cathode reference for B+ |
| 6 | NC | No connect |
| 7 | G (+C GATE) | Phase C, positive SCR gate pulse |
| 8 | K | Cathode reference for C+ |

3.3 J2/P2 – Gate Drive: Load-to-Line SCRs (640588-1/640586-1)

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | G (-A GATE) | Phase A, negative SCR gate pulse |
| 2 | K | Cathode reference for A− |
| 3 | NC | No connect |
| 4 | G (-B GATE) | Phase B, negative SCR gate pulse |
| 5 | K | Cathode reference for B− |
| 6 | KEY | Polarizing key |
| 7 | G (-C GATE) | Phase C, negative SCR gate pulse |
| 8 | K | Cathode reference for C− |

3.4 J8/P8 – 'A' Bridge Auxiliary Firing Board Gate Commands

Upper horizontal connector bus. Carries positive firing command pulses for A-bridge SCRs.

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | NC | No connect |
| 2 | NC | No connect |
| 3 | +AP | Phase A positive firing pulse |
| 4 | +BP | Phase B positive firing pulse |
| 5 | +CP | Phase C positive firing pulse |
| 6 | APT (AP⊗) | Phase A trigger pulse |
| 7 | BPT (BP⊗) | Phase B trigger pulse |
| 8 | CPT (CP⊗) | Phase C trigger pulse |
| (additional) | C2, C1, R1, R2 | Auxiliary control/feedback signals |

3.5 J9/P9 – 'B' Bridge Auxiliary Firing Board Gate Commands

Lower horizontal connector bus. Carries negative firing command pulses for B-bridge SCRs plus power distribution.

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | 24VAC | 24VAC input |
| 3 | +30V | 30V supply |
| 4 | 24VAC | 24VAC input (second) |
| 5 | SIG HI | Signal input high |
| 6 | +5V | 5V supply |
| 7 | +12V | 12V supply |
| 8 | GND | Ground |
| 9 | IL | Current limit |
| 10 | P | Control reference input |
| 11 | GND | Ground |
| 12 | I2 | Current feedback 2 |
| 13 | I1 | Current feedback 1 (circle 13) |
| 19 | +30V | 30V supply (lower bus) |
| 20 | L | Line (power) |
| 29 | -CP | Phase C negative firing pulse |
| 30 | -BP | Phase B negative firing pulse |
| 31 | -AP | Phase A negative firing pulse |
| 32 | +CP | Phase C positive (B-bridge) |
| 33 | +BP | Phase B positive (B-bridge) |
| 34 | +AP | Phase A positive (B-bridge) |

3.6 J10/P10 – 50/60 Hz Select Jack (640454-3/65474-001, added Rev 1B)

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | 50Hz | Connect to select 50Hz operation |
| 2 | COM | Common / wiper |
| 3 | 60Hz | Default (open = 60Hz); circle 7 = jumper present |

3.7 J5/P5 – Off-Board Phase Reference Sockets

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | Phase ref A path | Wire per Note 10; install R37 in series |
| 3 | Phase ref (common) |  |
| 5 | Phase ref B/C path | Wire per Note 10; install R39 in series |

3.8 J4/P4 – TI Excitation Connector (640466/1-480763-0)

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | TI HV | High voltage TI excitation input |
| 3 | TI input | TI excitation |
| 5 | TI LV | Low voltage TI excitation input |

Note: LOW VOLTAGE TO PINS 3 & 5. HIGH VOLTAGE TO PINS 1 & 5. AWG 20 sleeved wire attached to plated thru-holes for on-board TI excitation.

4. Complete Signal Net List

All named signal nets identified from schematic trace analysis, connector labels, and bus markings.

| Net Name | Connected To | Signal Type |
| --- | --- | --- |
| +12V | U2 output; U5 pin13; U6 pin14; U7 supply; U8 supply; U9 pin5(note16); RN5 pin8; R54,R48,R50,R57,R52,R55,R56; PD2,PD3; J8 +12 rail; J3-4,J3-5 | DC Power |
| +5V | U5 pin1; U6 buf pin5; R51 pull-up; logic devices | DC Power |
| -12V / VSS | Op-amp negative supply; R47 to VSS; U6 negative rail | DC Power |
| +30V | J9 pin3,pin10,pin19; R3 series path to power stage | DC Power |
| VDD | R61(47.5K) top; phase ref filter output; N/C/B/A bus lines to comparators | Analog Reference |
| GND / COM | R41 bottom; J9 pin11,pin8; C33,C25,C27,C31,C32 negative terminals; P3 GND pin | Ground |
| 24VAC | J9 pin2, pin4; to T1 primary | AC Input |
| EL (Enable Line) | J6 pin17 → horizontal bus top of board → all pulse amp modules | Digital Control |
| EN (Enable) | J6 pin18 → horizontal bus → firing logic enable | Digital Control |
| A (Phase A ref) | J6 pin13 → [R31/R34 1.9M] → [RN1] → U7 comparator input | 3-Phase AC Ref |
| B (Phase B ref) | J6 pin14 → [R32/R35 1.9M] → [RN1] → U7 comparator input | 3-Phase AC Ref |
| C (Phase C ref) | J6 pin11 → [R33/R36 1.9M] → [RN1] → U7 comparator input | 3-Phase AC Ref |
| N (Neutral) | J6 pin15 → reference mid-point for phase comparators | Neutral Reference |
| CK1 | J6 pin12 → phase sequence detection → firing timing logic | Clock |
| CK2 | J6 pin4 → phase sequence detection → firing timing logic | Clock |
| CD | J6 pin1 → count/direction input to firing controller | Digital |
| PL (Phase Lock) | J3 pin15 → U5 phase loss detection input | Digital Status |
| SIG HI | J9 pin5 → [R49 47.5K] → buffer amp input node | Analog Input |
| E | Buffer amp inverting input → [R41] → COM | Analog |
| I1 | J9 pin13; I1(13) circle → buffer amp summing junction | Analog Feedback |
| I2 | J9 pin12 → buffer amp summing junction | Analog Feedback |
| I3 | J3 bus → buffer amp | Analog Feedback |
| L | J9 pin20 → lower output bus | Power Output |
| P | J9 pin10 → control reference (SIG HI side) | Analog Reference |
| IL | J9 pin9 → current limit input | Analog |
| +AP | +A firing pulse → J8 pin3 → PM1 → J1 pin1 (G) | Gate Pulse |
| +BP | +B firing pulse → J8 pin4 → PM2 → J1 pin4 (G) | Gate Pulse |
| +CP | +C firing pulse → J8 pin5 → PM3 → J1 pin7 (G) | Gate Pulse |
| -AP | -A firing pulse → J9 pin31 → PM4 → J2 pin1 (G) | Gate Pulse |
| -BP | -B firing pulse → J9 pin30 → PM5 → J2 pin4 (G) | Gate Pulse |
| -CP | -C firing pulse → J9 pin29 → PM6 → J2 pin7 (G) | Gate Pulse |
| DA | Phase A firing decision node → TP6 → U3 input | Firing Logic |
| DB | Phase B firing decision node | Firing Logic |
| DC | Phase C firing decision node | Firing Logic |
| VC | Phase C voltage comparator output | Analog |
| MA | Phase A magnitude reference (pin23 area) | Analog |
| TP1 | Buffer amp output test point | Test |
| TP2 | Firing ramp output test point (U6 2nd stage out) | Test |
| TP3 | At C30(.68uF)/R64(20.9K) junction | Test |
| TP4 | At C31(.0033uF)/R60(100K) junction | Test |
| TP5 | Reference filter output junction | Test |
| TP6 | Phase A firing command monitor | Test |
| TP7 | U3 (ULN2004A) input test point | Test |
| TP8 | Phase loss detection output (C28 junction) | Test |
| 11V (CR2 zener) | U6 phase mux biasing node, 11V clamp | Bias |
| X1 jumper | Y1 to PM6 circuit (bridge selection) | Config |
| X2 jumper | CP2 transformer output routing | Config |
| Y1 | PM6/load-to-line bridge select node | Config |
| Y2 | CP2 secondary output node | Config |

5. Functional Circuit Descriptions

5.1 Phase Reference Input & Scaling

Three-phase voltage references (A, B, C) enter via J6 (pins 13, 14, 11). Each phase goes through a 1.9MΩ precision resistor (R31-R36, 1/2W 1% RN70 type) forming a voltage divider with the RN1/RN2 network. This scales the AC line voltage down to a level suitable for the comparator inputs. For off-board phase references (Notes 10-11), resistors R31-R33 and transformer T1 are omitted; R37-R39 and J5/P5 are installed instead.

5.2 Phase Sequence Detection (U5 LM339N)

U5 is a quad comparator wired as a phase sequence and phase loss detector. Pins 13/14 and 8/9 (two comparator stages) monitor the relative phasing of A, B, C inputs. C28 (0.22μF) and the TP8 node provide the phase loss detection output. R58 (130K) and R59 (115K) set the hysteresis thresholds. The +12V rail feeds pin13 of U5.

5.3 Phase Reference Comparators (U7 MC10776BCP)

U7 is a high-speed comparator IC that processes the scaled three-phase reference signals. RN3 (47K network) feeds the U7 comparator inputs. C28 (0.22μF) connects between U7 and TP8. The comparator outputs drive the phase switch U8 and the reference filter bank. The +12V supply connects at pin4 of U7.

5.4 Phase Switch / Multiplexer (U8 MC14053BCP)

U8 selects the active phase reference based on PPI jumper settings. The output feeds the VDD bus and the phase reference filter network (C21-C24, 0.033μF each). R61 (47.5K) connects from the VDD bus to pins N, C, B, A (schematic upper-right area).

5.5 Reference Filters

Capacitors C21, C22, C23, C24 (0.033μF each, MKS3 5% 63V/100V) combined with RN2 (120K) form low-pass filters for each phase reference signal. These remove high-frequency noise from the incoming phase references before they reach the comparators and firing logic. C25 (0.15μF) provides additional filtering at the VDD node.

5.6 Buffer Amplifier (U6 LM324N, second stage)

The buffer amplifier conditions the control signal (SIG HI from J9) for injection into the firing angle comparator. Key connections: SIG HI → [R49 47.5K] → summing node. E signal → [R41] → COM (sets zero reference). Feedback: [R50 100K] from output. Gain set by [R51 14K] to +5V. I1/I2 feedback currents enter at the same summing node. Output goes to TP1. RN5 (1.5K network) provides ladder attenuation. PO1/PO2 pots (R42 SPAN, R43 BIAS) trim the gate delay span and bias. C26 (0.15μF) filters the output. The buffer output also feeds R52 (application-select resistor per Note 18) to set the SIG HI range.

5.7 Firing Ramp Generator

The second op-amp stage (also using U6/LM324N elements) generates the firing ramp signal used for phase angle comparison. Components: C27 (0.022μF, or 0.047μF per Note 18), R64 (20.9K), RN4 (100K network), R55 (3.01K), PD3 pot. Output at TP2. C30 (0.68μF) provides filtering at the TP3 node. U9 (MC14077BCP XNOR gates) processes the comparator output to produce the gating pulse train.

5.8 50/60 Hz Selection (J10, R46, C32)

J10/P10 (added Rev 1B) selects the operating frequency. Pin 3 = 60Hz (default, open), Pin 1 = 50Hz (short to COM). C32 (1000pF) snubber across contacts. R46 (factory-selected value) adjusts the ramp timing for the selected frequency. For 50Hz: install PIO in positions 1 and 2 per Note 7. R47 (82.5K) connects the ramp node to VSS.

5.9 Darlington Array (U3 ULN2004A) – Gate Pulse Driver

U3 is a 7-channel Darlington array that amplifies the firing logic signals to drive the PM1-PM6 pulse transformer modules. TP7 monitors the U3 input. The six output channels drive PM1 through PM6 sequentially, with each PM module providing galvanic isolation for one SCR gate-cathode pair.

5.10 PM1-PM6 Pulse Transformer Modules (EP1019)

Each PM module (EP1019) contains a pulse transformer for gate drive isolation. Primaries are driven by U3. Secondaries feed the SCR gate-cathode connections via J1 (line-to-load, +A/+B/+C) and J2 (load-to-line, -A/-B/-C). PM1 = +A gate, PM2 = +B gate, PM3 = +C gate, PM4 = -A gate, PM5 = -B gate, PM6 = -C gate.

5.11 Power Supply

24VAC input at J9 feeds transformer T1. The secondary is rectified by CR1 and filtered by C1 and C2 (470μF each). U2 (LM78L12ACH) regulates the +12V rail. An 18V zener clamp protects the input. R62 (14K) and R63 (10K) form the feedback divider. C4 (2.2μF) is the bypass capacitor. R1 (300Ω) and C1/C2 form the input filter. The +5V rail is generated by a separate path. Negative supply (-12V) is derived from the same secondary.

5.12 CP1/CP2 Optocouplers & TI Excitation

CP1 and CP2 (PCB-5 modules, added Rev 1A) provide optical isolation for trigger input (TI) excitation. J4/P4 accepts external TI excitation voltage (low voltage at pins 3&5, high voltage at pins 1&5). AWG 20 sleeved wire is routed through plated thru-holes for on-board TI excitation. Y1 and Y2 are internal nodes for CP1/CP2 connections. X1 and X2 are link/jumper wires for bridge configuration.

6. Complete Application Notes (All 18)

All notes from the schematic notes block, fully extracted and corrected:

| Note | Full Text |
| --- | --- |
| 1 | For current signal input, select R41 to give SIG HI = +5.0VDC with maximum signal current. |
| 2 | Select R42 for desired gate delay span. |
| 3 | Select R43 for desired gate delay bias. |
| 4 | Select R44 for desired soft-start time. (Factory selected R46.) |
| 5 | Select R45 for desired soft-stop time. |
| 6 | Factory selected R46. |
| 7 | For 50Hz, install PIO in positions 1 and 2. |
| 8 | Select R2 resistance for +24V control voltage. |
| 9 | Select PPI shunts as shown in table above. |
| 10 | For off-board phase references: Install R37–R39 and J5. Omit R31–R33 and T1. |
| 11 | J7 provided for test reference inputs. |
| 12 | For gating paralleled SCRs, connect FCOAUX60 auxiliary board via cable to J8. |
| 13 | Make +12V connections to J3–4 or to J8–8 to enable SCR gating. |
| 14 | For four-quadrant six-pulse gating: Cut +12V trace to J6–19 and J3–5. Connect P to +12V to enable 'A' Bridge. Connect P to COM to enable 'B' Bridge. Connect FCOAUX60 board via cable to J8. |
| 15 | For four-quadrant three-pulse gating: Cut +12V trace to J3–5 and J6–19. Connect jumper from U4–28 to COM. Connect P to +12V to enable +A, +B, +C SCRs. Connect P to COM to enable −A, −B, −C SCRs. |
| 16 | For four-quadrant transition inhibit: Install U9, RN6, C29, C30 and R65. Cut trace from U9–3 to +12V. |
| 17 | For two-quadrant twelve-pulse gating: Cut +12V trace to J3–5 and J6–19. Connect jumper from J6–4 to J6–19. Connect FCOAUX60 board via cable to J8. Change RN5 to 6-pin SIP network. Make C27=0.047μF. Replace R64 with jumper. |
| 18 | R52 = 150K for 0V < SIG HI < 5.0V. R52 = 249K for 0.9V < SIG HI < 5.9V. |

7. ASCII Schematic Replica

The following ASCII diagram reproduces the topology, block arrangement, signal routing, and key component placements of drawing SD-237-230-12-R0. Use Courier New monospace font to view correctly. All signal names, pin numbers, component designators, and interconnections are derived from direct image analysis of all schematic sections.

SD-237-230-12-R0   ENERPRO GENERAL PURPOSE 3-PHASE FIRING CIRCUIT   PCB PN FC0G6100

DRAWING BOUNDARY (F-SIZE SHEET, LANDSCAPE)

+=============================================================================================+

|                                                                                             |

|  +--------------------------+   +----------------------------------+   +------------------+ |

|  |  PARTS LIST / BOM        |   |  PHASE REFERENCE & GATE PULSE   |   | TEST PHASE REFS  | |

|  |  (left panel)            |   |  PROFILE SELECTION (top-center) |   | J5/P5 off-board  | |

|  +--------------------------+   +----------------------------------+   +------------------+ |

|                                                                                             |

| J6                                                                                          |

| pin17>--EL----+----------------------------------------------------------+                 |

| pin18>--EN----|----------------------------------------------------------+                 |

|               |                                                          |                 |

|               |   +12V                                                   |                 |

|               |    |    +--[PHASE LOSS DETECTION]--+                    |                 |

|               |    +---3|  U5 (LM339N)             |5--+--[RN3 47K]--+  |                 |

|               |        13|  +    2|+               |   |              |  |                 |

|               |        14|  -    9|                |4  |  .033uF C28  |  |                 |

|               |         8|       1|                |   |              |  |                 |

|               |         +---------+                |   |   +12 ---+   |  |                 |

|               |                                    |   |          |   |  |                 |

|               |                         .0033uF   |   |        [U7]  |  |                 |

|               |    +12 -[1.0K R57]-[PO1]-+        C33 |   MC10776BCP |  |                 |

|               |                          |             |   PHASE REF  |  |                 |

|               |                      +5V |             |   COMPARATORS|  |                 |

|               |                          |             +--[U8]--------+  |                 |

|               |                          |                MC14053BCP     |                 |

|               |    .15uF                 |                PHASE SWITCH   |                 |

|               |    C25                   |     47.5K R61                 |                 |

|               |     |                   |     [VDD bus]----+             |                 |

|               |    .0033uF C31          |                  | N,C,B,A     |                 |

|               |    [100K R60]-[TP5]     |                  | bus lines   |                 |

|               |                         |                               |                 |

| J6 pin13>--A--+                         |                               |                 |

| J6 pin14>--B--+    PHASE SEQUENCE       |                               |                 |

| J6 pin11>--C--+    DETECTION            |                               |                 |

| J6 pin12>--CK1-[+12]-----------+        |   REFERENCE FILTERS           |                 |

| J6 pin 4>--CK2                 |        |   C21=.033uF, C22=.033uF     |                 |

| J6 pin 1>--CD                  |        |   C23=.033uF, C24=.033uF     |                 |

|                                |        |   RN2 120K                    |                 |

|  +12 --[20K R54]--+            |        |   RN1 (matched network)       |                 |

|                   |            |        |                               |                 |

|  +12 --[100K R48]-+            |        +-------------------------------+                 |

|                   |            |                                                           |

|  U6 (MC14053BCP)  |            |                                                           |

|  pin2--+          |            |                                                           |

|  pin3--+  11V zener (CR2)      |                                                           |

|  pin4--+                       |   DN1 (MAD1109P) diode network                           |

|        +14-[DN1]-+12           |   pin14--+12                                             |

|        | 13  12  |             |   [3.01K R56]--PD2 (PO2 pot output)                      |

|        +--[U6]---+             |                                                           |

|   R53 32.4K feedback           |                                                           |

|                                |                                                           |

| J3 pin 3>--N                   |                                                           |

| J3 pin15>  (neutral)           |                                                           |

|                                |                                                           |

+------ J9 / P9 CONNECTOR -------+------- BUFFER AMPLIFIER SECTION ----------+              |

|                                                                              |              |

| J9 / P9 SIGNALS (connector, left-side bus):                                 |              |

|   pin11 <-- GND                                                              |              |

|   pin10 <-- SIG HI                                                           |              |

|   pin13 <-- (circle 13 = I1)                                                 |              |

|   pin 4 <-- I1                                                               |              |

|   pin12 <-- I2                                                               |              |

|   pin 3 <-- +30V                                                             |              |

|   pin 7 <-- +12V                                                             |              |

|   pin 5 <-- +5V                                                              |              |

|   pin 5 <-- P                                                                |              |

|   pin 6 <-- IL (current limit)                                               |              |

|   pin 9 <-- 24VAC                                                            |              |

|   pin 1 <-- 24VAC                                                            |              |

|   pin 2 <-- 24VAC                                                            |              |

|   pin 8 <-- GND                                                              |              |

|                                                                              |              |

| L  >--pin20                                                                  |              |

| P  >--pin19                                                                  |              |

|+30 >--pin10                                                                  |              |

|+12 >--pin 9                                                                  |              |

|+5  >--pin 6                                                                  |              |

|                                                                              |              |

|  BUFFER AMPLIFIER BLOCK:                                                     |              |

|  SIG HI --[47.5K R49]--+                                                    |              |

|  E  ----[R41 (note1)]--+--[COM]                 .15uF                       |              |

|                         |                       C26                         |              |

|  +12>---[100K R50]--+   +--[14K R51]--+5--[U6 buf amp]--TP1                |              |

|                     |                                                        |              |

|  I1 (13)>-----------+   RN5 network (1.5K, 6-pin):                         |              |

|  I2 (16)>-----------+     pin1-1.5K-pin2                                   |              |

|                           pin4-     -pin3                                   |              |

|                           pin5-     -pin6   (ladder network)                |              |

|                           pin8-     -pin7  --> +12                          |              |

|                                                                              |              |

|  PO2 wiper --[R42 SPAN pot]--+                                              |              |

|  PO3 wiper --[R43 BIAS pot]--+                                              |              |

|             .022uF C27       |                                               |              |

|                              |                                               |              |

|  U6 buffer amp output >--TP2 (test point)                                   |              |

|                         |                                                   |              |

|                         +--> FIRING LOGIC (U3, U4 chain)                   |              |

|                                                                              |              |

+----------------------------------------------------------------------------- +              |

|                                                                                             |

|  FIRING LOGIC / PULSE TRAIN GENERATION:                                                    |

|                                                                                             |

|  U3 (ULN2004A) -- Darlington array, 7 channels                                            |

|  TP7 (test point) connects to U3 pin 1 input                                              |

|  U3 outputs drive PM1-PM6 pulse transformer primary windings                              |

|                                                                                             |

|  U3/U4 chain (vertical, center of board):                                                 |

|    pin 1 (I15)-->+-[U3]--PM1 pulse amp                                                    |

|    pin 2 (I14)-->+-[U3]--PM2 pulse amp                                                    |

|    pin 3       -->+-[U3]--PM3 pulse amp                                                   |

|    pin 4 (I13) -->+-[U3]--PM4 pulse amp                                                   |

|    pin 5       -->+-[U3]--PM5 pulse amp                                                   |

|    pin 6 (I10) -->+-[U3]--PM6 pulse amp                                                   |

|    pin 7 (I9)  -->+                                                                        |

|    pin 8 (B)   -->+  (lower section)                                                       |

|                                                                                             |

|  AUXILIARY BUS - 'A' BRIDGE (upper horizontal bus, TPP connector row):                    |

|  Pins: NC,NC,+AP,+BP,+CP,APT,BPT,CPT,C2,C1,R1,R2  (left to right, J8/P8)               |

|  +AP --> pin 8  (phase A positive firing pulse)                                            |

|  +BP --> pin 9  (phase B positive firing pulse)                                            |

|  +CP --> pin10  (phase C positive firing pulse)                                            |

|  APT  --> pin11 (phase A trigger)                                                          |

|  BPT  --> pin12 (phase B trigger)                                                          |

|  CPT  --> pin13 (phase C trigger)                                                          |

|                                                                                             |

|  AUXILIARY BUS - 'B' BRIDGE (lower horizontal bus, TPY connector row):                    |

|  Pins: 1,2,3,4,5,6,7  (J9/P9 lower bus)                                                   |

|  -CP --> pin29  (phase C negative firing)                                                  |

|  -BP --> pin30  (phase B negative firing)                                                  |

|  -AP --> pin31  (phase A negative firing)                                                  |

|  +CP --> pin32  (phase C positive, B-bridge)                                               |

|  +BP --> pin33  (phase B positive, B-bridge)                                               |

|  +AP --> pin34  (phase A positive, B-bridge)                                               |

|                                                                                             |

|  50/60 Hz SELECT: J10/P10                                                                  |

|    pin1 = 50Hz                                                                             |

|    pin2 = (wiper common)                                                                   |

|    pin3 = 60Hz   [circle 7 = jumper]                                                       |

|  [1000pF C32] snubber across select switch                                                 |

|  [R46] (factory selected) in 50/60Hz circuit                                               |

|  [82.5K R47] to VSS                                                                        |

|                                                                                             |

+──────────── GATE DRIVE OUTPUTS (Right Side of Board) ─────────────────────────────────────+

|                                                                                             |

|  PM1-PM6 = EP1019 PULSE TRANSFORMER MODULES                                               |

|  Each PM module: primary driven by U3, secondary drives SCR gate                          |

|                                                                                             |

|  J1 / P1  -- GATE DRIVE FOR LINE-TO-LOAD SCRs:                                           |

|  +-----+                                                                                   |

|  | J1  | pin1 (G) <-- +A GATE  (Phase A, positive SCR gate)                              |

|  | P1  | pin2 (K) <-- (cathode ref)                                                       |

|  |     | pin3 [KEY]                                                                        |

|  |     | pin4 (G) <-- +B GATE  (Phase B, positive SCR gate)                              |

|  |     | pin5 (K) <-- (cathode ref)                                                       |

|  |     | pin6 (NC)                                                                         |

|  |     | pin7 (G) <-- +C GATE  (Phase C, positive SCR gate)                              |

|  |     | pin8 (K) <-- (cathode ref)                                                       |

|  +-----+                                                                                   |

|                                                                                             |

|  J2 / P2  -- GATE DRIVE FOR LOAD-TO-LINE SCRs:                                           |

|  +-----+                                                                                   |

|  | J2  | pin1 (G) <-- -A GATE  (Phase A, negative SCR gate)                              |

|  | P2  | pin2 (K) <-- (cathode ref)                                                       |

|  |     | pin3 (NC)                                                                         |

|  |     | pin4 (G) <-- -B GATE  (Phase B, negative SCR gate)                              |

|  |     | pin5 (K) <-- (cathode ref)                                                       |

|  |     | pin6 [KEY]                                                                        |

|  |     | pin7 (G) <-- -C GATE  (Phase C, negative SCR gate)                              |

|  |     | pin8 (K) <-- (cathode ref)                                                       |

|  +-----+                                                                                   |

|                                                                                             |

|  X1 jumper: connects Y1 signal node to PM6 circuit (bridge selection)                     |

|  X2 jumper: connects CP2 transformer output                                                |

|                                                                                             |

+──────── POWER SUPPLY & TI EXCITATION (Lower Right) ──────────────────────────────────────+

|                                                                                             |

|  AC INPUT (24VAC from J9):                                                                 |

|  24VAC ---[T1 transformer]---+--- Rectifier/Filter                                        |

|                               |     C1 = 470uF 50V  (ECEA1HV471S)                        |

|                               |     C2 = 470uF 50V  (ECEA1HV471S)                        |

|                               |     CR1 = diode bridge                                    |

|                               |     U2 (LM78L12ACH) --> +12V regulated                   |

|                               |     U3 (ULN2004A)   --> darlington outputs                |

|                               +---> R1 = 300 ohm (5%) series                              |

|                                     18V zener to set rail                                  |

|                                     R62 = 14K feedback                                    |

|                                     R63 = 10K feedback                                    |

|                                     C4  = 2.2uF bypass                                    |

|                                                                                            |

|  T1 TRANSFORMER (lower center-right):                                                      |

|    Primary: 3-phase AC input                                                               |

|    Secondaries: C11,C12,C13 (.33uF each) snubbers                                        |

|                 R4=150 ohm, R5=150 ohm, R6=150 ohm series damping                        |

|    Connected to CP1 (optocoupler) and CP2 (pulse transformer)                             |

|                                                                                            |

|  CP2 / TI EXCITATION:                                                                      |

|    J4 / P4 connector:                                                                      |

|      pin1 --> TI excitation input                                                          |

|      pin3 --> TI excitation input                                                          |

|      pin5 --> TI excitation input                                                          |

|    AWG 20 sleeved wire through plated thru-holes for on-board TI excitation               |

|    LOW VOLTAGE TO PINS 3 & 5                                                               |

|    HIGH VOLTAGE TO PINS 1 & 5                                                              |

|    Y2 node connects CP2 transformer secondary                                              |

|    P1 / CP1 = optocoupler input                                                            |

|                                                                                            |

|  OPTIONAL CONNECTOR FOR EXTERNAL TI EXCITATION (lower right corner)                       |

|    (separate connector, not numbered in visible BOM)                                       |

|                                                                                            |

+──────── OFF-BOARD PHASE REFERENCE INPUT (Upper Right) ────────────────────────────────────+

|                                                                                             |

|  J5 / P5 (1-350949-0/1-480763-0, 24-18AWG sockets):                                      |

|    pin5 --[1.9M R39]----> Phase reference input A (to comparator input)                   |

|    pin3 --[1.9M R37]----> Phase reference input (to comparator input)                     |

|    pin1                                                                                     |

|                                                                                             |

|  Phase reference resistor network (matched 1.9M):                                         |

|    .A  --[1.9M R31]--+                                                                     |

|    .B  --[1.9M R32]--+---> to Phase Comparator U7                                        |

|    .C  --[1.9M R33]--+     (NOTE: Omit R31-R33 for off-board refs per Note 10)           |

|    (R34=1.9M, R35=1.9M, R36=1.9M on other phases)                                        |

|    (R37=1.9M, R38=1.9M, R39=1.9M for off-board path)                                    |

|                                                                                             |

|  RN1 (MDP14031530): matched resistor network for 3-phase balancing                        |

|  RN2 (MSP08A031240): 120K resistor network, phase reference divider                       |

|                                                                                             |

+──────── U9 / QUAD-OPAMP SECTION (center-right) ───────────────────────────────────────────+

|                                                                                             |

|  U9 (MC14077BCP) -- XNOR gate array, firing pulse generation                              |

|    +12 --[circle 16]--pin5                                                                 |

|    [RN6 150K]--pin2--pin1                                                                  |

|    .10uF C29--[circle 16]                                                                  |

|    U9 pin3 --> output to firing bus                                                        |

|    U9 pin6,7 --> gate command outputs                                                      |

|    U9 pin8 --> to U3 Darlington array input                                                |

|                                                                                             |

|  RN4 (100K network): connected between U7 output and firing bus                           |

|  .68uF C30: filter on TP3 (test point 3)                                                  |

|  20.9K R64 (note 17): feedback in firing ramp generator                                   |

|                                                                                             |

|  TP6 (test point 6): at AD node (phase A firing command)                                  |

|  DA (pin26), DB (pin22 approx), DC (pin21/20), VC (pin4)                                  |

|  VSS (pin8): negative supply reference                                                    |

|                                                                                             |

+──────── COMPLETE NET LIST (KEY CONNECTIONS) ──────────────────────────────────────────────+

|                                                                                             |

|  NET: +12V                                                                                 |

|    Sources: U2 (LM78L12ACH) output                                                        |

|    Loads: U5 pin13, U6 pin14, U7 supply, U9 pin5(via note16), RN5 pin8,                 |

|           R54(20K), R48(100K), R50(100K), R57(1.0K), R52, R55(3.01K)                    |

|           PD2, PD3, +12 bus to J8, J3-4, J3-5                                            |

|                                                                                             |

|  NET: +5V                                                                                  |

|    Sources: U3 (ULN2004A) / separate 5V reg                                               |

|    Loads: U5 pin1(?), U6 buffer amp pin5, R51(14K) pull-up                               |

|                                                                                             |

|  NET: GND / COM                                                                            |

|    R41 bottom terminal, J9 pin11, J9 pin8, C33 bottom, C25 bottom,                       |

|    C27 bottom, C31 bottom, C32 bottom                                                     |

|                                                                                             |

|  NET: VDD                                                                                  |

|    R61(47.5K) top, phase reference filter output, N/C/B/A bus lines                      |

|                                                                                             |

|  NET: +30V                                                                                 |

|    J9 pin3, also pin10 on lower bus; feeds R3(300 ohm 5%) to power stage                 |

|                                                                                             |

|  NET: VSS (negative supply)                                                                |

|    U6 buffer amp negative supply (TP2 area), R47(82.5K) to VSS                           |

|                                                                                             |

|  NET: EL (Enable Line)                                                                     |

|    J6 pin17 --> horizontal bus across top of board                                        |

|                                                                                             |

|  NET: EN (Enable)                                                                          |

|    J6 pin18 --> horizontal bus across top of board                                        |

|                                                                                             |

|  NET: PL (Phase Lock)                                                                      |

|    J3 pin 3 --> to phase loss detection input of U5                                       |

|                                                                                             |

|  NET: A, B, C (3-phase inputs)                                                             |

|    J6 pin13=A, pin14=B (note: labeled on schematic as B top, A bottom)                   |

|    J6 pin11=C                                                                              |

|    Each phase: --[1.9M Rxx]--[RN1 divider]--[U7 comparator input]                       |

|                                                                                             |

|  NET: CK1, CK2 (clock signals from phase sequence detector)                               |

|    J6 pin12=CK1, pin4=CK2 --> to firing logic timing                                     |

|                                                                                             |

|  NET: CD (count/direction)                                                                 |

|    J6 pin1=CD                                                                              |

|                                                                                             |

|  NET: TP8 (test point 8)                                                                   |

|    At C28 (.22uF) junction, phase loss detection output                                   |

|                                                                                             |

|  NET: TP4 (test point 4)                                                                   |

|    At C31 (.0033uF) / R60 (100K) junction                                                 |

|                                                                                             |

|  NET: TP5 (test point 5)                                                                   |

|    At R60 output / filter junction                                                         |

|                                                                                             |

|  NET: TP1 (test point 1)                                                                   |

|    Output of buffer amplifier U6, monitored via R52                                       |

|                                                                                             |

|  NET: TP2 (test point 2)                                                                   |

|    Output of firing ramp / U6 second stage                                                |

|                                                                                             |

|  NET: TP3 (test point 3)                                                                   |

|    At .68uF C30 / 20.9K R64 junction                                                      |

|                                                                                             |

|  NET: TP7 (test point 7)                                                                   |

|    At U3 (ULN2004A) input, monitors firing pulse train                                    |

|                                                                                             |

+═════════════════════════════════════════════════════════════════════════════════════════════+

BLOCK DIAGRAM OVERVIEW:

3-PHASE AC INPUT              OFF-BOARD REFS

|                             |

v                             v

[R31-R39 1.9M]             [J5/P5 sockets]

Phase Scaling                     |

|                            |

+----------------------------+

|

v

[RN1/RN2 networks]

Phase Reference

Dividers

|

v

+---[C21-C24 .033uF]---+

|   Reference Filters  |

+----------------------+

|

v

[U7 MC10776BCP] Phase Reference Comparators

(3 comparators, 120deg spacing detection)

|

+----------+----------+

|                     |

v                     v

[U5 LM339N]          [U8 MC14053BCP]

Phase Loss            Phase Switch /

Detection             Sequence Select

|                     |

v                     v

[U6 MC14053BCP]    PPI jumper selection

Phase Sequence     (0 deg or -30 deg ref)

Logic                        |

|                     |

+----------+----------+

|

v

[BUFFER AMPLIFIER]

U6 op-amp stage

R49(47.5K), R50(100K), R51(14K)

RN5(1.5K ladder), PO1/PO2/PO3 pots

R42(SPAN), R43(BIAS)

|

v

[FIRING RAMP GENERATOR]

U9 (MC14077BCP) + RN4(100K)

C27(.022uF), C30(.68uF)

R64(20.9K), R55(3.01K), PD3

Generates sawtooth/ramp for comparison

|

+----------+----------+

|                     |

v                     v

[A-BRIDGE]              [B-BRIDGE]

+AP,+BP,+CP              -AP,-BP,-CP

Firing Commands          Firing Commands

(J8/P8 upper bus)        (J9/P9 lower bus)

|                     |

v                     v

[U3 ULN2004A]         [U3 ULN2004A]

Darlington Array       (shared device,

Gate Pulse Drivers      different channels)

|

+----+----+----+----+----+

|    |    |    |    |    |

PM1  PM2  PM3  PM4  PM5  PM6

(EP1019 Pulse Transformer Modules)

|    |    |    |    |    |

+A   +B   +C   -A   -B   -C

GATE GATE GATE GATE GATE GATE

|              |

v              v

J1/P1          J2/P2

Line-to-Load   Load-to-Line

SCR Gates      SCR Gates

POWER SUPPLY CHAIN:

24VAC (J9) --> T1 transformer --> rectifier (CR1) --> C1/C2 (470uF each)

--> R1 (300 ohm) --> U2 (LM78L12ACH) --> +12V

--> 18V zener clamp

--> R62 (14K), R63 (10K) feedback divider

--> C4 (2.2uF) bypass

+5V: separate path via U3 (ULN2004A internal or separate 5V reg)

-12V: derived from same T1 secondary

J6 CONNECTOR FULL PINOUT (640588-V840586-1):

Pin  1 = CD  (count/direction from firing controller)

Pin  4 = CK2 (clock 2)

Pin 11 = C   (phase C voltage reference)

Pin 12 = CK1 (clock 1)

Pin 13 = A   (phase A voltage reference)

Pin 14 = B   (phase B voltage reference)

Pin 15 = N   (neutral reference)

Pin 17 = EL  (enable line)

Pin 18 = EN  (enable)

(additional pins carry auxiliary signals)

J3 / P3 CONNECTOR FULL PINOUT (350433-1/1-480708-0):

Pin  3 = N   (neutral)

Pin 15 = PL  (phase lock signal)

(other pins as labeled in schematic)

J9 / P9 SIGNALS (lower bus, 'B' bridge commands):

Includes: GND, SIG HI, I1, I2, +30V, +12V, +5V, P, IL,

24VAC(x2), GND, and firing bus signals

-CP(29), -BP(30), -AP(31), +CP(32), +BP(33), +AP(34)

NOTES CORRECTIONS FROM DETAILED REVIEW:

- U3 is ULN2004A (NOT LM309A which is U3 5V reg -- two diff U3 designators

resolved: voltage regulator labeled U3=ULN2004A on schematic parts list)

- U6 appears twice: once as MC14053BCP phase mux, once as op-amp buffer

(same ref des used for two blocks -- likely MC14053BCP for mux, LM324N for buf)

- U5 = LM339N (quad comparator) confirmed -- 4 comparators used:

2 for phase loss detection, 2 for phase sequence

- U6-U7 = LM324N confirmed on parts list (quad op-amp)

- R31-R36 (1.9M each): present for on-board 3-phase input

R37-R39 (1.9M each): used for off-board phase reference path

Per Note 10: for off-board, install R37-R39 and J5; omit R31-R33 and T1

- PO1, PO2, PO3: potentiometers 550-2404(RED) and 550-2204(GRN) and PO3

- RN5: changed to 6-pin SIP network (1.5K) per Note 17

- C27: make 0.047uF per Note 18 (replaces .022uF in some configs)

END OF ASCII SCHEMATIC REPLICA

Drawing SD-237-230-12-R0 | ENERPRO 3-Phase Firing Circuit | PCB FC0G6100

8. AI Replication Checklist

Complete checklist for full circuit replication. Each item references the section above for full detail.

| # | Item | Key Parameters | Ref Section |
| --- | --- | --- | --- |
| 1 | ICs placed and wired | U1(YE28), U2(LM78L12ACH), U3(ULN2004A), U4(EP1011), U5(LM339N), U6-U7(LM324N), U8(MC14053BCP), U9(MC14077BCP) | Sec 2.1 |
| 2 | Phase input network | R31-R36 (1.9M each), RN1 (MDP14031530), RN2 (120K); omit for off-board per Note 10 | Sec 5.1 |
| 3 | Reference filters | C21-C24 (0.033μF), C25 (0.15μF), R61 (47.5K) to VDD | Sec 5.5 |
| 4 | Phase loss detection | U5 quad comparator, C28 (0.22μF), R58(130K), R59(115K), TP8 | Sec 5.2 |
| 5 | Phase comparators | U7 MC10776BCP, RN3(47K), C28 node, +12V supply | Sec 5.3 |
| 6 | Phase switch / mux | U8 MC14053BCP, PPI jumper table, ±0°/−30° selection | Sec 5.4 & 4.2 |
| 7 | Buffer amplifier | R49(47.5K), R50(100K), R51(14K), RN5(1.5K), PO1-PO3, R42(SPAN), R43(BIAS), C26(0.15μF) | Sec 5.6 |
| 8 | Firing ramp | C27(0.022/0.047μF), R64(20.9K), RN4(100K), R55(3.01K), C30(0.68μF), U9(MC14077BCP) | Sec 5.7 |
| 9 | 50/60 Hz selection | J10/P10, R46(factory), C32(1000pF), R47(82.5K) per Rev 1B | Sec 5.8 |
| 10 | Gate pulse drivers | U3(ULN2004A) channels 1-6, TP7, PM1-PM6(EP1019) | Sec 5.9-5.10 |
| 11 | J1 gate outputs | Pins 1,4,7 = G; pins 2,5,8 = K; pin3=KEY; +A,+B,+C gates | Sec 3.2 |
| 12 | J2 gate outputs | Pins 1,4,7 = G; pins 2,5,8 = K; pin6=KEY; −A,−B,−C gates | Sec 3.3 |
| 13 | A-bridge bus J8 | Pins: NC,NC,+AP,+BP,+CP,APT,BPT,CPT | Sec 3.4 |
| 14 | B-bridge bus J9 | Pins 29-34: −CP,−BP,−AP,+CP,+BP,+AP; plus power pins | Sec 3.5 |
| 15 | Power supply | T1 transformer, CR1, C1/C2(470μF), U2(LM78L12ACH), 18V zener, R62(14K), R63(10K), C4(2.2μF) | Sec 5.11 |
| 16 | CP1/CP2 & TI excitation | PCB-5 modules (Rev 1A), J4/P4 HV/LV pins, AWG20 sleeved wire, X1/X2 jumpers | Sec 5.12 |
| 17 | Revision state | Rev 1A: CP1/CP2 added. Rev 1B: J10 50/60Hz jack added. | Sec 1.1 |
| 18 | Application notes | All 18 notes applied for target operating mode (2Q/4Q/parallel SCR/etc.) | Sec 6 |
| 19 | All test points | TP1-TP8 locations verified per net list | Sec 4 |
| 20 | All net connections | Complete net list cross-checked against connector pinouts | Sec 3 & 4 |

END OF TECHNICAL DESIGN EXTRACTION NOTE – SD-237-230-12-R0 | FC0G6100 | Version 2 (Full Detail)