TECHNICAL DESIGN EXTRACTION NOTE

Enerpro Voltage and Current Regulator Board

Drawing No.: SD-237-230-14-C1  |  SLAC / Stanford University

Version 1 - Full Detail including Component Values, Signal Nets, and Functional Descriptions

# 1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-237-230-14-C1 |
| Title | Enerpro Voltage and Current Regulator Board |
| System | PEP II RF System |
| Organization | Stanford Linear Accelerator Center (SLAC) |
| Location | Stanford University, Stanford, California |
| Sheet | 1 of 1 |
| CAD File | 23723014.SD1 |
| Engineer | M. Nguyen (9-17-97) |
| Drafter | T. Phan (9-17-97) |
| Checker | J. Olszewski (10-5-99, 9-20-02) |
| Revision | C1 (Added notes for NLCTA, 5/21/03) |

## 1.1 Revision History

| Rev | Description | By | Date |
| --- | --- | --- | --- |
| C1 | Added notes for NLCTA | WSG / JO / MN | 5/21/03 |

# 2. Complete Bill of Materials

## 2.1 Integrated Circuits

| Ref Des | Part Number | Description |
| --- | --- | --- |
| U1 | BUF634 | High-speed unity-gain buffer amplifier |
| U2 | OP77 | Precision operational amplifier (current sense path) |
| U3-U8 | 4N32 | Optocoupler (galvanic isolation, 6 instances) |
| U10A-U10D | MC34074 | Quad operational amplifier (multiple stages) |
| U11A-U11C | INA117 | High common-mode voltage difference amplifier (3 instances) |
| U13 | INA117 | Instrumentation amplifier (voltage sense) |
| U14A-U14D | MC34074 | Quad operational amplifier (control/protection) |
| U15 | 75116 | Differential line driver/receiver |
| U16 | INA117 | Instrumentation amplifier (additional sense path) |
| U19 | CD4044B | Quad RS latch (trip/protection logic) |
| U20 | INA114 | Precision instrumentation amplifier |
| U8 | OP77 | Precision op-amp (SCR phase-control output path) |

## 2.2 Diodes and Zener Diodes

| Ref Des | Part Number | Description |
| --- | --- | --- |
| D1-D12 | 1N3064 | Signal diodes (protection/clamping, multiple) |
| D2 | 1N3064 | Signal diode |
| D3 | 1N3064 | Signal diode |
| D5 | 1N4742 | 12V Zener diode (reference) |
| D6 | 1N3064 | Signal diode |
| D8 | 1N4747 | 20V Zener diode |
| D17 | 1N4740 | 10V Zener diode |
| D20 | 1N4728 | 3.3V Zener diode (Power ON LED indicator) |
| DS1 | RED LED | Over-Current indicator |
| DS2 | RED LED | Over-Voltage indicator |
| DS3 | RED LED | Manual Trip indicator |
| DS4 | YEL LED | Current Limit indicator |
| DS5 | YEL LED | Voltage Limit indicator |
| DS6 | RED LED | Under-voltage lockout indicator |

## 2.3 Resistors

| Ref Des | Value | Notes |
| --- | --- | --- |
| R1 | N/U | Not used (see Notes) |
| R2 | 1K | Current sense path |
| R4 | 10.50K | Voltage feedback (not used for NLCTA) |
| R8 | 10.00K | Current sense return |
| R10 | 10.00K | Current sense amplifier |
| R13-R16 | 43K | LED indicator resistors (4x) |
| R17 | 10K | Voltage sense path |
| R18 | 10K | Current sense path |
| R20 | N/U (PEP II) / 5.6K (NLCTA) | Gain selection: Gain=1 (PEP II) / Gain=10 (NLCTA) |
| R23 | 10.00K | Op-amp feedback |
| R24 | 43K | Bias resistor |
| R28 | 10K | Feedback divider |
| R29 | 10.00K | Precision feedback |
| R32 | 1K | Output series |
| R33 | 10K | Feedback |
| R34 | 10K | Feedback |
| R36 | 10K | Feedback |
| R37 | 41K | Feedback network |
| R39 | 10K | Input network |
| R40 | 10.00K | Precision feedback |
| R41 | 10.00K | Precision feedback |
| R42 | 100K | Feedback |
| R43 | 5K | Current limit adjust |
| R44 | 1K | Series resistor |
| R45 | 10K | Feedback |
| R46 | 10K | Feedback |
| R47 | 100K | Voltage monitor path |
| R49 | 10K | Auto-reset network |
| R50 | 100K | Feedback |
| R51 | 300K | High impedance path |
| R52 | 250K | Adjustable feedback (+30V) |
| R54 | 10.00K | Precision feedback |
| R55 | N/U | Not used |
| R57 | 4.99K | Current limit amplifier |
| R58 | 5K | Current limit adjust |
| R59 | 10K | Input resistor |
| R60 | 10K | Feedback |
| R61 | 10K | Op-amp feedback |
| R62 | 10K | Feedback divider |
| R63 | N/U | Not used |
| R66 | 4.99K | Voltage limit amplifier input |
| R67 | 3K | Output buffer |
| R69 | 10.00K | Precision input |
| R70 | 10K | Feedback |

## 2.4 Capacitors

| Ref Des | Value | Notes |
| --- | --- | --- |
| C1-C3 | 0.1uF 50V | Decoupling (monolithic) |
| C5 | 1uF | Bypass |
| C6 | 10uF | Filter (tantalum) |
| C7 | 0.1uF | Decoupling |
| C10 | 0.1uF | Decoupling |
| C11 | 2uF (PEP II) / 5uF 50V (NLCTA) | Under-voltage lockout filter |
| C14 | 0.001uF | High-frequency bypass |
| C15 | 0.1uF | Decoupling |
| C17 | 0.001uF | High-frequency bypass |
| C18 | 10uF | Filter |
| C19 | 0.1uF | Decoupling |
| C20 | 0.1uF | Decoupling |
| C21 | 0.001uF | High-frequency bypass |
| C22 | 0.001uF | High-frequency bypass |
| C23 | 10uF | Filter |
| C24 | 0.1uF | Decoupling |
| C26 | 0.1uF | Decoupling |
| C28 | 0.1uF | Decoupling |
| C31 | 2uF | Filter |
| C32 | 10uF | Auto-reset filter |
| C33 | 0.1uF | Decoupling |
| C34 | 0.1uF | Decoupling |
| C37 | 0.001uF | High-frequency bypass |
| C38 | 0.1uF | Decoupling |
| C42 | 0.1uF | Output decoupling |
| C44 | 500pF 15V | High-frequency compensation |
| C48 | 0.1uF | Decoupling |

## 2.5 Connectors and Jumpers

| Ref Des | Type | Description |
| --- | --- | --- |
| J1A-J1B | Board connector | Voltage/current sense input |
| J2A-J2H | Board connector | Multi-section output connector |
| J3 | BNC | External test/monitor connector |
| J4D | Connector | INH/-SIG-HI output |
| J4H | Connector | +30VDC input |
| JP1-JP12 | JMP-2P/JMP-3P | Configuration jumpers (12 total) |
| TP3 | Test point | +15V supply monitoring |
| TP4 | Test point | Negative voltage monitoring |
| TP6 | Test point | -15V supply monitoring |
| TP8 | Test point | Manual trip monitoring |
| TP9 | Test point | Power-on status |
| TP10 | Test point | Ground reference |
| TP12 | Test point | Trip output monitoring |

# 3. Power Supply Architecture

| Rail | Source | Notes |
| --- | --- | --- |
| +15V | On-board regulator from +30V input | Primary positive supply |
| -15V | On-board regulator | Primary negative supply |
| +30V | External via J4H | High-voltage rail for output stages |
| GND/COM | Board ground | System ground reference |

# 4. Functional Circuit Descriptions

## 4.1 Voltage Limit Amplifier

The voltage limit amplifier uses INA117 (U13) with R66 (4.99K) input resistor and C44 (500pF 15V) compensation capacitor. The POS. VOLTAGE LIMIT COMMAND input sets the maximum output voltage. The amplifier compares the command against the voltage feedback and drives the SCR control to limit output voltage. R37 (41K) provides the feedback network to U13.

## 4.2 Voltage Difference Amplifier

The voltage difference amplifier stage uses INA117 (U11A) with precision feedback resistors R23 (10.00K). The NEG. VOLTAGE SENSE input comes through JMP-3P jumper configuration. Op-amp U10A (MC34074) provides gain stage. C5 (1uF) and R4 (10.50K) set the time constant. The output feeds the over-voltage trip comparator.

## 4.3 Current Limit Amplifier

The current limit amplifier uses R57 (4.99K) and R58 (5K) in the input network. The POS. CURRENT LIMIT COMMAND sets the threshold. Op-amp stages in U14 (MC34074) process the current feedback signal. INA117 (U11C) provides the differential measurement. The output drives the current limit indicator DS4 (YEL LED).

## 4.4 Current Difference Amplifier

Uses INA114 (U20) or INA117 configuration for precision current measurement. R59 (10K), R69 (10.00K), and feedback resistors set the gain. The isolated current preamp section uses OP77 (U2) with feedback from R54 (10.00K) and C24 (0.1uF). POS. CURRENT SENSE MONITOR connects through BUF634 (U1) buffer.

## 4.5 Over-Voltage Trip

U10A (MC34074) comparator monitors the voltage against the set limit. When triggered, the output drives through 1N3064 diodes to the CD4044B (U19) RS latch, which latches the trip condition. DS2 (RED LED) indicates over-voltage trip status. The trip signal inhibits SCR gate signals.

## 4.6 Over-Current Trip

U10B (MC34074) comparator monitors current against the set limit. R43 (5K) and R44 (1K) set thresholds. CD4044B (U19) latches the trip. DS1 (RED LED) indicates over-current condition. 1N4747 (D8, 20V) provides voltage clamping. C23 (10uF) provides filtering.

## 4.7 Manual Trip

Manual trip input through R32 (1K) and D3 (1N3064). The signal latches in CD4044B (U19). DS3 (RED LED) indicates manual trip status. TP8 monitors the manual trip node.

## 4.8 Under-Voltage Lockout

R47 (100K) and C11 (2uF for PEP II, 5uF for NLCTA) form the under-voltage detection time constant. DS6 (RED LED, labeled AV RED) indicates under-voltage condition. 1N3064 diodes provide logic OR function with other trip sources.

## 4.9 Voltage/Current/Manual Trip and Soft-Start

The trip and soft-start logic block combines all trip sources (over-voltage, over-current, manual trip) and the soft-start sequencing. U14B (MC34074) provides the soft-start ramp generation with C32 (10uF) and R49 (10K). JP7 (JMP-2P) provides auto-reset configuration. D12 (1N3064) provides directional control. R51 (300K) sets the soft-start time constant.

## 4.10 SCR Phase-Control Output

The SCR phase-control output stage uses OP77 (U8) for precision output. D17 (1N4740, 10V) provides voltage clamping. R52 (250K) feedback to +30V rail. C42 (0.1uF) output filter. The INH/-SIG-HI output at J4D provides the SCR gate/inhibit command. JP10 (JMP-3P) configures the output mode.

## 4.11 SCR Gate Inhibit

R67 (3K) connects the SCR GATE INHIBIT COMMAND through 1N3064 diodes to the CD4044B (U19) logic. When asserted, all SCR gate signals are inhibited, providing an emergency shutdown path.

## 4.12 Isolated Current Preamp

The isolated current preamp uses 4N32 optocouplers (U3-U8) for galvanic isolation between the high-voltage sensing side and the control electronics. 1N4728 (3.3V zener) provides reference. The preamp output feeds the current difference amplifier through precision resistors.

## 4.13 Power-On Indicator

D20 (1N4728, 3.3V zener) with green LED indicates power-on status. TP9 monitors power-on state.

# 5. Configuration Options

## 5.1 PEP II Configuration (2MW PEP II P/S)

| Jumper | PIN1 | PIN2 | PIN3 |
| --- | --- | --- | --- |
| JP1 | Connected | Connected | - |
| JP2 | Connected | Connected | - |
| JP3 | Connected | - | - |
| JP4 | Connected | Connected | - |
| JP5 | Connected | Connected | - |
| JP6 | Connected | - | - |
| JP7 | Connected | - | - |
| JP8 | Connected | - | - |
| JP9 | Connected | - | Connected |
| JP10 | Connected | - | Connected |
| JP11 | Connected | - | - |
| JP12 | Connected | - | - |

Notes: R20=Not Used (Gain of 1); standard component values for 2MW operation

## 5.2 NLCTA Configuration (NLCTA P/S)

Key differences from PEP II: R4 not used; C11=5uF 50V; R20=5.6K (Gain of 10); modified jumper connections for NLCTA operation.

# 6. Design Notes

NOTES (FOR 2MW PEP II P/S) UNLESS OTHERWISE SPECIFIED:

1. N/U = Not Used, No Connection

2. Resistors are in Ohms

3. Capacitors are in uF, Monolithic, 50V, 10% Tolerance

4. Polarizing capacitors are in uF, 25V, Dipped Tantalum

5. R20=Not Used for Gain of 1

6. Jumper connections as specified in configuration tables

# 7. AI Replication Checklist

| # | Item | Key Parameters | Section |
| --- | --- | --- | --- |
| 1 | Power supply rails | +15V, -15V, +30V, GND | Sec 3 |
| 2 | Voltage limit amplifier | INA117 (U13), R66(4.99K), C44(500pF) | Sec 4.1 |
| 3 | Voltage difference amplifier | INA117 (U11A), MC34074 (U10A), R23(10K) | Sec 4.2 |
| 4 | Current limit amplifier | R57(4.99K), R58(5K), MC34074 (U14) | Sec 4.3 |
| 5 | Current difference amplifier | INA114/INA117 (U20), OP77 (U2) | Sec 4.4 |
| 6 | Over-voltage trip | MC34074 (U10A), CD4044B (U19), DS2 RED | Sec 4.5 |
| 7 | Over-current trip | MC34074 (U10B), CD4044B (U19), DS1 RED | Sec 4.6 |
| 8 | Manual trip | R32(1K), CD4044B (U19), DS3 RED | Sec 4.7 |
| 9 | Under-voltage lockout | R47(100K), C11(2uF/5uF), DS6 RED | Sec 4.8 |
| 10 | Trip/soft-start logic | U14B, C32(10uF), R49(10K), R51(300K) | Sec 4.9 |
| 11 | SCR phase-control output | OP77 (U8), D17(1N4740,10V), R52(250K) | Sec 4.10 |
| 12 | SCR gate inhibit | R67(3K), 1N3064 diodes, CD4044B | Sec 4.11 |
| 13 | Isolated current preamp | 4N32 (U3-U8), 1N4728 reference | Sec 4.12 |
| 14 | Optocoupler isolation | 6x 4N32 instances | Sec 4.12 |
| 15 | Power-on indicator | D20(1N4728,3.3V), GRN LED | Sec 4.13 |
| 16 | Jumper configuration | JP1-JP12 per PEP II or NLCTA table | Sec 5 |
| 17 | Test points | TP3,TP4,TP6,TP8,TP9,TP10,TP12 | Sec 2.5 |

END OF TECHNICAL DESIGN EXTRACTION NOTE - SD-237-230-14-C1