TECHNICAL DESIGN EXTRACTION NOTE

PEP II 2MW Klystron Power Supply - Grounding Tank Schematic

Drawing No.: SD-730-790-05-C1  |  SLAC / Stanford University

# 1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-730-790-05-C1 |
| Title | PEP II 2MW Klystron Power Supply - Grounding Tank |
| Organization | Stanford Linear Accelerator Center (SLAC) |
| Location | Stanford University, Stanford, California |
| Sheet | 1 of 1 |
| Engineer | R. Cassel |
| Drafter | W. Corporon |
| Checker | J. Olszewski |
| Revision | C1 |
| Drawing Class | Grounding Tank Assembly Schematic |

## 1.1 Revision History

| Rev | Description | Date |
| --- | --- | --- |
| C1 | Revised per engineering changes |  |

# 2. System Overview

This schematic shows the grounding tank assembly for the PEP II 2MW klystron power supply. The grounding tank contains the high-voltage filter components, current sensing, switching devices, and safety interlock systems. It serves as the interface between the rectifier output and the klystron load.

# 3. Complete Bill of Materials

## 3.1 High-Voltage Components

| Ref Des | Description | Rating / Value |
| --- | --- | --- |
| CT1 (+) | Current Transformer, positive output | DC-CT (Danfysik) |
| D1 | High-Voltage Diode | HVT-2 |
| HVT-G | High-Voltage Termination, Ground | 50 OHM 90KV |
| L1 | Filter Inductor | 350 UHY 40A |
| L2 | Filter Inductor | 350 UHY 40A |
| C3 | Filter Capacitor | 30 NFD 37KV |
| C4 | Filter Capacitor | 10 NFD 56KV |
| C5 (spare) | Spare Filter Capacitor | 30 NFD 37KV |
| VT | Voltage Transducer | 25KV 100A |
| S1 | Shunt Resistor | 15A/50MV |

## 3.2 Switches and Indicators

| Ref Des | Description | Type / Notes |
| --- | --- | --- |
| SW1 | Switch 1 | ROSS-GRN-SW (Green) |
| SW2 | Switch 2 | ROSS-GRN-SW (Green) |
| LEV3 | Oil Level Indicator | OIL-LEVEL sensor |
| F2 | Fuse | Connector power protection |

## 3.3 Connectors

| Ref Des | Part Number | Description |
| --- | --- | --- |
| J1 | MS3102R18-1P | Multi-pin circular connector (main interface) |
| P1 | Mating connector | Power supply interface |

# 4. Functional Circuit Description

## 4.1 High-Voltage Filter Section

The grounding tank contains an LC filter network for smoothing the rectified high-voltage output. Two inductors (L1, L2) rated at 350 UHY 40A are connected in series with the output. Filter capacitors C3 (30 NFD 37KV), C4 (10 NFD 56KV), and spare C5 (30 NFD 37KV) provide energy storage and ripple reduction.

## 4.2 Current Sensing

CT1 is a Danfysik DC current transformer providing the positive output current measurement. A shunt resistor S1 (15A/50MV) provides an additional current measurement path. The +OUT FOR POS CURRENT signal routes to the power supply control system for regulation feedback at 10A/V sensitivity.

## 4.3 Voltage Sensing

VT (Voltage Transducer) is rated at 25KV 100A and provides voltage feedback. The HVT-2 (D1) and HVT-G (50 OHM, 90KV) provide high-voltage termination and grounding reference.

## 4.4 Safety and Status Indicators

Multiple indicator LEDs and switches provide status: SW1/SW2 are ROSS green switches for interlock status. Oil level sensor (LEV3) monitors the insulating oil level in the grounding tank. Indicator lights include GRN (power), RED (fault), and WAR (warning) status.

# 5. Connector Pinout - J1 (MS3102R18-1P)

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | +240V | Positive 240V supply |
| 2 | +20V | Positive 20V supply |
| 3 | GND | Ground reference |
| 4 | +5V | Positive 5V supply |
| 5 | AC OUT | AC output |
| 6 | AC OUT | AC output (second) |
| Other | Various status/interlock | LED drives, switch status |

# 6. Signal Net List

| Net Name | Connected To | Type |
| --- | --- | --- |
| +OUT FOR POS CURRENT | CT1 output to power supply | Analog (10A/V) |
| +OUT FOR VOLTAGE | VT output to power supply | Analog |
| OIL-LEVEL | LEV3 sensor | Status |
| PWR GRN | Power indicator LED | Status |
| GND | System ground | Ground |
| +240V | Supply rail | Power |
| +20V | Supply rail | Power |
| +5V | Supply rail | Power |

# 7. Power Distribution

| Rail | Source | Loads |
| --- | --- | --- |
| +240V | External supply via J1 | Indicator circuits |
| +20V | External supply via J1 | Control circuits |
| +5V | External supply via J1 | Logic/sensors |
| GND | External supply via J1 | System reference |

# 8. Design Notes

- The grounding tank houses the high-voltage filter and sensing components in an oil-filled enclosure

- Oil level monitoring (LEV3) is critical for safe operation - low oil can cause arcing

- The dual inductor configuration (L1+L2, 350 UHY each) provides enhanced filtering

- High-voltage termination (HVT-G, 50 OHM 90KV) provides proper impedance matching

- Current sensing uses both a DC-CT (Danfysik) and shunt (15A/50MV) for redundancy

- Fuse F2 provides overcurrent protection for low-voltage circuits

- MS3102R18-1P military-style connector ensures robust electrical connection

# 9. AI Replication Checklist

| # | Item | Key Parameters |
| --- | --- | --- |
| 1 | Filter Inductors L1, L2 | 350 UHY 40A each, series connected |
| 2 | Filter Capacitors C3, C5 | 30 NFD 37KV |
| 3 | Filter Capacitor C4 | 10 NFD 56KV |
| 4 | DC Current Transformer CT1 | Danfysik DC-CT, +output |
| 5 | Voltage Transducer VT | 25KV 100A |
| 6 | HV Diode D1 | HVT-2 type |
| 7 | HV Termination HVT-G | 50 OHM 90KV |
| 8 | Shunt S1 | 15A/50MV |
| 9 | Connector J1 | MS3102R18-1P |
| 10 | Oil Level Sensor LEV3 | OIL-LEVEL type |
| 11 | Switches SW1, SW2 | ROSS-GRN-SW |
| 12 | Fuse F2 | Overcurrent protection |

END OF TECHNICAL DESIGN EXTRACTION NOTE - SD-730-790-05-C1