TECHNICAL DESIGN EXTRACTION NOTE

PEP II RF System - 2MW Klystron Power Supply SCR Crowbar Driver Board

Drawing No.: SD-730-793-04-C2  |  SLAC / Stanford University

# 1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-730-793-04-C2 |
| Title | SCR Crowbar Driver Board |
| System | PEP II RF System, 2MW Klystron Power Supply |
| Organization | Stanford Linear Accelerator Center (SLAC) |
| Location | Stanford University, Stanford, California |
| Sheet | 1 of 1 |
| CAD File | 73079304.DDB |
| Engineer | M. Nguyen / R. Cassel |
| Drafter | M. Nguyen |
| Checker | J. Olszewski |
| Revision | C2 |

## 1.1 Revision History

| Rev | Description | By | Date |
| --- | --- | --- | --- |
| C2 | REV'D TO REFLECT UNUSED COMPONENTS ON THE BOARD | WSG/JO/MN | 03/13/03 |

# 2. System Overview

The SCR Crowbar Driver Board generates gate drive pulses for the crowbar thyristor protection circuit. When a fault is detected, it fires the crowbar SCR to short-circuit the high-voltage output, protecting the klystron from damage. It accepts a single-shot trigger input and generates properly timed crowbar firing pulses, as well as an optical SCR trigger output for isolated slave trigger boards.

# 3. Complete Bill of Materials

## 3.1 Integrated Circuits

| Ref Des | Part Number | Description |
| --- | --- | --- |
| U1A/U1B | CD4047B | CMOS monostable/astable multivibrator (two stages) |
| U2A/U2B | CD4538 | CMOS dual monostable multivibrator |
| U3A-U3F | CD4049 | CMOS hex inverting buffer |
| U5 | MIC-4451 | High-speed MOSFET gate driver |
| U6A/U6B | CD4538 | CMOS dual monostable multivibrator (second) |
| U8 | MIC-4427 | Dual low-side MOSFET driver (optical SCR trigger) |

## 3.2 Power MOSFETs

| Ref Des | Part Number | Description |
| --- | --- | --- |
| Q1 | IXFH12N90 | N-channel power MOSFET (900V, 12A) |
| Q2 | IXFH12N90 | N-channel power MOSFET (900V, 12A) |

## 3.3 Diodes

| Ref Des | Part Number | Description |
| --- | --- | --- |
| D1/D2 | Standard | Signal diodes |
| D3/D4 | 51 1/2W | Zener diodes (gate clamping) |
| D5 | MR856 | Fast-recovery rectifier |
| D7/D8 | 1N4744A | 15V Zener diodes (MOSFET gate protection) |
| D10 | MR856 | Fast-recovery rectifier |

## 3.4 LEDs

| Ref Des | Part Number | Description |
| --- | --- | --- |
| LED1 | HLMP-1700 | Status LED |
| LED2 | MV 5474C | HV Power LED |
| LED3 | HLMP-1700 | Status LED |
| LED4 | HLMP-1700 | Trigger active LED |

## 3.5 Key Resistors

| Ref Des | Value | Notes |
| --- | --- | --- |
| R1 | Variable | Timing adjust for 50mS (adjustable) |
| R2 | Variable | Timing adjust for 44uS (adjustable) |
| R9 | 1K | LED current limiting |
| R10 | 2K 1W | Power indicator |
| R14 | 5.1K | Gate pull-down |
| R17 | 100K | Input resistor |
| R18/R19 | 24.9K | Timing resistors (5uS period) |
| R20 | 2.2K | Timing resistor |
| R21 | 18K | Bias resistor |
| R23 | 500 | Current limiting |
| R26 | 1K | Bias resistor |
| R37 | 24.9K | Timing resistor (5uS) |

## 3.6 Key Capacitors

| Ref Des | Value | Notes |
| --- | --- | --- |
| C6/C7 | 4.7nF | Snubber capacitors |
| C10 | 10uF | Power supply filter |
| C15 | 0.001uF | Bypass |
| C17/C19 | 10uF | Power supply filter |
| C18 | Various | Timing capacitor |
| Various | 200pF | Timing caps for CD4538 |

# 4. Functional Circuit Descriptions

## 4.1 Crowbar Trigger Input

Single-shot trigger input from the fault detection system. The CD4047B (U1A/U1B) monostable stages generate the initial crowbar timing pulse. R1 adjusts the 50mS pulse width, R2 adjusts the 44uS sub-pulse.

## 4.2 Timing Chain

CD4538 (U2A/U2B, U6A/U6B) dual monostable multivibrators generate precision timing. Timing diagram with single-shot trigger shows: 50mS main enable (adjustable by R1), 5mS gate pulse, 44uS sub-pulse (adjustable by R2), 15-60mS range, 20-120uS range. Multiple 5uS pulses at 24.9K resistor timing.

## 4.3 MOSFET Power Stage

Same architecture as the SCR Driver Board: MIC-4451 (U5) drives IXFH12N90 MOSFETs (Q1/Q2) through 1N4744A zener-protected gates. MR856 fast diodes provide output rectification.

## 4.4 Optical SCR Trigger Output

MIC-4427 (U8) drives an optical SCR trigger output plug for connection to old slave trigger boards. This provides galvanic isolation via fiber-optic link. 5uS pulse output at connector.

# 5. Timing Diagram (Single-Shot Trigger)

TP1: Trigger input (single-shot)

TP4: 50mS main enable (adj. by R1)

TP2: Gate pulse output

TP4-TP5: MOSFET drive signals

- 5mS gate pulse width

- 44uS sub-pulse (adj. by R2)

- Optical output: 5uS pulses

# 6. Design Notes

- This board is designed to drive the optical SCR trigger for the crowbar thyristor

- Output plug connects to old slave trigger board

- Board is variant of the SCR Driver Board (SD-730-793-03) adapted for crowbar application

- Single-shot operation (not continuous like SCR driver)

# 7. AI Replication Checklist

| # | Item | Key Parameters |
| --- | --- | --- |
| 1 | CD4047B monostables | U1A/B, crowbar timing |
| 2 | CD4538 dual monostables | U2/U6, 200pF timing caps |
| 3 | CD4049 hex inverters | U3A-F, signal conditioning |
| 4 | MIC-4451 MOSFET driver | U5, high-speed gate drive |
| 5 | MIC-4427 optical driver | U8, fiber-optic SCR trigger |
| 6 | IXFH12N90 MOSFETs | Q1/Q2, 900V 12A |
| 7 | 1N4744A gate protection | D7/D8, 15V zener |
| 8 | Timing: 50mS/5mS/44uS/5uS | Crowbar pulse params |
| 9 | R18/R19/R37 = 24.9K | 5uS timing resistors |
| 10 | Optical output plug | To slave trigger board |

END OF TECHNICAL DESIGN EXTRACTION NOTE - SD-730-793-04-C2