TECHNICAL DESIGN EXTRACTION NOTE

PEP II RF System - SCR Control Driver / Right Side Trigger Interconnect Board

Drawing No.: SD-730-793-07-C2  |  SLAC / Stanford University

# 1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-730-793-07-C2 |
| Title | SCR Control Driver - Right Side Trigger Interconnect Board |
| System | PEP II RF System, 2MW Klystron Power Supply |
| Organization | Stanford Linear Accelerator Center (SLAC) |
| Location | Stanford University, Stanford, California |
| Sheet | 1 of 1 |
| CAD File | 73079307.DDB |
| Engineer | M. Nguyen |
| Drafter | M. Nguyen |
| Checker | J. Olszewski |
| Revision | C2 |

## 1.1 Revision History

| Rev | Description | By | Date |
| --- | --- | --- | --- |
| C2 | Change R19 to 1K OHM per ECO A0763 | WSG/JO/MN | 09/03 |

# 2. System Overview

The Right Side Trigger Interconnect Board is the central control and monitoring hub for the right-side SCR trigger circuits in the 2MW klystron power supply. It provides SCR trigger generation, monitoring of all three phases (+A/+B/+C and -A/-B/-C), fault/enable logic, crowbar control, transformer arc detection, fiber-optic status communication, and 24V fault/enable interface to the left-side trigger interconnect board.

# 3. Complete Bill of Materials

## 3.1 Integrated Circuits

| Ref Des | Part Number | Description |
| --- | --- | --- |
| U1A/U1B | CD4538 | Dual monostable multivibrator (crowbar timing) |
| U2A/U2B | CD4538 | Dual monostable multivibrator (SCR enable timing) |
| U3A | CD4538 | Monostable (HV status) |
| U4A/U4B | CD4075 | Triple 3-input OR gate |
| U5A-U5C | MIC-4427 | Dual MOSFET driver (SCR triggers, 3 instances) |
| U6A/U6B | CD4001 | Quad NOR gate (logic) |
| U7A/U7B | CD4538 | Dual monostable (monitoring) |
| U8A/U8B | CD4538 | Dual monostable (monitoring) |
| U9A/U9B | CD4538 | Dual monostable (monitoring) |
| U10A/U10B | CD4538 | Dual monostable (monitoring) |
| U11A/U11B | CD4538 | Dual monostable (monitoring) |
| U12A-U12C | CD4538 | Dual monostable (monitoring, 3 sections) |
| U13A/U13B | CD4538 | Dual monostable (monitoring) |
| HCPL-4503 | HCPL-4503 | High-speed optocoupler (fiber-optic interface) |
| HFBR-2412 | HFBR-2412 | Fiber-optic receiver (crowbar status) |
| HFBR-1412 | HFBR-1412 | Fiber-optic transmitter (status output) |

## 3.2 Discrete Components

| Ref Des | Part Number / Value | Description |
| --- | --- | --- |
| Q1-Q3 | IRFD110 | N-channel MOSFETs (SCR trigger switching) |
| D1-D11 | 1N3064 | Signal diodes |
| D3 | 1N4734 | 5.6V Zener |
| D8 | 1N4734 | 5.6V Zener |
| P3KE7 | P3KE7 | TVS diode (transient voltage suppressor) |
| LED (various) | HLMP-1700/1719/1790 | Status indicator LEDs |

## 3.3 Key Resistors

| Ref Des | Value | Notes |
| --- | --- | --- |
| R1-R5 | 1K | Various bias/current limiting |
| R6/R7 | 1K | MOSFET bias |
| R12 | 108K | Timing (1 second) |
| R15/R30 | 100K | Timing resistors |
| R16/R18/R23 | 100/3K/1K | Various |
| R17 | 1K (per ECO A0763) | Changed from previous value |
| R19 | 1K | Per ECO A0763 |
| R20 | 100K | Power-on timing |
| R24 | Various | Timing |
| R25 | 3K | Bias |
| R26 | 33K | Bias |
| R29 | 237K | Crowbar ready timing |
| R33 | 35K | Bias |
| R38 | 1K | Output limiting |
| RP1-RP6 | Various | Resistor packs for monitoring |

## 3.4 Key Capacitors

| Ref Des | Value | Notes |
| --- | --- | --- |
| C2 | 10uF | Power supply filter |
| C9-C11 | 0.001uF | Bypass/timing |
| C13-C16 | Various | Monitoring monostable timing |
| C14/C30/C31 | 0.1uF/10uF | Filter/bypass |
| C21/C26 | 0.1uF | Decoupling |
| C24/C27/C30/C34 | Various | Monitoring timing |
| C35 | 0.1uF | Filter |

## 3.5 Connectors

| Ref Des | Description | Notes |
| --- | --- | --- |
| P1-P12 | MIC-4427 output headers | SCR trigger outputs to driver boards |
| BNC | BNC connector | Klystron arc trigger / transformer arc trigger |
| JP1 | JMP-3P | SCR monitor bypass (normal/bypassed) |
| IDC14 | IDC14 ribbon | External interface |

# 4. Functional Circuit Descriptions

## 4.1 SCR Trigger Generation

Three MIC-4427 (U5A-C) dual MOSFET drivers generate the SCR trigger pulses for all six SCR phases (+A, +B, +C, -A, -B, -C). Each driver channel feeds an isolated trigger output through connectors P1-P12 to the SCR driver boards. IRFD110 MOSFETs (Q1-Q3) switch the +12V to generate gate pulses.

## 4.2 Phase Monitoring

Six monitoring channels track the SCR firing for each phase: +A MON, +B MON, +C MON, -A MON, -B MON, -C MON. Each channel uses CD4538 monostable multivibrators (U7-U12) with 20mS timing. The monitoring outputs feed an 8-input NAND gate for system health indication. JP1 (JMP-3P) selects between NORMAL and BYPASSED monitoring modes.

## 4.3 Fault/Enable Logic

The 24V FAULT/ENABLE bus interfaces to the left-side trigger interconnect board. CD4001 quad NOR gates (U6A/B) and CD4075 triple OR gates (U4A/B) combine fault conditions. D3/D8 (1N4734, 5.6V) provide voltage clamping. The fault logic controls the SCR enable signal and crowbar trigger.

## 4.4 Klystron Arc Detection

A BNC connector accepts the klystron arc trigger signal. P3KE7 TVS diode provides transient protection. R26 (33K) and R33 (35K) condition the signal. The arc detection feeds into the fault logic to trigger the crowbar protection. A similar circuit handles transformer arc detection.

## 4.5 Crowbar Control

CD4538 (U1A/B) provides crowbar timing. R29 (237K) sets the 1-second CBREADY timing. HFBR-2412 fiber-optic receiver receives crowbar status. CD4075 (U4B) OR gate combines crowbar triggers. The output drives the crowbar SCR driver board. HLMP-1700 LED indicates crowbar ready status.

## 4.6 Fiber-Optic Status

HCPL-4503 optocoupler provides high-speed isolated status. HFBR-1412 fiber-optic transmitter sends SCR ENABLE and status information. HFBR-2412 receiver receives crowbar status from remote boards. R7 (1K) and R38 (1K) provide current limiting.

## 4.7 Power-On Sequencing

R20 (100K) and associated timing components provide power-on delay. D3 (HLMP-1790) power-on LED indicator. +12V supply distributed to all subsystems through multiple decoupling capacitors.

## 4.8 HV Status Monitoring

CD4538 (U3A) monitors the HV (high-voltage) status with associated timing network. The HV BON (HV Bus ON) signal indicates system readiness. IDC14 connector provides interface to external monitoring systems.

# 5. Signal Net List

| Net Name | Type | Description |
| --- | --- | --- |
| +12V | Power | Primary logic supply |
| COM/GND | Ground | System ground reference |
| +A MON / +B MON / +C MON | Monitor | Positive phase SCR monitoring |
| -A MON / -B MON / -C MON | Monitor | Negative phase SCR monitoring |
| 24V FAULT/ENABLE | Control | Fault/enable bus to left-side board |
| KLYSTRON ARC TRIGGER | Fault | Arc detection input (BNC) |
| TRANSFORMER ARC TRIGGER | Fault | Transformer arc detection (BNC) |
| CBREADY | Status | Crowbar ready signal |
| SCR ENABLE MON | Monitor | SCR enable monitoring output |
| SCR TRIGGERS | Output | Gate drive outputs to P1-P12 |
| HV BON | Status | High-voltage bus on indicator |
| F.O. STATUS | Fiber-optic | HFBR-1412 status output |
| F.O. KLYSTRON CROWBAR | Fiber-optic | HFBR-2412 crowbar input |
| POWER ON | Status | D3 HLMP-1790 LED |

# 6. AI Replication Checklist

| # | Item | Key Parameters |
| --- | --- | --- |
| 1 | MIC-4427 SCR triggers | 3x U5A-C, 6-phase output to P1-P12 |
| 2 | IRFD110 switch MOSFETs | Q1-Q3, SCR pulse switching |
| 3 | CD4538 monitoring | 6 channels, 20mS timing, U7-U12 |
| 4 | CD4001/CD4075 fault logic | U6/U4, fault combination |
| 5 | Crowbar control | CD4538 U1, R29(237K) 1sec timing |
| 6 | Fiber-optic I/O | HCPL-4503, HFBR-1412/2412 |
| 7 | Arc detection | BNC inputs, P3KE7 TVS, R26/R33 |
| 8 | Phase monitoring | 8-input NAND, JP1 bypass |
| 9 | Power supply | +12V, decoupling caps throughout |
| 10 | 24V fault/enable bus | Interface to left-side board |

END OF TECHNICAL DESIGN EXTRACTION NOTE - SD-730-793-07-C2