TECHNICAL DESIGN EXTRACTION NOTE

PEP II RF System - 2MW Klystron Power Supply HV Power Circuit Modification

Drawing No.: SD-730-793-13-C1  |  SLAC / Stanford University

# 1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-730-793-13-C1 |
| Title | HV Power Circuit Modification |
| System | PEP II RF System, 2MW Klystron Power Supply |
| Organization | Stanford Linear Accelerator Center (SLAC) |
| Location | Stanford University, Stanford, California |
| Sheet | 1 of 1 |
| Engineer | M. Nguyen |
| Drafter | M. Nguyen |
| Checker | J. Olszewski |
| Revision | C1 |
| Date | 12/06 |

## 1.1 Revision History

| Rev | Description | By | Date |
| --- | --- | --- | --- |
| C1 | HV POWER CIRCUIT MODIFICATION | GKL/JA | 12/06 |

# 2. System Overview

This schematic documents a modification to the HV power circuit for the 2MW klystron power supply. It is a variant of the SCR driver/crowbar driver boards (SD-730-793-03/04) adapted for modified HV power circuit operation. The board uses the same fundamental architecture - CD4047B/CD4538/CD4049 timing chain, MIC-4451 MOSFET drivers, and IXFH12N90 power MOSFETs - but with modified timing parameters and component values for the specific HV power circuit application.

# 3. Complete Bill of Materials

## 3.1 Integrated Circuits

| Ref Des | Part Number | Description |
| --- | --- | --- |
| U1 | CD4047B | CMOS monostable/astable multivibrator |
| U2A/U2B | CD4538 | CMOS dual monostable multivibrator |
| U3A-U3F | CD4049 | CMOS hex inverting buffer |
| U5 | MIC-4451 | High-speed MOSFET gate driver |
| U6A/U6B | CD4538 | CMOS dual monostable multivibrator (second) |

## 3.2 Power MOSFETs

| Ref Des | Part Number | Description |
| --- | --- | --- |
| Q1 | IXFH12N90 | N-channel power MOSFET (900V, 12A) |
| Q2 | IXFH12N90 | N-channel power MOSFET (900V, 12A) |

## 3.3 Diodes

| Ref Des | Part Number | Description |
| --- | --- | --- |
| D1/D2 | Standard | Signal diodes |
| D3/D4 | 51 1/2W | Zener diodes |
| D5 | MR856 | Fast-recovery rectifier |
| D7/D8 | 1N4744A | 15V Zener (MOSFET gate protection) |
| D10 | MR856 | Fast-recovery rectifier |

## 3.4 LEDs

| Ref Des | Part Number | Description |
| --- | --- | --- |
| LED1 | HLMP-1700 | Status LED |
| LED2 | MV 5474C | HV Power LED |
| LED3 | HLMP-1700 | Status LED |
| LED4 | 6N-139 | Optocoupler LED / 500mA output |

## 3.5 Key Resistors (Modified Values)

| Ref Des | Value | Notes |
| --- | --- | --- |
| R3 | 49.9K | Modified timing (was different in SCR driver) |
| R4/R5 | Various | Gate drive series |
| R9 | 1K | LED current limiting |
| R10 | 2K 1W | Power indicator |
| R11 | 1K | Bias |
| R14 | 5.1K | Gate pull-down |
| R17 | 100K | Input |
| R18 | 2.2K | Modified timing resistor |
| R20 | 2.2K | Modified timing resistor (was 22K in SCR driver) |
| R26 | 1K | Bias |
| R37 | 24.9K | Modified timing (5uS) |
| R38 | Various | Timing adjust |

## 3.6 Key Capacitors

| Ref Des | Value | Notes |
| --- | --- | --- |
| C5 | 0.1uF | Modified bypass |
| C6 | 4.7nF | Snubber |
| C8 | 0.001uF | Bypass |
| C10 | 10uF | Power supply filter |
| C15 | 0.001uF | Bypass |
| C17/C19 | 10uF | Power supply filter |
| C18 | 0.1uF | Modified timing |
| Various | 200pF | Timing caps for CD4538 |

## 3.7 Connectors

| Ref Des | Type | Description |
| --- | --- | --- |
| P1 | ADB-P107-06 | HV Power connector |
| P2 | IDC10 | Trigger I/O |

# 4. Functional Circuit Descriptions

## 4.1 Trigger Input

Single-shot trigger input from the test box. The circuit uses 6N-139 optocoupler (LED4) for isolated trigger reception at 500mA drive capability. R17 (100K) provides input conditioning.

## 4.2 Timing Generation

CD4047B (U1) generates the base timing. CD4538 monostables (U2A/B, U6A/B) generate precision pulse widths. Key timing parameters: 50uS main pulse, 44uS sub-pulse (adjustable by R2). R37 (24.9K) and 200pF timing capacitors set the 5uS base period. R3 (49.9K) is a modified value from the standard SCR driver.

## 4.3 MOSFET Gate Drive and Output

MIC-4451 (U5) drives IXFH12N90 power MOSFETs (Q1/Q2) through 1N4744A-protected gates. Same architecture as SCR driver boards. MR856 fast diodes (D5/D10) provide output rectification. HV Power connector P1 (ADB-P107-06) provides the power interface.

## 4.4 Timing Diagram (Single-Shot)

TP1: Trigger input

- 50 uS main pulse

- 44 uS sub-pulse (adj. by R2)

- TP4-TP5: MOSFET drive outputs

# 5. Differences from Standard SCR Driver (SD-730-793-03)

| Parameter | SCR Driver (C4) | HV Power Mod (C1) |
| --- | --- | --- |
| R20 | 22K | 2.2K (10x lower) |
| R18 | Standard | 2.2K (modified) |
| R3 | Standard | 49.9K (modified) |
| LED4 | HLMP-1700 | 6N-139 optocoupler (500mA) |
| Trigger mode | 60Hz continuous | Single-shot |
| Application | Normal SCR firing | HV power circuit mod |
| CD4538 timing | CD4528 | CD4538 (different IC) |

# 6. AI Replication Checklist

| # | Item | Key Parameters |
| --- | --- | --- |
| 1 | CD4047B timing | U1, base timing generation |
| 2 | CD4538 monostables | U2/U6, 200pF timing caps |
| 3 | CD4049 inverters | U3A-F, signal conditioning |
| 4 | MIC-4451 MOSFET driver | U5, gate drive |
| 5 | IXFH12N90 MOSFETs | Q1/Q2, 900V 12A |
| 6 | 1N4744A gate protection | D7/D8, 15V zener |
| 7 | Modified timing | R3(49.9K), R20(2.2K), R37(24.9K) |
| 8 | 6N-139 optocoupler | LED4, isolated trigger input |
| 9 | HV power connector | P1 ADB-P107-06 |
| 10 | Single-shot operation | Modified from 60Hz continuous |

END OF TECHNICAL DESIGN EXTRACTION NOTE - SD-730-793-13-C1