TECHNICAL DESIGN EXTRACTION NOTE

PEP II RF System - 2MW Klystron Power Supply SCR Crowbar Trigger Board

Drawing No.: SD-730-794-04-C0  |  SLAC / Stanford University

# 1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-730-794-04-C0 |
| Title | SCR Crowbar Trigger Board |
| System | PEP II RF System, 2MW Klystron Power Supply |
| Organization | Stanford Linear Accelerator Center (SLAC) |
| Location | Stanford University, Stanford, California |
| Sheet | 1 of 1 |
| CAD File | 73075494.SCH |
| Engineer | R. Cassel |
| Drafter | R. Cassel |
| Checker | J. Olszewski |
| Revision | C0 |

# 2. System Overview

The SCR Crowbar Trigger Board generates the crowbar thyristor gate drive pulses for the fault protection system. This board is part of the 730-794 series (distinct from the 730-793 series). It features MOSFET-based power switching with ferrite bead filtering, CD4047B/CM14528 timing chain, and IRFDP120 power MOSFETs for the crowbar gate drive. It uses +15V power supply (different from the +12V used in other trigger boards).

# 3. Complete Bill of Materials

## 3.1 Integrated Circuits

| Ref Des | Part Number | Description |
| --- | --- | --- |
| U1A/U1B | CD4047B | CMOS monostable/astable multivibrator (two stages) |
| U2A/U2B | CM14528 | CMOS dual monostable multivibrator |
| U3A-U3F | CM-4049 | CMOS hex inverting buffer |
| U6A/U6B | CM14528 | CMOS dual monostable multivibrator (second) |

## 3.2 Power MOSFETs

| Ref Des | Part Number | Description |
| --- | --- | --- |
| Q1 | IRFDP120 | N-channel power MOSFET (crowbar drive) |
| Q2 | IRFDP120 | N-channel power MOSFET (crowbar drive) |
| Q3 | IRFDP120 | N-channel power MOSFET (additional channel) |

## 3.3 Passive Components

| Ref Des | Value | Description |
| --- | --- | --- |
| R1/R2 | Various | Timing resistors |
| R3 | 14.7K | Timing resistor |
| R5/R7 | Various | Gate drive series resistors |
| R9 | 5.1K | Gate pull-down |
| R10 | 2K 1W | Power indicator |
| R12 | 24.3K | Timing resistor |
| R16 | Various | Timing |
| R20 | Various | Timing/bias |
| R37 | 24.9K | Timing resistor |
| L1/L2 | Ferrite bead | 300 ohm impedance (EMI filtering) |
| Ferrite beads | 500 ohm | Additional EMI filtering |
| C1 | 0.1uF | Input filter |
| C7 | Various | Snubber/timing |
| C19 | Various | Power supply filter |
| SIP4 | SIP resistor pack | Resistor network |
| Various | 200pF | Timing caps for CM14528 |

## 3.4 Diodes

| Ref Des | Part Number | Description |
| --- | --- | --- |
| D1/D2 | Standard | Signal diodes |
| D7 | Standard | Protection diode |
| LED2-LED5 | HLMP-1700 | Status LEDs |

## 3.5 Connectors

| Ref Des | Type | Description |
| --- | --- | --- |
| P1 | Power connector | Crowbar drive output |
| P2 | Control connector | Trigger input/status |

# 4. Functional Circuit Descriptions

## 4.1 Crowbar Trigger Input

Trigger input through a 6N-138 optocoupler provides galvanic isolation. R9 (86 OHM) and R14 (100 OHM) condition the signal. Ferrite beads (300 ohm and 500 ohm impedance) filter high-frequency noise on both the input and output paths.

## 4.2 Timing Chain

CD4047B (U1A/U1B) astable/monostable stages generate base timing. CM14528 (U2A/U2B, U6A/U6B) dual monostable multivibrators generate precision pulse widths. Timing capacitors (200pF) and resistors (R12=24.3K, R37=24.9K) set the pulse parameters. CM-4049 (U3A-F) inverters provide signal conditioning. Timing resistors include 14.7K and 24.9K for 5uS pulse generation.

## 4.3 MOSFET Power Stage

Three IRFDP120 MOSFETs (Q1/Q2/Q3) provide the crowbar gate drive. This is different from the IXFH12N90 used in the other SCR driver boards - IRFDP120 is a lower voltage but faster switching device suitable for crowbar application. Ferrite beads on the output provide EMI filtering.

## 4.4 Power Supply

+15V supply (different from +12V used on 793-series boards). MV 5474C LED (LED2) indicates power presence. R10 (2K 1W) provides current limiting. Additional ferrite bead filtering on power rails.

# 5. Key Differences from SD-730-793-04 (SCR Crowbar Driver)

| Parameter | 793-04 (Crowbar Driver) | 794-04 (Crowbar Trigger) |
| --- | --- | --- |
| Power supply | +12V | +15V |
| Power MOSFETs | IXFH12N90 (900V 12A) | IRFDP120 (lower voltage, faster) |
| Monostable ICs | CD4538 | CM14528 (equivalent) |
| MOSFETs count | 2 (Q1/Q2) | 3 (Q1/Q2/Q3) |
| EMI filtering | Minimal | Ferrite beads (300/500 ohm) |
| Optocoupler | MIC-4427 optical output | 6N-138 input optocoupler |
| Series | 730-793 | 730-794 |
| Designer | M. Nguyen | R. Cassel |

# 6. Design Notes

- Part of the 730-794 series (different from 730-793 series boards)

- Uses IRFDP120 MOSFETs instead of IXFH12N90 for faster crowbar response

- Ferrite bead filtering on input/output for EMI rejection

- +15V power supply (not +12V)

- 6N-138 optocoupler provides isolated trigger input

- Three MOSFET channels for redundant crowbar drive

- CM14528 is pin-compatible replacement for CD4528/CD4538

# 7. AI Replication Checklist

| # | Item | Key Parameters |
| --- | --- | --- |
| 1 | CD4047B timing | U1A/B, astable/monostable |
| 2 | CM14528 monostables | U2/U6, 200pF timing caps |
| 3 | CM-4049 inverters | U3A-F, signal conditioning |
| 4 | IRFDP120 MOSFETs | Q1/Q2/Q3, crowbar power stage |
| 5 | 6N-138 optocoupler | Isolated trigger input |
| 6 | Ferrite beads | 300/500 ohm, EMI filtering |
| 7 | Timing resistors | R12(24.3K), R37(24.9K), 5uS |
| 8 | +15V power supply | Different from +12V on 793-series |
| 9 | LED indicators | HLMP-1700 (x3), MV 5474C |
| 10 | SIP4 resistor pack | Network for timing/bias |

END OF TECHNICAL DESIGN EXTRACTION NOTE - SD-730-794-04-C0