# Selection of Default OpAmps and High Current Analog Drivers

# Purpose

# Proposed ICs

OPA189/OPA2189/OPA4189
https://www.ti.com/lit/ds/symlink/opa4189.pdf?ts=1666248383230&ref_url=https%253A%252F%252Fwww.ti.com%252Famplifier-circuit%252Fop-amps%252Fprecision%252Fproducts.html 
OPA388?
BUF634A
https://www.ti.com/lit/ds/symlink/buf634a.pdf?ts=1666329795559&ref_url=https%253A%252F%252Fwww.ti.com%252Fproduct%252FBUF634A 
INA851, INA823, INA849?

## Minh’s ICs from Kicker Control Board

OPA2227  TI dual low noise op amp  (TI recommends OPA2210 as a better upgrade)

LM7171  TI high speed (200 MHz), high output current (100 mA) voltage feedback amplifier

ILQ615  Vishay quad opto-coupler  This has a response time of 2-3μs.  Vishay makes higher speed optocouplers, if required.  For example the 6N137A has a typical delay time of 50 ns.  This is only a single device.  VO2630, VO2631, and VO4661 all have two channels.  The difference between these three are the voltage isolation.  VO2630 has 1000 V/us common mode rejection (CMR).

Broadcom also makes opto-couplers.  The ACSL-6400 is a quad package that has a typical delay time of 50 ns.  The recommended supply voltage is 5 V and the pull-down resistor can be between 330 Ω and 4 kΩ.  The typical input threshold current is 2.7 mA, but to minimize aging effects, a design current of 7 mA is recommended.  Absolute maximum value is 15 mA.

IL610  NVE Corporation passive digital isolator  This works on the basis of a voltage input due to an LdIdt on the input.  This requires a certain minimum drive current and a transition that is guaranteed to be faster than 1 μs.

REC6-2405DRW/H2/A  Recom DC-DC converter

Pi filters with 33 μF capacitors and RL622-471K inductors on the output then 510 Ω and 1 μF low pass filter

REC6-2415SRW_DIP24/A  Recom DC-DC converter

SI8622  Skyworks digital isolators  (TI advertises ISO7721 as a replacement chip; it also has others.)  Do we need a digital isolator?  These work by having an internal high frequency oscillator and require good high frequency layout techniques.  Is this required for our application?  I suggest we just use a fast opto-coupler.

# Application Notes

An Engineer’s Guide to Designing with Precision Amplifiers
https://www.ti.com/lit/eb/slyy209/slyy209.pdf?ts=1666297009906&ref_url=https%253A%252F%252Fwww.ti.com%252Fproduct%252FOPA320 
TIDA-01402
High-Precision Reference Design for Buffering a DAC Signal
file:///C:/tmp/downloads/TIDUCT8.PDF