TECHNICAL DESIGN EXTRACTION NOTE

PEP II RF System - 2MW Klystron Power Supply SCR Driver Board

Drawing No.: SD-730-793-03-C4  |  SLAC / Stanford University

# 1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-730-793-03-C4 |
| Title | SCR Driver Board |
| System | PEP II RF System, 2MW Klystron Power Supply |
| Organization | Stanford Linear Accelerator Center (SLAC) |
| Location | Stanford University, Stanford, California |
| Sheet | 1 of 1 |
| CAD File | 73079303.DDB |
| Engineer | M. Nguyen / R. Cassel |
| Drafter | M. Nguyen |
| Checker | J. Olszewski |
| Revision | C4 |

## 1.1 Revision History

| Rev | Description | By | Date |
| --- | --- | --- | --- |
| C3 | Components on the board | WSG/JO/MN | 09/15/03 |
| C4 | Components on the board (updated) | WSG/JO/MN | 03/13/03 |

# 2. System Overview

The SCR Driver Board generates precision-timed gate drive pulses for the thyristor (SCR) power devices in the 2MW klystron power supply. It accepts a 60Hz trigger input and generates properly timed, isolated gate pulses to fire the SCRs at the correct phase angle for voltage regulation.

# 3. Complete Bill of Materials

## 3.1 Integrated Circuits

| Ref Des | Part Number | Description |
| --- | --- | --- |
| U1A/U1B | CD4047B | CMOS monostable/astable multivibrator |
| U2A/U2B | CD4528 | CMOS dual monostable multivibrator |
| U3A-U3F | CD4049 | CMOS hex inverting buffer (multiple sections) |
| U5 | MIC-4451 | High-speed MOSFET gate driver |
| U6A/U6B | CD4528 | CMOS dual monostable multivibrator (second) |

## 3.2 Power MOSFETs

| Ref Des | Part Number | Description |
| --- | --- | --- |
| Q1 | IXFH12N90 | N-channel power MOSFET (900V, 12A) |
| Q2 | IXFH12N90 | N-channel power MOSFET (900V, 12A) |

## 3.3 Diodes

| Ref Des | Part Number | Description |
| --- | --- | --- |
| D1/D2 | Standard | Signal diodes (flyback protection) |
| D3/D4 | 51 1/2W | Zener diodes (gate clamping) |
| D5 | MR856 | Fast-recovery rectifier |
| D7/D8 | 1N4744 | 15V Zener diodes (MOSFET gate protection) |
| DS | MR856 | Fast-recovery rectifier (power stage) |

## 3.4 LEDs

| Ref Des | Part Number | Description |
| --- | --- | --- |
| LED1 | HLMP-1700 | Status LED (trigger active) |
| LED2 | MV 5474C | Status LED (HV power) |
| LED3 | HLMP-1700 | Status LED |
| LED4 | HLMP-1700 | Status LED |

## 3.5 Resistors

| Ref Des | Value | Notes |
| --- | --- | --- |
| R1-R3 | Various | Timing resistors (astable) |
| R5/R6 | Various | Gate drive series resistors |
| R7/R8 | Various | Gate drive series resistors |
| R9 | 1K | LED current limiting |
| R10 | 2K 1W | Power indicator resistor |
| R11 | 1K | Bias resistor |
| R14 | 5.1K | Gate pull-down |
| R17 | 100K | Input resistor |
| R20 | 22K | Timing resistor |
| R26 | 1K | Bias resistor |
| R37 | Various | Timing network |

## 3.6 Capacitors

| Ref Des | Value | Notes |
| --- | --- | --- |
| C5 | 0.001uF | High-frequency bypass |
| C6/C7 | 4.7nF | Snubber capacitors |
| C10 | 10uF | Power supply filter |
| C13 | 0.01uF | Timing capacitor |
| C15 | 0.001uF | Bypass |
| C17/C19 | 10uF | Power supply filter |
| C18 | Various | Timing capacitor |
| Various | 200pF | Timing capacitors for CD4528 monostables |

## 3.7 Connectors

| Ref Des | Description | Notes |
| --- | --- | --- |
| P1 | HV Power Input | ADB-P107-06 connector |
| P2 | Trigger Input/Output | IDC10 ribbon connector |
| F2 | Fuse | Power protection |

# 4. Connector Pinouts

## 4.1 P1 - HV Power Connector (ADB-P107-06)

| Pin | Signal | Notes |
| --- | --- | --- |
| 1 | +240V | High-voltage DC input |
| 2 | +20V | Control supply |
| 3 | GND | Ground |
| 4 | +5V | Logic supply |
| 5 | AC OUT | AC output |
| 6 | AC OUT | AC output (second) |

## 4.2 P2 - Trigger I/O (IDC10)

| Pin | Signal | Notes |
| --- | --- | --- |
| 1-5 | Trigger inputs | From timing controller |
| 6-10 | Gate outputs | To SCR gate transformers |

# 5. Functional Circuit Descriptions

## 5.1 Trigger Input Stage

The trigger input from the test box or controller enters through R17 (100K) and feeds the CD4047B (U1) astable/monostable multivibrator. The timing is set for 60Hz trigger operation. LED indicators show trigger activity.

## 5.2 Timing Generation

The CD4047B (U1) generates the base timing signal. This feeds two CD4528 dual monostable multivibrators (U2A/U2B and U6A/U6B) that generate precise pulse widths. The timing diagram shows: 50 uS +/- 5% main pulse width, 16 uS +/- 1 uS adjustable sub-pulse (adjusted by R2), 18 uS +/- 5% and 48 uS timing intervals. CD4049 (U3) hex inverters provide signal conditioning and buffering between stages.

## 5.3 MOSFET Gate Drive

The MIC-4451 (U5) high-speed MOSFET driver provides the gate drive for the IXFH12N90 power MOSFETs (Q1, Q2). These are 900V 12A N-channel devices that switch the high-voltage to generate the SCR gate pulses. Zener diodes D7/D8 (1N4744, 15V) protect the MOSFET gates. Snubber capacitors C6/C7 (4.7nF) and series resistors provide dV/dt protection.

## 5.4 Output Stage

The power MOSFETs Q1 and Q2 drive the SCR gate pulse transformers through MR856 fast-recovery diodes (D5, DS). The output provides isolated gate pulses for the thyristor (SCR) devices. Test points TP1-TP5 provide access to key timing and output signals.

## 5.5 Power Supply Section

+12V supply is derived from the +240V input. MR856 diodes provide rectification. L1 inductor and filter capacitors (C17, C19, 10uF each) provide filtering. LED2 (MV 5474C) indicates HV power presence. R10 (2K 1W) provides current limiting.

# 6. Timing Diagram (60Hz Trigger Input)

SOT/GOT/GOT trigger input sequence:

- Main pulse: 50 uS +/- 5%

- Sub-pulse: 16 uS +/- 1 uS (adjustable by R2)

- Interval: 18 uS +/- 5%

- Period: 48 uS total cycle

- Test points: TP4-TP5 for output monitoring

# 7. Test Points

| Test Point | Signal | Notes |
| --- | --- | --- |
| TP1 | Trigger input | Input timing verification |
| TP2 | Timing chain output | Monostable output verification |
| TP3 | Intermediate timing | Timing chain midpoint |
| TP4 | MOSFET gate drive Q1 | Gate pulse verification |
| TP5 | MOSFET gate drive Q2 | Gate pulse verification |

# 8. AI Replication Checklist

| # | Item | Key Parameters |
| --- | --- | --- |
| 1 | CD4047B astable/monostable | U1, 60Hz trigger timing |
| 2 | CD4528 dual monostables | U2A/B, U6A/B, 200pF timing caps |
| 3 | CD4049 hex inverters | U3A-F, signal conditioning |
| 4 | MIC-4451 MOSFET driver | U5, high-speed gate drive |
| 5 | IXFH12N90 power MOSFETs | Q1/Q2, 900V 12A |
| 6 | 1N4744 gate protection | D7/D8, 15V zener |
| 7 | MR856 fast diodes | D5/DS, output rectification |
| 8 | Timing: 50uS/16uS/18uS | Main pulse parameters |
| 9 | Power input | ADB-P107-06, +240V |
| 10 | Status LEDs | HLMP-1700, MV 5474C |

END OF TECHNICAL DESIGN EXTRACTION NOTE - SD-730-793-03-C4