TECHNICAL DESIGN EXTRACTION NOTE

PEP II RF System - SCR Control Driver / Left Side Trigger Interconnect Board

Drawing No.: SD-730-793-08-C1  |  SLAC / Stanford University

# 1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-730-793-08-C1 |
| Title | SCR Control Driver - Left Side Trigger Interconnect Board |
| System | PEP II RF System, 2MW Klystron Power Supply |
| Organization | Stanford Linear Accelerator Center (SLAC) |
| Location | Stanford University, Stanford, California |
| Sheet | 1 of 1 |
| CAD File | 73079308.DDB |
| Engineer | M. Nguyen |
| Drafter | M. Nguyen (Nov-14-02) |
| Checker | J. Olszewski |
| Revision | C1 |

# 2. System Overview

The Left Side Trigger Interconnect Board is the companion to the Right Side board (SD-730-793-07). It provides SCR trigger generation, monitoring, fault/enable logic, klystron arc detection, crowbar control, and fiber-optic status communication for the left-side SCR thyristor circuits. It connects to the right-side board via the 24V FAULT/ENABLE bus.

# 3. Complete Bill of Materials

## 3.1 Integrated Circuits

| Ref Des | Part Number | Description |
| --- | --- | --- |
| U1A/U1B | CD4538 | Dual monostable (crowbar timing) |
| U2A/U2B/U3A | CD4538 | Dual monostable (SCR enable, HV status) |
| U4A/U4B | CD4075 | Triple 3-input OR gate |
| U5A-U5C | MIC-4427 | Dual MOSFET driver (SCR triggers, 3 instances) |
| U6A/U6B | CD4001 | Quad NOR gate (logic) |
| U7-U13 | CD4538 | Monostable multivibrators (monitoring, multiple) |
| HCPL-4503 | HCPL-4503 | High-speed optocoupler |
| HFBR-2412 | HFBR-2412 | Fiber-optic receiver (crowbar status) |
| HFBR-1412 | HFBR-1412 | Fiber-optic transmitter (status output) |

## 3.2 Discrete Components

| Ref Des | Part Number / Value | Description |
| --- | --- | --- |
| D3/D10/D11 | 1N4734 | 5.6V Zener diodes |
| D40 | 1N3064 | Signal diode |
| P3KE7 | P3KE7 | TVS diode (klystron arc protection) |
| R7-R38 | Various | Bias, timing, current limiting resistors |
| R12 | 108K | 1-second timing |
| R15/R30 | 100K | Timing resistors |
| R18/R23 | 100/1K | Bias resistors |
| R20 | 100K | Power-on timing |
| R29 | 237K | Crowbar ready timing |
| RP1-RP6 | Resistor packs | Monitoring networks |
| C2/C31 | 10uF | Filter capacitors |
| C9/C10 | 0.001uF | Bypass/timing |
| C16-C34 | Various | Monitoring timing/decoupling |

## 3.3 Connectors

| Ref Des | Description | Notes |
| --- | --- | --- |
| P1-P12 | MIC-4427 output headers | SCR trigger outputs |
| P3-P8 | MIC-4427 outputs | Negative phase triggers |
| BNC | BNC connector | Klystron arc trigger |
| JP1 | JMP-3P | SCR monitor bypass (normal/bypassed) |
| IDC14 | IDC14 ribbon | External interface |
| IDC20 | IDC20 ribbon | Commands interface |

# 4. Functional Circuit Descriptions

## 4.1 SCR Trigger Generation

Three MIC-4427 (U5A-C) dual MOSFET drivers generate SCR trigger pulses for all six phases. The architecture mirrors the right-side board. Output connectors P1-P12 feed the individual SCR driver boards.

## 4.2 Phase Monitoring

Six monitoring channels track SCR firing: +A MON through -C MON. CD4538 monostable multivibrators (U7-U12) with 20mS timing monitor each phase. An 8-input NAND gate combines monitoring outputs for system health. JP1 selects between NORMAL and BYPASSED monitoring.

## 4.3 Fault/Enable Logic

24V FAULT/ENABLE bus connects to the right-side trigger interconnect board. CD4001 (U6A/B) NOR gates and CD4075 (U4A/B) OR gates combine fault conditions. The combined fault output controls SCR enable and crowbar trigger. Commands connector (IDC20) provides external control interface.

## 4.4 Klystron Arc Detection

BNC connector accepts klystron arc trigger signal. P3KE7 TVS diode provides protection. R26 (33K) and associated conditioning circuitry feed the fault logic. Arc detection triggers crowbar protection through the fault/enable system.

## 4.5 Crowbar Control

CD4538 (U1A/B) provides crowbar timing with R29 (237K) setting the 1-second CBREADY timing. HFBR-2412 fiber-optic receiver monitors crowbar status. HLMP-1700 LED indicates crowbar ready state. Output drives to right-side board for coordinated crowbar firing.

## 4.6 Fiber-Optic Status I/O

HCPL-4503 provides high-speed isolated status. HFBR-1412 transmitter sends SCR ENABLE and F.O. STATUS information. HFBR-2412 receiver monitors F.O. KLYSTRON CROWBAR status. 1N4734 (D3, 5.6V) provides voltage clamping on fiber-optic interface.

## 4.7 Power-On and HV Status

R20 (100K) provides power-on delay timing. HLMP-1790 LED indicates power-on status. CD4538 (U3A) monitors HV BON (HV Bus ON) status with associated timing network.

# 5. Signal Net List

| Net Name | Type | Description |
| --- | --- | --- |
| +12V | Power | Primary logic supply |
| COM/GND | Ground | System ground reference |
| +A MON through -C MON | Monitor | 6 phase monitoring channels |
| 24V FAULT/ENABLE | Control | Bus to right-side trigger interconnect board |
| KLYSTRON ARC TRIGGER | Fault | Arc detection input (BNC) |
| CBREADY | Status | Crowbar ready signal |
| SCR ENABLE MON | Monitor | SCR enable monitoring |
| SCR TRIGGERS | Output | Gate drive outputs |
| HV BON | Status | High-voltage bus on indicator |
| F.O. STATUS | Fiber-optic | HFBR-1412 output |
| F.O. KLYSTRON CROWBAR | Fiber-optic | HFBR-2412 input |
| COMMANDS | Digital | External commands via IDC20 |
| POWER ON | Status | HLMP-1790 LED |

# 6. Design Notes

- Mirror architecture to right-side board (SD-730-793-07-C2)

- Connected to right-side board via 24V FAULT/ENABLE bus

- Commands interface via IDC20 connector

- Same IC complement as right-side board for standardization

- Test points TP1-TP11 available for diagnostics

# 7. AI Replication Checklist

| # | Item | Key Parameters |
| --- | --- | --- |
| 1 | MIC-4427 SCR triggers | 3x, 6-phase output |
| 2 | CD4538 monitoring | 6 channels, 20mS timing |
| 3 | CD4001/CD4075 fault logic | Fault combination, enable control |
| 4 | Crowbar control | CD4538, R29(237K) 1sec |
| 5 | Fiber-optic I/O | HCPL-4503, HFBR-1412/2412 |
| 6 | Arc detection | BNC, P3KE7 TVS, R26(33K) |
| 7 | Phase monitoring | 8-input NAND, JP1 bypass |
| 8 | Commands interface | IDC20 connector |
| 9 | Power supply | +12V distribution |
| 10 | 24V fault/enable bus | To right-side board |

END OF TECHNICAL DESIGN EXTRACTION NOTE - SD-730-793-08-C1