TECHNICAL DESIGN EXTRACTION NOTE

PEP II RF System - 2MW Klystron Power Supply Monitor Board

Drawing No.: SD-730-793-12-C3  |  SLAC / Stanford University

# 1. Document Identification

| Field | Value |
| --- | --- |
| Drawing Number | SD-730-793-12-C3 |
| Title | 2MW Klystron PS Monitor Board |
| System | PEP II RF System, 2MW Klystron Power Supply |
| Organization | Stanford Linear Accelerator Center (SLAC) |
| Location | Stanford University, Stanford, California |
| Sheet | 1 of 1 |
| CAD File | SD730793120301.DWG |
| Engineer | M. Nguyen |
| Drafter | M. Nguyen |
| Revision | C3 |

# 2. System Overview

The Monitor Board provides precision voltage and current monitoring for the 2MW klystron power supply. It features isolated DC-DC converters, precision buffer amplifiers, reference voltage generation, and multiple BNC output connectors for instrument-grade monitoring of power supply voltage (10 kV/V scale) and current (5A/V scale). The board uses dual isolated power domains (GND1/GND2) for noise rejection and measurement accuracy.

# 3. Complete Bill of Materials

## 3.1 Integrated Circuits

| Ref Des | Part Number | Description |
| --- | --- | --- |
| U2 | BUF634 | High-speed unity-gain buffer (crowbar monitor) |
| U7 | BUF634 | High-speed unity-gain buffer (channel 1) |
| U8 | BUF634 | High-speed unity-gain buffer (channel 2) |
| U9 | BUF634 | High-speed unity-gain buffer (channel 3) |
| U10 | BUF634 | High-speed unity-gain buffer (PS voltage monitor #2) |

## 3.2 DC-DC Converters

| Ref Des | Part Number | Description |
| --- | --- | --- |
| DC1 | NMH2415S | Isolated DC-DC converter (+15V to +/-15V1, GND1 domain) |
| DC2 | NMH2415S | Isolated DC-DC converter (+15V to +/-15V2, GND2 domain) |

## 3.3 Diodes

| Ref Des | Part Number | Description |
| --- | --- | --- |
| D1 | 1N4728 | 3.3V Zener (DC1 protection) |
| D2 | 1N4728 | 3.3V Zener (DC2 protection) |

## 3.4 Resistors

| Ref Des | Value | Notes |
| --- | --- | --- |
| R1 | N/U | Not used |
| R2 | 1K | PS Current Monitor input |
| R3 | N/U | Not used |
| R5/R6 | N/U | Not used |
| R7/R8 | 1K | BNC output current limiting |
| R10/R11 | 43 ohm | BUF634 output isolation (repeated 5x) |
| R12 | 43 ohm | Buffer output isolation |
| R13 | 100 1/2W | Power resistor |
| R14 | 100 1/2W | Power resistor |
| R15 | 100 1/2W | Power resistor |
| R16-R21 | 43 ohm | BUF634 output isolation (6 instances for channels) |
| R22/R23 | 43 ohm | PS Voltage Monitor #2 output |
| R25 | 2K 12T | Precision trimmer (ref voltage calibration) |
| R26 | 9.09K | Precision divider |
| R27 | Precision | Divider network |
| R28/R29/R30 | 1K | Feedback resistors |
| R31 | 10K | Crowbar input |
| R32 | 10K | PS Current Monitor input |
| R33 | 511 1% | Precision (PS Voltage monitor) |
| R34 | 10K | Feedback |
| R35 | 1K | Current limiting |
| R36 | 536 1% | Precision (PS Voltage monitor) |

## 3.5 Capacitors

| Ref Des | Value | Notes |
| --- | --- | --- |
| C1 | 0.47uF | Input filter |
| C2-C21 | 0.47uF | Decoupling (20 capacitors, 0.47uF each) |

Note: 20 decoupling capacitors (C2-C21) of 0.47uF each are distributed across both ground domains (GND1 and GND2) for comprehensive noise suppression.

## 3.6 Inductors

| Ref Des | Value | Description |
| --- | --- | --- |
| L1 | 470uH | Input filter inductor |

## 3.7 Connectors

| Ref Des | Type | Description |
| --- | --- | --- |
| BNC1 | BNC | + Voltage 10 kV/V output |
| BNC2 | BNC | + Current 5A/V output |
| BNC3 | BNC | Crowbar monitor output |
| J2A-J2H | Multi-section | Board-level connectors |
| P1A-P1H | Multi-section | Power/signal input connectors |
| TB1 | TBLOCK-4 | 4-terminal block |
| TB2 | TBLOCK-2 | 2-terminal block |

# 4. Power Supply Architecture

The monitor board uses a dual isolated power domain architecture for noise rejection:

| Domain | Rails | Source | Purpose |
| --- | --- | --- | --- |
| Domain 1 | +15V1, -15V1, GND1 | DC1 (NMH2415S) | Channels 1-3, crowbar monitor |
| Domain 2 | +15V2, -15V2, GND2 | DC2 (NMH2415S) | Voltage/current BNC outputs |

Input: +15V from P1A through L1 (470uH) filter inductor and D1/D2 (1N4728) protection diodes. NMH2415S converters provide isolated +/-15V supplies with separate ground references.

# 5. Functional Circuit Descriptions

## 5.1 Reference Voltage Generator

TB2 terminal block provides the reference voltage input. R25 (2K 12-turn trimmer) allows precision calibration. R26 (9.09K) and R27 form the voltage divider for reference setting. The reference feeds the BUF634 buffer amplifiers for precision monitoring outputs.

## 5.2 PS Voltage Monitor (BNC1)

R33 (511 ohm 1%) and R36 (536 ohm 1%) precision resistors set the 10 kV/V scale factor. BUF634 (U10) buffer provides low-impedance output at BNC1. Domain 2 (GND2) isolation provides clean measurement.

## 5.3 PS Current Monitor (BNC2)

R2 (1K) and R32 (10K) set the 5A/V scale factor. BUF634 buffer provides output at BNC2. R8 (1K) provides output current limiting. Operates in Domain 2 (GND2) for isolation.

## 5.4 Crowbar Monitor (BNC3)

R31 (10K) and R34 (10K) set the crowbar monitoring gain. BUF634 (U2) in Domain 1 (GND1) buffers the crowbar status signal to BNC3.

## 5.5 Buffer Amplifier Channels

Three BUF634 buffer amplifiers (U7, U8, U9) in Domain 1 provide buffered monitoring outputs for additional channels. Each uses R16-R21 (43 ohm) output isolation resistors and R28-R30 (1K) feedback. +/-15V1 supply with GND1 reference.

# 6. Connector Pinout - P1 (Main Power/Signal)

| Section | Pin | Signal |
| --- | --- | --- |
| P1A | 1 | -10 kV/V (Voltage input) |
| P1E | - | N/U (Not used) |
| P1F | 6 | NMH2415S input |
| P1G | 7 | +15V RTN |
| P1H | - | +15V1 input |

# 7. Design Notes

- Dual isolated ground domains (GND1/GND2) prevent ground loops in monitoring

- NMH2415S DC-DC converters provide 1500V isolation between domains

- 20 decoupling capacitors (0.47uF each) provide comprehensive noise suppression

- BUF634 unity-gain buffers provide high input impedance, low output impedance

- Precision resistors (511/536 ohm 1%) for accurate voltage scaling

- L1 (470uH) input filter provides additional noise rejection

- 1N4728 (3.3V) zeners protect DC-DC converter inputs

- 12-turn trimmer R25 allows fine calibration of reference voltage

# 8. AI Replication Checklist

| # | Item | Key Parameters |
| --- | --- | --- |
| 1 | DC-DC converters | NMH2415S x2, isolated +/-15V domains |
| 2 | BUF634 buffers | 5 instances (U2/U7/U8/U9/U10) |
| 3 | Voltage monitor | 10 kV/V scale, R33(511)/R36(536) 1% |
| 4 | Current monitor | 5A/V scale, R2(1K)/R32(10K) |
| 5 | Crowbar monitor | R31(10K)/R34(10K), BUF634(U2) |
| 6 | Reference calibration | R25 2K 12-turn trimmer, R26(9.09K) |
| 7 | Decoupling | 20x 0.47uF capacitors |
| 8 | Input filter | L1(470uH), D1/D2(1N4728) |
| 9 | Ground isolation | GND1/GND2 separate domains |
| 10 | Output connectors | BNC1(voltage), BNC2(current), BNC3(crowbar) |

END OF TECHNICAL DESIGN EXTRACTION NOTE - SD-730-793-12-C3