SPEAR3 LLRF Upgrade System

Physical Design Report

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

| Document ID: | SPEAR3-LLRF-PDR-001 |
| --- | --- |
| Revision: | R0 |
| Date: | March 3, 2026 |
| Author: | RF Department, SSRL/Accelerator |
| Classification: | Top-Level System Design Reference |

## Table of Contents

────────────────────────────────────────────────────────────────────────────────

1. Executive Summary

2. System-Level Architecture

3. Physical Layout and Locations

4. RF Plant — Retained Physical Infrastructure

5. Subsystem 1: LLRF Controller

6. Subsystem 2: High Voltage Power Supply (HVPS)

7. Subsystem 3: Klystron Machine Protection System (Kly MPS)

8. Subsystem 4: Interface Chassis

9. Subsystem 5: Personnel Protection System (PPS) Interface

10. Subsystem 6: Tuner Control System

11. Subsystem 7: Waveform Buffer System

12. Subsystem 8: Arc Detection System

13. Subsystem 9: Klystron Cathode Heater

14. Subsystem 10: Control Software

15. Control Loop Architecture

16. Inter-Subsystem Interface Matrix

17. Protection Chain and Interlock Architecture

18. Communication Architecture

19. Implementation Phases and Risk Summary

20. Appendix: Source Document Index

## 1. Executive Summary

────────────────────────────────────────────────────────────────────────────────

The SPEAR3 RF station provides 476 MHz RF power to the SPEAR3 storage ring at the Stanford Synchrotron Radiation Lightsource (SSRL). A single klystron, driven at approximately 1 MW, feeds four single-cell RF cavities through a waveguide distribution network. The combined cavity gap voltage is approximately 3.2 MV.

The LLRF Upgrade Project replaces the entire control electronics chain — not merely the low-level RF controller, but also the klystron machine protection system (Kly MPS), HVPS controller, tuner motor controllers, and supporting infrastructure. The project also introduces several new subsystems that did not exist in the legacy system: an Interface Chassis for centralized hardware interlock coordination, a Waveform Buffer System for extended signal monitoring and klystron collector protection, an optical arc detection system, and a modernized klystron cathode heater controller.

### Key System Parameters

| Parameter | Value |
| --- | --- |
| RF Frequency | ~476.3 MHz |
| Total Gap Voltage | ~3.2 MV (sum of 4 cavities) |
| Cavity Gap Voltage | ~800 kV each |
| Klystron Power | ~1.5 MW rated, ~1 MW typical operating |
| HVPS Voltage | up to ~90 kV (negative polarity) |
| HVPS Operating Voltage | ~74.7 kV at 500 mA beam |
| Drive Power | ~50 W nominal |
| Number of Cavities | 4 (single-cell, individual tuners) |
| LLRF9 Units | 2 active, 2 spare (4 purchased) |

│ Source: Gap voltage and beam current data from LLRF9 commissioning measurements (llrf/tests/llrf9Tests.tex, J. Sebek, 2021). Klystron rated power from HVPS hazard documentation (hvps/documentation/procedures/spear3HvpsHazards.tex).

### Upgrade Drivers

1. Hardware obsolescence — VXI, CAMAC, PLC-5, SLC-500, and analog modules are end-of-life

2. PPS compliance — Legacy design routes PPS wiring through HVPS controller and has PLC in the safety chain

3. Performance improvement — Digital FPGA-based feedback (270 ns loop delay) replaces analog processing

4. Diagnostics — 16k-sample waveform capture, circular buffers, first-fault detection, and collector protection

5. Maintainability — Modern hardware, comprehensive documentation, Python/EPICS software

## 2. System-Level Architecture

────────────────────────────────────────────────────────────────────────────────

### 2.1 Legacy System Architecture

The legacy SPEAR3 LLRF system was originally designed for the PEP-II B-Factory (circa 1997) and later adapted for SPEAR3. It consists of:

• LLRF Controller: Custom PEP-II analog RF Processor (RFP) module in a VXI chassis, with associated analog signal processing modules:

• CFM (Comb Filter Module) — comb filter for power-line harmonic rejection (two units: CFM1, CFM2)

• GVF (Gap Voltage Feedback) — gap voltage feedback loop, monitored via CAMAC TAXI interface

• GFF (Gap Feed-Forward) — gap feed-forward reference path providing drive power and gap voltage setpoints through dedicated octal DACs

• See: llrf/legacyLLRF/rf_dac_loop.st (RFP/GFF DAC loop), llrf/legacyLLRF/rf_calib.st (CFM calibration)

• Control Software: Six State Notation Language (SNL) programs on VxWorks RTOS, running on the VXI crate processor, compiled into a single rfSeq IOC library:

• rf_tuner_loop.st — Cavity tuner stepper motor control

• rf_hvps_loop.st — HVPS supervisory control and regulation

• rf_states.st — RF station state machine control (PEP-II heritage, R. Sass, 1997)

• rf_dac_loop.st — Drive Power and Gap Voltage RFP/GFF DAC loop (S. Allison, 1997)

• rf_calib.st — Calibration sequences (R. Claus, SLAC/PEP-II LLRF Group)

• rf_msgs.st — Message logging and CAMAC TAXI error monitoring

• See: llrf/legacyLLRF/Makefile, llrf/legacyLLRF/rf_dac_loop_pvs.h

• HVPS Controller: Allen-Bradley SLC-500 PLC (processor: AB-1747-L532, scanner: AB-1747-DCM) with Enerpro SCR gate driver boards, housed in a Hoffman NEMA enclosure in Building B118

• See: pps/HoffmanBoxPPSWiring.docx

• Klystron Machine Protection System (Kly MPS): Allen-Bradley PLC-5 (1771 series), since converted to ControlLogix 1756 (hardware assembled, software written, tested without RF power). Scope: klystron/RF station equipment protection only — distinct from the facility-wide SPEAR MPS which provides an external permit input.

• See: llrf/documentation/mpsWiringDiagrams/, hvps/architecture/designNotes/RFSystemMPSRequirements.docx

• Tuner Motor Controllers: Allen-Bradley 1746-HSTP1 stepper modules with Superior Electric SS2000MD4-M Slo-Syn PWM drivers (obsolete); stepper motors are Superior Electric Slo-Syn M093-FC11 (NEMA 34D)

• See: llrf/tuners/SLO-SYN_SS2000MD4M_Step_Drive_Translator_Manual.pdf, llrf/tuners/SLO-SYN.pdf

• Interlock System: Distributed across analog modules, PLC I/O, and direct wiring with no central coordination point

• Communication: VXI backplane, CAMAC bus with TAXI serial link, field bus, limited EPICS Channel Access. The GVF module is monitored via CAMAC TAXI; a dedicated SNL sequence (rf_msgs.st) detects TAXI errors and triggers automatic Low Frequency Bypass (LFB) resynchronization with randomized delay to prevent IOC collision.

• See: llrf/legacyLLRF/rf_msgs.st

### 2.2 Upgraded System Architecture

The upgraded system replaces all control electronics while retaining the RF plant physical infrastructure (klystron, cavities, waveguide, HVPS power section, tuner mechanical assemblies). The upgraded architecture is:

Figure 1: The upgraded system replaces all control electronics while retaining the RF plant physical infrastructure (klystron, cavities, waveguide, HVPS power section, tuner mechanical assemblies). The upgraded architecture is

Key Architectural Principle: The Interface Chassis implements machine/equipment protection and operational interlocks (LLRF/HVPS/Kly MPS coordination), while personnel safety (PPS) functions are implemented exclusively in a completely separate, dedicated PPS Interface Box. These two safety-related subsystems are architecturally independent:

• Interface Chassis (Machine/Equipment Protection): First-fault detection, optocoupler isolation, fiber I/O for LLRF9/HVPS/Kly MPS/orbit coordination and other machine-protection interlocks

• PPS Interface Box (Personnel Safety / PPS): Separate Bud enclosure with 4 relays, status LEDs, PPS-lockable connector for K4 relay and Ross switch control

### 2.3 What Stays, What Changes, What Is New

Retained Physical Hardware (same equipment):

• Klystron and its RF output

• 4 RF cavities and waveguide distribution (circulator, magic-tees, waveguide loads)

• Stepper motors (M093-FC11 or equivalent) and mechanical tuner assemblies

• Linear potentiometers on tuners (position indication)

• HVPS power electronics (transformer, rectifier, oil system, crowbar)

• Vacuum contactor (Ross HQ3) and contactor controller (Ross HCA-1-A)

• Ross grounding switch, Danfysik DC-CT, Pearson CT-110

• Field cabling (Belden 83715 15C #16 Teflon, Belden 83709 9C #16 Teflon)

Replaced / Upgraded:

• LLRF Controller: Analog RFP → Dimtel LLRF9/476 (×2 units)

• Kly MPS: PLC-5 1771 → ControlLogix 1756

• HVPS Controller: SLC-500 → CompactLogix PLC + new Enerpro boards + redesigned analog regulator

• Tuner Motor Controllers: AB 1746-HSTP1 + Slo-Syn → Galil DMC-4143 Rev 1.3h 4-axis motion controller (commissioned August 2025, operational)

• See: llrf/tuners/galil/functioningGalil20250825SwapABToManual.txt, llrf/tuners/galil/firstMotion2024.txt

• Control Software: SNL/VxWorks → Python/PyEPICS + LLRF9 internal EPICS IOC

• Operator Interface: Legacy EDM panels → modernized panels

New Subsystems (did not exist in legacy):

• Interface Chassis — central hardware interlock coordination hub

• Waveform Buffer System — 8 RF + 4 HVPS channel extended monitoring

• Arc Detection — total 10 of Microstep-MIS optical sensors on 4 cavity windows and 1 klystron windown, process chassis

• PPS interface Box

Enhanced Subsystems (upgraded from legacy):

• Klystron Heater Controller — Motor-driven variac → SCR-based with EPICS integration

• Collector Power Protection — Forward power proxy → Direct DC-RF power calculation in Waveform Buffer System

## 3. Physical Layout and Locations

────────────────────────────────────────────────────────────────────────────────

The SPEAR3 RF station spans multiple buildings and locations at SSRL:

| Location | Equipment | Notes |
| --- | --- | --- |
| Building B118 (Power Supply Room) | HVPS Controller (Hoffman Box) | HVPS control location |
| Building B514 (HVPS Vault) | HVPS Main Tank (transformer, rectifier, inductor, filter caps), Phase Tank (12 thyristor stacks), Crowbar Tank (4 thyristor stacks, output V-divider), Grounding Tank (Danfysik DC-CT, Pearson CT-110, Ross switch) | High-voltage power section; FR3 oil-filled, N₂ blanket |
| Contactor Disconnect Panel (Switchgear, adjacent to B514) | Vacuum contactor (Ross HQ3), Contactor controller (Ross HCA-1-A), K4/MX/RR/L1 relays, S5 auxiliary contact | 12.47 kV AC switchgear |
| Termination Tank (near klystron) | HV cable termination, Ross Engineering HV relay | Mineral oil filled |
| Switch-over Tank (adjacent B514) | HV cable connections between HVPS1/HVPS2 and klystron | FR3 oil filled |
| Building B132 (Klystron ) | Klystron, drive amplifier, LLRF9 units, Kly MPS PLC, Interface Chassis, Waveform Buffer, Motion Controller, Arc Detector Chassis, Python coordinator server | Main control electronics location |
| SPEAR3 Storage Ring Tunnel | 4 RF cavities, waveguide distribution, tuner assemblies, arc detection sensors | Radiation area |

### Cabling Between Locations

| Cable Run | Cable Type | Conductors | Route |
| --- | --- | --- | --- |
| B118 → Switchgear (Contactor) | Belden 83715 | 15C #16 Teflon | TS-5 to contactor controller |
| B118 → Termination Tank (Grounding) | Belding 83709 + Belden 83715 | 9C + 15C #16 Teflon | TS-6 to grounding tank |
| B118 → B514 (HVPS) | Electrical cable pairs | SCR trigger cables (12 pairs) | Controller to Phase Tank thyristor stacks |
| B118 → B514 (HVPS) | Fiber optic | SCR ENABLE, CROWBAR, STATUS | Controller to HVPS (upgrade: via Interface Chassis) |
| B132 → Tunnel | Coax cables | RF signals (forward, reflected, probe) | LLRF inputs from cavities |
| B132 → Tunnel | Multi-conductor | Motor power + encoder signals | To tuner assemblies |

## 4. RF Plant — Retained Physical Infrastructure

────────────────────────────────────────────────────────────────────────────────

The RF plant is the physical chain that delivers 476 MHz RF power from the klystron to the storage ring beam. All of this hardware is retained as-is during the upgrade.

### 4.1 Klystron

The single klystron is located in Building B132 and operates at approximately 1 MW output power, driven at ~50 W input from the drive amplifier. The klystron cathode is powered by the HVPS at up to ~90 kV (nominal ~74.7 kV at 500 mA beam current). It has a non-full-power collector requiring dedicated protection (see Section 11.3 for upgrade and current implementation in Section 4.5).

### 4.2 Waveguide Distribution

The klystron output feeds a waveguide network consisting of:

• A circulator (protects klystron from reflected power)

• Magic-tee power splitters (two stage splitter that distributes power to 4 cavities)

• Waveguide loads (absorb rejected power)

### 4.3 RF Cavities

Four single-cell cavities at 476 MHz, each contributing ~800 kV gap voltage for a total of ~3.2 MV. Each cavity has:

• A cavity probe (monitors internal field amplitude and phase)

• A forward power coupler

• A reflected power coupler

• An individual stepper motor tuner (adjusts resonant frequency via mechanical plunger)

• Cavity window viewports (for arc detection sensor mounting)

### 4.4 Drive Amplifier

The drive amplifier is located in Building B132 near the klystron and boosts the LLRF9 output signal (~0 dBm) to ~50 W to drive the klystron input. In the upgrade, the LLRF9 Unit 1 output (from the thermally stabilized output chain on Board 1 or Board 2) connects to the drive amplifier input via coax cable. A KAW2051M12 amplifier datasheet is available in llrf/driveAmp/.

### 4.5 Legacy Collector Power Protection

The current SPEAR3 system includes collector power protection implemented through the HVPS control loop. The legacy system monitors klystron forward power and compares it against configurable limits:

Current Implementation:

• Monitoring: LLRF:HVPS:PCOLL_MAX PV sets maximum collector power limit

• Protection Logic: HVPS loop (rf_hvps_loop.st) monitors klystron_forward_power vs. max_klystron_forward_power

• Action: When klystron forward power exceeds limits, HVPS voltage is reduced (delta_proc_voltage_down)

• Integration: Collector limit displayed in HVPS EDL panel ("COLLECTOR LIMIT")

Legacy Limitations:

• Indirect protection through HVPS voltage adjustment (slow response)

• No direct DC power measurement or calculation

• Limited diagnostic capability for collector power trends

• Protection relies on forward power proxy rather than actual collector power calculation

### 4.6 RF Signal Monitoring

The amplitudes of the RF data need to be acquired. The RF signals that are currently monitored are listed in the table below, along with their location in the RF plant signal flow.

#### RF Plant Signal Flow

Figure 2: RF Plant Signal Flow

│ Note on waveguide loads: By the magic-tee symmetry, equal reflected power from the two cavities feeding a magic tee exits port 4 into the attached waveguide load, not back toward the klystron. The circulator load absorbs power that the circulator does not properly direct to the cavities along with any net reflection returning from the first magic tee.

#### Monitored RF Signals [TBD]

| # | Signal | Monitored By | Notes |
| --- | --- | --- | --- |
| 1 | Klystron Output Forward Power | LLRF9 Unit 2 BRD2 | RF power at the klystron output traveling toward the circulator; primary measure of station RF output level. |
| 2 | Klystron Output Reflected Power | LLRF9 Unit 2 BRD2 | RF power reflected back into the klystron output port from the load chain; should be near zero; used for klystron protection. |
| 3 | Circulator Load Forward Power | Waveform Buffer Ch1 | Power delivered to the circulator load (circulator port 3); absorbs power the circulator does not properly direct to the cavities plus any net reflection returning from the waveguide network. |
| 4 | Circulator Load Reflected Power | Waveform Buffer Ch2 | Power reflected from the circulator load back into the circulator; indicates load match quality. |
| 5 | Klystron Drive Power | LLRF9 Unit 2 BRD1 | Input drive signal level to the klystron from the drive amplifier; sets the klystron gain and operating point. |
| 6 | Station Reference Power | Waveform Buffer Ch3 | Monitor of the 476 MHz reference signal level distributed to the LLRF system; loss of this signal indicates a reference chain fault. |
| 7 | Waveguide Load 1 Forward Power | LLRF9 Unit 1 BRD3 | Power entering WG Load 1 at port 4 of Magic Tee 1; absorbs the combined out-of-phase reflections from the two second-layer magic tees that make it back to Magic Tee 1. |
| 8 | Waveguide Load 1 Reflected Power | Waveform Buffer Ch4 | Power reflected from WG Load 1 back into Magic Tee 1; indicates load match quality. |
| 9 | Cavity A Forward Power | LLRF9 Unit 1 BRD1 | RF power traveling into Cavity A from Magic Tee 2; used to monitor power distribution balance across cavities. |
| 10 | Cavity A Reflected Power | LLRF9 Unit 2 BRD3 | Power reflected from Cavity A back into the waveguide; elevated reflection indicates cavity detuning or mismatch. |
| 11 | Cavity B Forward Power | LLRF9 Unit 1 BRD2 | RF power traveling into Cavity B from Magic Tee 2; compared with Cavity A forward to verify magic-tee balance. |
| 12 | Cavity B Reflected Power | LLRF9 Unit 2 BRD3 | Power reflected from Cavity B; monitored for detuning and arc detection. |
| 13 | Cavity A Probe Signal | LLRF9 Unit 1 BRD1 | Electric field sampled by Cavity A's internal pickup probe; direct measure of the accelerating voltage amplitude and phase; primary LLRF fast feedback and tuner control input for Cavity A. |
| 14 | Cavity B Probe Signal | LLRF9 Unit 1 BRD1 | Electric field from Cavity B's internal probe; used in the LLRF vector-sum feedback and Cavity B tuner control. |
| 15 | Waveguide Load 2 Forward Power | LLRF9 Unit 1 BRD3 | Power entering WG Load 2 at port 4 of Magic Tee 2; absorbs the sum of equal reflected power from Cavities A and B. |
| 16 | Waveguide Load 2 Reflected Power | Waveform Buffer Ch5 | Power reflected from WG Load 2; indicates match quality of the load. |
| 17 | Cavity C Forward Power | LLRF9 Unit 1 BRD2 | RF power traveling into Cavity C from Magic Tee 3. |
| 18 | Cavity C Reflected Power | LLRF9 Unit 2 BRD3 | Power reflected from Cavity C; monitored for detuning and arc detection. |
| 19 | Cavity D Forward Power | LLRF9 Unit 2 BRD1 | RF power traveling into Cavity D from Magic Tee 3. |
| 20 | Cavity D Reflected Power | LLRF9 Unit 2 BRD2 | Power reflected from Cavity D. |
| 21 | Cavity C Probe Signal | LLRF9 Unit 1 BRD2 | Electric field from Cavity C's internal probe; used in the LLRF vector-sum feedback and Cavity C tuner control. |
| 22 | Cavity D Probe Signal | LLRF9 Unit 2 BRD1 | Electric field from Cavity D's internal probe; used in the LLRF vector-sum feedback and Cavity D tuner control. |
| 23 | Waveguide Load 3 Forward Power | LLRF9 Unit 1 BRD3 | Power entering WG Load 3 at port 4 of Magic Tee 3; absorbs the sum of equal reflected power from Cavities C and D. |
| 24 | Waveguide Load 3 Reflected Power | Waveform Buffer Ch6 | Power reflected from WG Load 3; indicates match quality of the load. |

## 5. Subsystem 1: LLRF Controller

────────────────────────────────────────────────────────────────────────────────

│ Detailed reference: llrf/llrf9

### 5.1 Legacy System

The legacy LLRF controller is a custom PEP-II analog RF Processor (RFP) module in a VXI chassis. It performs analog I/Q processing at ~kHz bandwidth for the fast RF feedback loop. Associated modules include the CFM (Comb Filter Module) for multi-bunch stabilization and the GVF (Gap Voltage Feedback) for gap voltage regulation. The VXI chassis also hosts a processor running VxWorks RTOS with SNL (State Notation Language) control programs.

The legacy system processes 24 RF channels through the VXI system and uses analog-domain signal processing for feedback, calibration, ripple rejection, and comb filtering.

### 5.2 Upgraded System — Dimtel LLRF9/476

Two Dimtel LLRF9/476 units replace the entire VXI-based LLRF system (four units purchased; two active, two spares).

Hardware per unit:

• 3 × LLRF4.6 boards: each with Xilinx Spartan-6 FPGA, 4 high-speed ADC channels, 2 DAC channels, 3 RF channel.

• LO/Interconnect module: divide-and-mix LO synthesis for low phase noise, RF reference distribution, output amplification/filtering, interlock logic

• Linux SBC (mini-ITX): runs the built-in EPICS IOC (EPICS Base 3.14)

• Thermal stabilization: aluminum cold plate with 3 TEC modules under PID control, only available to board1&2

• Power supply: 90-264 VAC auto-ranging

• 3U 19" rack chassis

LO Frequency Plan (LLRF9/476):

| Signal | Ratio to f_rf | Frequency (MHz) |
| --- | --- | --- |
| Reference (f_rf) | 1 | 476.000 |
| IF | 1/12 | 39.667 |
| Local Oscillator | 11/12 | 436.333 |
| ADC Clock | 11/48 | 109.083 |
| DAC Clock | 11/24 | 218.167 |

Auxiliary I/O per unit:

• 8 slow ADC channels (12-bit, selectable ranges, galvanically isolated)

• 12 slow DAC outputs (AD5644)

• Opto-isolated interlock input (3.3V/5V/24V selectable)

• Interlock output (+4.75V high / <0.1V tripped, 220 Ω)

• Two opto-isolated trigger inputs

### 5.3 Two-Unit Configuration for SPEAR3

Unit 1 — Field Control & Tuner Loops:

| Board | Ch0 (Ref) | Ch1 | Ch2 | Ch3 | Output | Thermal Stabilization |
| --- | --- | --- | --- | --- | --- | --- |
| BRD1 | Station Ref | Cav A Probe | Cav B Probe | Cav A Fwd | Klystron Drive | Y |
| BRD2 | Station Ref | Cav C Probe | Cav C Fwd | Cav B Fwd | (Spare / Monitor) | Y |
| BRD3 | Station Ref | WG Load 1 Fwd | WG Load 2 Fwd | WG Load 3 Fwd | (Not used) | N |

• Primary vector sum: Only Cavities A & B (on BRD1, same board as output) participate in the 270 ns direct feedback loop for klystron drive

• Critical constraint: Cavities C & D are monitored but NOT in the main fast feedback loop

• Tuner phase data: All 4 cavity probe phases available at 10 Hz for tuner control

• BRD3: Monitors the three waveguide load forward power signals (WG Load 1, 2, 3); RF connections on rear panel only; no thermal stabilization

Unit 2 — Monitoring & Interlocks:

| Board | Ch0 (Ref) | Ch1 | Ch2 | Ch3 | Output | Thermal Stabilization |
| --- | --- | --- | --- | --- | --- | --- |
| BRD1 | Station Ref | Cav D Probe | Cav D Fwd | Drv Fwd | (Not used) | Y |
| BRD2 | Station Ref | Cav D Refl | Kly Refl | Kly Fwd | (Not used) | Y |
| BRD3 | Station Ref | Cav A Refl | Cav B Refl | Cav C Refl | (Not used) | N |

• Reflected power monitoring: All 4 cavity reflected + klystron reflected for arc/mismatch detection

• Interlock chain: Reflected power events → Unit 2 interlock output → Interface Chassis → disables Unit 1 drive

• No drive output required from Unit 2

│ Remaining RF signals: The 6 signals not covered by the two LLRF9 units (Circulator Load Fwd/Refl, Station Reference, WG Load 1/2/3 Reflected) are monitored by the Waveform Buffer System — see Section 11.

### 5.4 Key Performance Specifications

| Parameter | Value |
| --- | --- |
| Direct loop delay | 270 ns |
| RF input channels | 9 per unit (18 total) |
| RF input range | +2 dBm full-scale |
| ADC resolution | 12-bit |
| Setpoint profile points | 512 |
| Setpoint step time range | 70 μs to 37 ms per step |
| Waveform samples/channel | 16,384 |
| Scalar readback rate | 10 Hz |
| Phase readback resolution | Sub-degree |
| Interlock timestamp resolution | ±17.4 ns |

### 5.5 Interfaces

• RF inputs: 50 Ω SMA from cavity probes, forward couplers, reflected couplers, station reference

• RF output: SMA to drive amplifier (Unit 1 only, thermally stabilized, BRD1 or BRD2)

• Interlock I/O: LEMO connectors; Unit 1 external interlock input from Interface Chassis, Unit 2 interlock output to Interface Chassis

• Slow ADC: DA-15 connector; HVPS monitoring signals, auxiliary sensors

• Ethernet: Channel Access for EPICS communication (default ports 5064/5065)

• Unit 1 ↔ Unit 2: Interlock daisy-chain (eg: Unit 2 reflected power trip disables Unit 1 drive)

## 6. Subsystem 2: High Voltage Power Supply (HVPS)

────────────────────────────────────────────────────────────────────────────────

│ Primary source references:

│ - Architecture: hvps/architecture/designNotes/interfacesBetweenRFSystemControllers.docx, RFSystemMPSRequirements.docx, EnerproVoltageandCurrentRegulatorBoardNotes.docx

│ - PLC Documentation: hvps/documentation/plc/plcNotesR1.docx, CasselPLCCode.pdf

│ - Legacy System: hvps/architecture/designNotes/rfedmHvpsLabelsPvs.docx, HoffmanBoxPPSWiring.docx

│ - Wiring/Schematics: hvps/documentation/wiringDiagrams/

### 6.1 Power Section (Retained)

The HVPS converts 12.47 kV RMS 3-phase AC to regulated DC high voltage for the klystron cathode.

Power chain:

Figure 3: **Power chain**

| Parameter | Value |
| --- | --- |
| Maximum output voltage | −90 kV DC |
| Maximum output current | 27 A |
| Maximum output power | 2.5 MW |
| Nominal operating voltage | −74.7 kV |
| Nominal operating current | 22.0 A |
| Rectifier topology | 12-pulse, thyristor phase-controlled |
| Number of HVPSs | 2 (SPEAR1 active, SPEAR2 warm spare) |

• Nominal operatiing beam current: 500 mA

The power section, including transformers, thyristor stacks, filter inductors, secondary rectifiers, crowbar, oil system, and all power cabling, is retained unchanged.

### 6.2 Legacy Controller

The legacy controller is housed in a Hoffman NEMA enclosure (34"×42") in Building B118 and contains:

• PLC: Allen-Bradley SLC-500 (1747-L532 CPU) with I/O modules (OBSOLETE):

| Slot | Module | Function |
| --- | --- | --- |
| CPU | AB-1747-L532 | Processor |
| 1 | AB-1747-DCM | Data Communications (Scanner) |
| 2 | AB-1746-IO8 | 8-pt Digital I/O — Ross switch coil (OUT3) |
| 3 | AB-1746-THERMC | Thermocouple inputs (SCR/air/transformer temps) |
| 5 | AB-1746-OX8 | 8-pt Relay Output — Contactor enable K4 (OUT2), Contactor On/Off (OUT1) |
| 6 | AB-1746-IB16 | 16 DC Input — PPS 1 (IN14), PPS 2 (IN15), Oil Level (IN8), Manual GRN SW (IN9) |
| 7 | AB-1746-IV16 | 16 DC Input (various permits) |
| 8 | AB-1746-NIO4V | 4-ch Analog I/O — voltage setpoint output to Enerpro via regulator (N7:10 → OUT0) |
| 9 | AB-1746-NI4 | 4-ch Analog Input — Danfysik HVPS current (IN3) |

│ Source: hvps/documentation/plc/plcNotesR1.docx (N7:10/N7:11 analysis, slot assignments)

• Enerpro Firing Board: FCOG6100 + FCOAUX60 (30° delayed triggering daughter board) — Current SCR gate driver

• Regulator Card (SD-237-230-14-C1): SLAC-designed analog voltage regulation loop with INA117 (difference amp), INA114 (instrumentation amp), OP77 (op-amp, 600 kHz GBW), BUF634 (high-current buffer), 4N32 optocoupler, and VTL5C opto-controlled variable resistor (OBSOLETE component)

• Power Supplies: SOLA ±15V/+5V/24V, Kepko 120V (×2), Kepko 5V/20A, Kepko 240V

• PS Monitor Board (SD-730-793-12): Monitors power supply health

• Terminal Strips: TS-5 (contactor, 15 terminals), TS-6 (grounding tank, 21 terminals), TS-3 (PPS LEDs), TS-7 (power distribution)

• PPS Connector: Originally Burndy circular 8-pin; possibly replaced with Souriau Trim Trio equivalent (GOB12-88PNE) — exact current part number requires field verification

The SLC-500 PLC executes ladder logic for voltage regulation, contactor management, temperature monitoring, and — critically — PPS safety chain control:

• PPS 1: Controls K4 contactor enable path; also has hardware fail-safe wired directly to OX8 relay input (bypasses PLC)

• PPS 2: Controls Ross grounding switch coil via Slot-2 IO8 OUT3 (Rung 0016)

• Hardware fail-safe: K4 relay input side wired directly to PPS 1 signal (24 VDC), preventing PLC failure from keeping K4 energized without PPS enable

│ Sources: hvps/architecture/designNotes/HoffmanBoxPPSWiring.docx, RFSystemMPSRequirements.docx

### 6.3 Upgraded Controller

The HVPS controller upgrade replaces the PLC and SCR gate driver while retaining the power section and Hoffman enclosure.

New hardware:

• CompactLogix PLC: Replaces SLC-500; handles voltage regulation, temperature monitoring, fault management, and EPICS communication

• Enerpro FCOG1200 Board: Upgraded SCR gate driver with modified RN4 resistors for SPEAR monitor winding impedance (2 MΩ)

• Redesigned Analog Regulator: Replaces obsolete regulator card components

• Fiber Optic Interfaces: Bidirectional communication with Interface Chassis:

• STATUS: HVPS controller → HFBR-1412 transmitter → Interface Chassis (indicates control supply presence + no crowbar fired)

• SCR ENABLE: Interface Chassis → HFBR-2412 receiver → HVPS controller (permit signal, active-high for fail-safe)

• CROWBAR: Interface Chassis → HFBR-2412 receiver → HVPS controller (crowbar fire command)

• Normal operation: All signals illuminated/active; signal loss = fault condition

Key change: The CompactLogix PLC is removed from the PPS safety chain. PPS functions (K4 relay, Ross switch control) are handled by a separate dedicated PPS interface box, not the Interface Chassis. The Interface Chassis handles only non-PPS interlocks. The PLC handles only non-safety functions: voltage regulation, temperature monitoring, and EPICS interface.

### 6.4 HVPS Controller Interface

-with EPICS:

| PV Category | Example PV | Update Rate |
| --- | --- | --- |
| Setpoint | SRF1:HVPS:VOLT:CTRL.VAL | ≤1 Hz (from Python) |
| Readback | SRF1:HVPS:VOLT:RBCK | ~1 Hz |
| Status | SRF1:HVPS:STATUS:READY | ~1 Hz |
| Control | SRF1:HVPS:CONTACTOR:CMD | On demand |

Legacy HVPS Status PVs (for reference and troubleshooting):

| Status Parameter | Function | Source |
| --- | --- | --- |
| Contactor Closed | K4 relay status / Aux relay status | Slot-6 inputs |
| Over Voltage | Regulator card latch indication | SD-237-230-14-C1 |
| Klystron Arc | Grounding tank + LHS trigger board detection | BNC-12 input |
| Transformer Arc | HVPS + RHS trigger board detection | BNC-0 input |
| Crowbar Status | Thyristor pulse detection | Enerpro board |
| PPS Chain Status | PPS 1 and PPS 2 availability | Slot-6 IN14/IN15 |
| SCR 1/2 Firing | Left/right side thyristor trigger status | Enerpro monitoring |
| AC Current | Input current measurement | Transformer monitoring |
| Temperature/Oil | SCR/air/transformer temperatures, oil level | Slot-3 thermocouples |
| 12 kV Availability | Supply status/readiness | Switchgear feedback |
| Fast Inhibit/Slow Start | Enerpro protection states | Gate driver board |

│ Source: hvps/architecture/designNotes/rfedmHvpsLabelsPvs.docx (RF expert panel configuration)

-with InterfaceChassis:

• Interface Chassis (fiber optic): SCR ENABLE (in), CROWBAR inhibit (in), STATUS (out)

Crowbar Triggering Sources (five independent sources for defense-in-depth protection):

1. SCR ENABLE (fiber-optic): Interface Chassis → HVPS controller (loss of signal disables SCR triggers)

2. TRANSFORMER ARC TRIGGER (BNC-0): Electronic signal from Hoffman box (AC current measurement or Stangenes transformer arc detection)

3. KLYSTRON CROWBAR (fiber-optic): LLRF/Interface Chassis → HVPS controller (klystron protection command)

4. KLYSTRON ARC TRIGGER (BNC-12): Electronic signal from Hoffman box (termination tank shunt sensing excessive current)

5. PLC FORCE CROWBAR (Slot-5 OUT3): Active-low signal from SLC-500, monitored on BNC connector

│ Protection philosophy: Any single source can disable SCR triggers or fire crowbar; redundancy ensures protection even with single-point failures. Filter inductor stored energy is safely discharged through Enerpro logic and secondary rectifiers.

│

│ Source: hvps/architecture/designNotes/RFSystemMPSRequirements.docx

• Python Coordinator (EPICS/Ethernet): Voltage setpoint, readbacks, fault status

• Waveform Buffer System: HVPS voltage, current, inductor voltages monitored on 4 dedicated channels

• Switchgear: Existing field cables to vacuum contactor controller and grounding tank

## 7. Subsystem 3: Klystron Machine Protection System (Kly MPS)

────────────────────────────────────────────────────────────────────────────────

│ Scope clarification: The Kly MPS is the klystron/RF station equipment protection PLC. It is distinct from the facility-wide SPEAR MPS, which is an external system that provides a permit input signal to the Kly MPS via the Interface Chassis.

### 7.1 Purpose and Protection Philosophy

The Klystron Machine Protection System (Kly MPS) is the subsystem-level protection controller for the SPEAR3 RF station. Its purpose is to aggregate fault status from all RF subsystems and external safety systems (including the SPEAR MPS permit), manage the overall klystron/RF permit, and coordinate fault recovery.

The protection philosophy, as defined in the RF system MPS requirements (hvps/architecture/designNotes/RFSystemMPSRequirements.docx), is:

1. Protect high-power elements (HVPS, klystron, cavities) from damage by eliminating stored energy and disabling upstream power sources when faults occur.

2. Redundant protection — multiple independent mechanisms protect each element (e.g., fiber-optic SCR disable, crowbar firing, contactor opening).

3. Fail-safe design — all permits are active-high so that a broken cable or lost signal removes the permit.

4. Graceful shutdown — when upstream elements trip, downstream elements are notified so they can shut down in an orderly fashion.

The Kly MPS does not directly control the LLRF9 or HVPS hardware. Instead, it provides permit and control signals to the Interface Chassis (Section 8), which performs the fast hardware AND-logic and drives the actual interlock outputs to the LLRF9 and HVPS controller. This separation ensures that hardware interlock response times remain at the microsecond scale (Interface Chassis), while the Kly MPS operates at PLC scan rates (~milliseconds) for fault aggregation, logging, and coordination.

### 7.2 Legacy System

The legacy Kly MPS uses an Allen-Bradley PLC-5 processor with 1771-series I/O modules. It aggregates interlock signals from the RF system (via distributed analog modules and direct wiring), the HVPS, and external sources (SPEAR MPS permit, orbit interlock), and manages the RF permit chain.

In the legacy system, interlock coordination was distributed across analog modules, PLC I/O, and direct point-to-point wiring with no central coordination point. The PLC-5 communicated with the EPICS control system via a 1771-DCM scanner module.

│ Legacy Kly MPS wiring diagrams: 33 sheets in llrf/documentation/mpsWiringDiagrams/ (drawings wd3403300200 through wd3403303400).

### 7.3 Upgraded System — ControlLogix 1756

The Kly MPS PLC is upgraded from PLC-5 to Allen-Bradley ControlLogix 1756 platform using a Rockwell Automation conversion kit. The ControlLogix platform provides modern Ethernet/IP communication, faster scan times, and long-term vendor support.

Status: Hardware assembled, software written, tested on SPEAR3 without RF power. Ready for EPICS IOC development and system integration.

### 7.4 Kly MPS Outputs to Interface Chassis

In the upgraded system, the Kly MPS communicates with the Interface Chassis via digital signals. These are the only direct hardware outputs from the Kly MPS PLC to the interlock system:

| Output Signal | Type | Description |
| --- | --- | --- |
| Kly MPS Summary Permit | Digital | Global RF permit from the Kly MPS. One of several AND-gate inputs in the Interface Chassis. When removed, the Interface Chassis removes LLRF9 Enable and HVPS SCR ENABLE. |
| Kly MPS Heartbeat | Digital | Watchdog signal. If the Interface Chassis stops receiving the heartbeat (indicating Kly MPS PLC failure or communication loss), it removes all output permits. |
| Kly MPS Reset | Digital | Clears all latched faults in the Interface Chassis first-fault register simultaneously. Required to restore system permits after a fault. |

Important: The Kly MPS does not directly drive the LLRF9 Enable, HVPS SCR ENABLE, or HVPS KLYSTRON CROWBAR signals. Those are Interface Chassis outputs, driven by the AND-logic of all input permits (see Section 8). The Kly MPS contributes its Summary Permit as one input to that AND gate.

### 7.5 Kly MPS Inputs from Interface Chassis

The Kly MPS receives comprehensive status from the Interface Chassis via a multi-conductor digital cable:

| Input Signal | Description |
| --- | --- |
| All input permit states | Status of every Interface Chassis input (LLRF9 Status, HVPS STATUS, SPEAR MPS permit, Orbit Interlock, Arc Detection, Waveform Buffer, expansion ports) |
| All output permit states | Status of every Interface Chassis output (LLRF9 Enable, HVPS SCR ENABLE, HVPS CROWBAR) |
| First-fault register | Identifies the initiating fault source when multiple faults cascade. Hardware-latched at the moment of initial fault detection. |

This information allows the Kly MPS to:

• Determine which subsystem caused a trip

• Log complete fault histories with timestamps

• Report detailed status to the Python/EPICS coordinator for operator diagnostics

### 7.6 Kly MPS Role in the Protection Chain

The RF system implements a layered protection architecture. The Kly MPS operates at Layer 3 (PLC scan rate, ~milliseconds):

| Protection Layer | Subsystem | Response Time | Function |
| --- | --- | --- | --- |
| Layer 1 | LLRF9 FPGA | <1 μs | RF overvoltage interlocks, baseband window comparators, DAC zeroing, RF switch |
| Layer 2 | Interface Chassis | <1 μs | Hardware AND-logic, first-fault latching, HVPS fiber-optic control, LLRF9 enable |
| Layer 3 | Kly MPS PLC | ~ms (PLC scan) | Fault aggregation, permit management, reset coordination, event logging |
| Layer 4 | Python coordinator | ~1 Hz | State machine, supervisory control, operator interface, auto-recovery |

The Kly MPS functions include:

• Aggregating fault status from all RF station subsystems (via Interface Chassis digital status lines)

• Receiving the SPEAR MPS permit as one of the AND-gate inputs via the Interface Chassis

• Providing the Kly MPS Summary Permit to the Interface Chassis AND-gate

• Sending the heartbeat watchdog signal to the Interface Chassis

• Issuing the reset signal to the Interface Chassis to clear latched faults after a trip

• Logging fault events with timestamps for post-mortem analysis

• Providing redundant collector power calculation (from Waveform Buffer System data sent to Kly MPS for independent verification)

• Reporting complete system status to the Python/EPICS coordinator

### 7.7 EPICS Interface

The Kly MPS ControlLogix PLC will provide an EPICS interface for operator monitoring and diagnostics. The PV namespace is SRF1:MPS:. Key PVs include:

| PV | Type | Description |
| --- | --- | --- |
| SRF1:MPS:PERMIT | BINARY | Overall MPS permit status |
| SRF1:MPS:FAULTS:ACTIVE | MULTI-BIT | Currently active fault sources |
| SRF1:MPS:FAULTS:FIRST | ENUM | First-fault identification (from Interface Chassis) |
| SRF1:MPS:FAULTS:COUNT | INTEGER | Accumulated fault event count |

The Python coordinator reads these PVs to:

• Determine whether the MPS permit is active before allowing state transitions (e.g., OFF → STANDBY → ON)

• Force a transition to OFF state if MPS permit is lost during operation

• Display fault summaries and first-fault information to operators

### 7.8 Interfaces Summary

| Interface Partner | Signal Direction | Signals | Medium |
| --- | --- | --- | --- |
| Interface Chassis | KLY MPS → IC | Summary Permit, Heartbeat, Reset | Digital (multi-conductor cable) |
| Interface Chassis | IC → MPS | All input/output states, first-fault register | Digital (multi-conductor cable) |
| Python Coordinator | Bidirectional | Permit status, fault history, reset commands | EPICS Channel Access (Ethernet) |

Note: External safety systems (SPEAR MPS permit, orbit interlock) connect to the Interface Chassis directly, not to the MPS PLC. The MPS receives their status indirectly via the Interface Chassis digital status lines. This design ensures that external safety permits participate in the fast hardware AND-logic (<1 μs) within the Interface Chassis, rather than being limited to PLC scan rates.

### 7.9 Source Documents

| File | Description |
| --- | --- |
| hvps/architecture/designNotes/RFSystemMPSRequirements.docx | RF system machine protection requirements and protection philosophy (J. Sebek) |
| hvps/architecture/designNotes/interfacesBetweenRFSystemControllers.docx | Interface Chassis design — defines the MPS-to-Interface Chassis signal interface |
| llrf/documentation/mpsWiringDiagrams/ | 33 legacy MPS wiring diagram sheets (wd3403300200–wd3403303400) |
| pps/diagrams/00_SYSTEM_OVERVIEW.md | System-level architecture showing MPS in the upgraded system context |
| llrf/llrf9/iGp/dl_llrf/interlock_summary.edl | LLRF9 interlock summary display (EDM) — shows LLRF9-side interlock PVs that feed into the protection chain |

## 8. Subsystem 4: Interface Chassis

────────────────────────────────────────────────────────────────────────────────

│ Detailed reference: llrf/architecture/llrfInterfaceChassis.docx

### 8.1 Purpose

The Interface Chassis is a completely new subsystem that serves as the central hub for all hardware interlock coordination. It did not exist in the legacy system, where interlocks were distributed across analog modules, PLC I/O, and direct wiring with no central coordination point.

Key capabilities:

• First-fault detection — hardware latching identifies the initiating fault in cascade scenarios

• Microsecond-scale response — combinational logic with no processor in the critical path

• Electrical isolation — all external signals isolated via optocouplers and fiber-optic transceivers

• Fault latching — all inputs latch when faulted until external MPS reset

• Status reporting — comprehensive digital reporting to MPS PLC

### 8.2 Interface Summary

Primary inputs: LLRF9 status, HVPS fiber-optic status, MPS permits/heartbeat/reset, SPEAR MPS, orbit interlock, arc detection, power monitoring, expansion ports

Primary outputs: LLRF9 enable, HVPS SCR ENABLE (fiber), HVPS CROWBAR (fiber), fault status reporting to MPS

Critical design consideration: The Interface Chassis creates a bidirectional feedback loop between LLRF9 and HVPS that requires careful recovery sequencing logic.

### 8.3 Implementation Status

• Interface specification: In progress (J. Sebek, llrf/architecture/llrfInterfaceChassis.docx)

• Chassis design: Not started

• PCB design: Not started

• Fabrication: Not started

Note: The Interface Chassis is on the critical path for system integration.

## 9. Subsystem 5: Personnel Protection System (PPS) Interface

────────────────────────────────────────────────────────────────────────────────

│ Detailed reference: pps/

### 9.1 PPS Overview

The PPS is the radiation and electrical safety interlock system managed by SLAC's radiation protection group. It controls two points in the HVPS power chain:

• Chain 1: Vacuum contactor (removes 12.47 kV input power to HVPS)

• Chain 2: Ross grounding switch (shorts the HVPS output to ground)

The PPS interface uses a GOB12-88PNE (Burndy/Souriau Trim Trio) 8-pin circular connector mounted in a locked box on the Hoffman enclosure.

### 9.2 Legacy PPS Design (Problems)

In the legacy system, PPS signals are routed through the Hoffman Box:

1. PPS wiring exposure — PPS wires terminate on terminal strips (TS-5, TS-6) inside the HVPS controller, co-located with non-PPS wiring

2. PLC in safety chain — The Ross grounding switch is controlled by the SLC-500 PLC (Slot-2 IO8 OUT3, 120 VAC). If the PLC fails in a stuck-on state, the Ross switch coil stays energized (unsafe)

3. K4 relay via PLC — Although the K4 relay coil uses PPS 1 voltage for its power rail (providing a hardware fail-safe), the enable path runs through PLC OX8 OUT2

### 9.3 Upgraded PPS Design

The upgraded design adopts the standard PPS interface solution already approved and in use by the PPS group for the 6575 modulator and gallery systems. It uses a dedicated PPS interface box.

PPS Interface Box Characteristics (per Ben Morris, March 5, 2026):

• Small Bud enclosure (≈6″ × 5″ × 4″)

• Contains 4 relays, status LEDs (Permit A/B granted & enabled), and inhibit push-buttons

• Single board with one connector that the PPS group can lock with their collar

• Provides two independent permit channels + two readback channels (dry contacts)

• Completely isolated from the HVPS internals

• Labeled "PPS Interface – RSWCF required to open" (standard PPS practice)

PPS Signal Flow:

• Chain 1 (Contactor): PPS Enable 1 → PPS Interface Box → K4 relay direct drive → MX → L1 holding coil. S5 NC auxiliary contact readback via PPS Interface Box to PPS.

• Chain 2 (Ross Switch): PPS Enable 2 → PPS Interface Box → Ross switch direct drive. Ross NC auxiliary contact readback via PPS Interface Box to PPS.

Additional Safety Relays: Because the inhibit and safety-discharge wiring cannot be physically separated inside the existing HVPS and termination tank, a second relay is added in series with the inhibit/safety-discharge lines (one relay per channel). These relays are labeled "PPS Controlled – RSWCF required before work" with PPS-provided cable tags.

Key improvements:

• No PLC dependency for safety functions

• Uses proven PPS-approved design (identical to gallery systems)

• Electrical isolation of all PPS signals

• PPS wiring completely isolated from non-PPS equipment

• Clear visual feedback and independent inhibit capability

• Meets all current PPS requirements (two independent channels, lockable interface, visible verification)

• Eliminates the "unique RF situation" that the PPS group dislikes

│ Source: pps/pps_Ben.md (March 5 2026 meeting), pps/MSG from Jim Sebek to Faya about PPS.md

### 9.4 PPS Regulatory Considerations

Any modification to PPS wiring, control logic, or readback paths requires:

• Review and approval by SLAC AD Safety Division

• Formal change control documentation

• Radiation safety verification testing

• Possibly a radiation safety work control form

This adds significant administrative scope beyond the engineering work and requires early engagement with the PPS/protection group.

## 10. Subsystem 6: Tuner Control System

────────────────────────────────────────────────────────────────────────────────

### 10.1 Physical Mechanism

Each of the 4 RF cavities has a mechanical tuner that adjusts the cavity resonant frequency by moving a plunger into or out of the cavity volume. The tuner is driven by a stepper motor (Superior Electric M093-FC11 or equivalent) through a gear reduction mechanism.

Tuner parameters (from legacy documentation):

• Gear ratio: 80:1 (worm gear)

• Lead screw pitch: 10 turns/inch

• Microstep resolution: 0.002–0.003 mm per microstep (legacy)

• Linear potentiometer: Provides absolute position indication (not used in closed-loop control)

• Travel range: ~12 mm total mechanical range

• ON home positions: ~10.1–10.7 mm (cavity-dependent)

• Park positions: retracted position for safe storage

### 10.2 Legacy Controller

• Motor controller: Allen-Bradley 1746-HSTP1 (high-speed stepper module, OBSOLETE)

• Motor driver: Superior Electric SS2000MD4-M Slo-Syn PWM driver (OBSOLETE)

• Control software: SNL rf_tuner_loop.st  — phase-based feedback with home reset, motion monitoring, and load angle offset computation

• Phase source: Legacy analog RFP module

### 10.3 Upgraded Controller

The tuner motor controller is being replaced with a modern motion controller. The leading candidate is the Galil DMC-4143 (4-axis), which is supported by the LLRF9's built-in tuner control features and EPICS motor records. Other candidates under investigation include the Domenico/Dunning design and Danh's design.

Upgraded tuner control loop:

1. LLRF9 measures cavity probe phase relative to station reference (10 Hz, sub-degree resolution)

2. Python coordinator processes phase data and computes motor commands

3. Motor commands sent via EPICS motor records to motion controller

4. Motion controller drives stepper motors

LLRF9 tuner support: The LLRF9 includes built-in tuner control PVs (per cavity):

• LLRF:TUNER:Cn:GAIN_P — proportional gain

• LLRF:TUNER:Cn:GAIN_I — integral gain

• LLRF:TUNER:Cn:OFFSET — detuning phase offset

• LLRF:TUNER:Cn:CLOSE — loop state (open/closed)

• LLRF:TUNER:Cn:MINFWD — minimum forward power threshold

### 10.4 Load Angle Offset Loop

The load angle offset loop is a supervisory function that balances gap voltage across all 4 cavities by adjusting the individual tuner phase setpoints. In the legacy system this was embedded in rf_tuner_loop.st; in the upgrade it becomes a separate Python module (eg: load_angle_controller.py) that:

1. Reads all 4 cavity probe amplitudes from LLRF9

2. Computes amplitude imbalance

3. Adjusts individual tuner phase offset PVs to redistribute power

### 10.5 Tuner Interfaces

• LLRF9 Unit 1: Provides 10 Hz phase measurements for all 4 cavities (BRD1: Cav 1,2; BRD2: Cav 3,4)

• Motion Controller: EPICS motor records via Ethernet

• Stepper Motors: Power and encoder cables to tunnel (existing field cabling)

• Python Coordinator: Tuner manager module processes phase data, commands motor moves, manages load angle

### 10.6 Risk Note

The tuner motor controller is identified as the hardest-to-prove subsystem. Previous attempts with candidate controllers have had reliability issues. The mitigation plan is to test the chosen controller on the SPEAR3 booster tuners first before committing to storage ring installation.

## 11. Subsystem 7: Waveform Buffer System

────────────────────────────────────────────────────────────────────────────────

│ Detailed reference: llrf/architecture/WaveformBuffersforLLRFUpgrade.docx

### 11.1 Purpose

The Waveform Buffer System is a new signal conditioning and monitoring chassis that extends the LLRF9's RF monitoring capabilities and adds dedicated HVPS signal monitoring. It serves three functions:

1. Extended RF signal monitoring — 8 RF channels with circular waveform buffers for pre-fault capture; monitors the 6 RF signals not assigned to the two LLRF9 units

2. HVPS signal monitoring — 4 channels (voltage, current, 2 inductor voltages) with circular buffers

3. Enhanced klystron collector power protection — computes DC power minus RF power (vs. legacy forward power proxy) and triggers a hardware trip if collector power exceeds the limit

### 11.2 Channel Configuration

RF Channels (8):

The two LLRF9 units together cover 18 of the 24 monitored RF signals (see Section 5.3). The 6 remaining signals are routed to the Waveform Buffer RF inputs:

| Channel | Signal # | Signal | Purpose |
| --- | --- | --- | --- |
| 1 | 3 | Circulator Load Forward Power | Monitors power dissipated in circulator load; cross-check of circulator isolation |
| 2 | 4 | Circulator Load Reflected Power | Indicates circulator load match quality |
| 3 | 6 | Station Reference Power | Monitors 476 MHz reference level; loss of reference triggers fault |
| 4 | 8 | Waveguide Load 1 Reflected Power | Monitors WG Load 1 match quality |
| 5 | 16 | Waveguide Load 2 Reflected Power | Monitors WG Load 2 match quality |
| 6 | 24 | Waveguide Load 3 Reflected Power | Monitors WG Load 3 match quality |
| 7–8 | — | (Spare) | Reserved for future use |

HVPS Channels (4):

| Channel | Signal | Conditioning | Purpose |
| --- | --- | --- | --- |
| 1 | HVPS Voltage | Voltage divider | Regulation monitoring |
| 2 | HVPS Current | Current transformer | Load monitoring |
| 3 | Inductor 1 Voltage | Voltage divider | Firing circuit health |
| 4 | Inductor 2 Voltage | Voltage divider | Firing circuit health |

### 11.3 Key Features

• Circular buffers: Continuous acquisition at kHz rate; freeze on fault trigger for pre/post-fault capture (~100 ms pre-fault data)

• Analog comparator trips: Hardware-based threshold detection on each channel; comparator outputs feed Interface Chassis

• Enhanced collector power protection algorithm (improvement over legacy forward power proxy):

DC_Power = HVPS_Voltage × HVPS_Current

RF_Power = Klystron_Forward_Power

Collector_Power = DC_Power - RF_Power

IF Collector_Power > Collector_Limit THEN Trip

This provides direct collector power calculation vs. the legacy system's forward power proxy method.

• EPICS interface: Waveform readout, threshold configuration, trip status

### 11.4 Interfaces

• RF signal inputs: RF detectors on cavity forward/reflected couplers (signal conditioned)

• HVPS signal inputs: Voltage dividers and current transformers from HVPS

• Interface Chassis: Comparator trip outputs (digital) feed into Interface Chassis permit logic

• Python Coordinator (EPICS): Waveform readout, configuration, collector power trend monitoring

## 12. Subsystem 8: Arc Detection System

────────────────────────────────────────────────────────────────────────────────

### 12.1 Purpose

The arc detection system is a new subsystem that provides optical monitoring of the RF cavity windows and klystron for electrical arcing. The legacy system had a non-functional arc detection capability that is being replaced entirely.

### 12.2 Technology

Microstep-MIS Waveguide Arc Detectors: Optical sensors that detect the light flash produced by an electrical arc inside a waveguide or cavity viewport. These are commercial off-the-shelf devices providing:

• Optical fiber sensors mounted at cavity window viewports and klystron window

• Controller unit with dry-contact relay outputs per channel

• Response time on the order of microseconds

• Configurable sensitivity thresholds

### 12.3 Installation

There are 5 sensors total:

• 8 sensors mounted on cavity window viewports (one per cavity: Cav A, B, C, D)

• 1 sensor mounted on the klystron window

Mechanical mounting requires custom adapters for the existing CF flange viewports (see llrf/arcDetector/Mechanical/Reference/ for viewport specifications). Sensor fibers run to the Microstep-MIS controller unit located in B132.

### 12.4 Signal Path and Design

The Arc Detection Chassis routes both paths to the Interface Chassis — the fast trip permit on one wire, and the 5-bit fired-detector identification on five separate status lines. This centralizes all arc signal handling in the Interface Chassis, consistent with its role as the hub for all protection signals.

Figure 4: The Arc Detection Chassis routes both paths to the Interface Chassis — the fast trip permit on one wire, and the 5-bit fired-detector identification on five separate status lines. This centralizes all arc signal handling in the Interface Chassis, consistent with its role as the hub for all protection signals.

Fast trip path: The 5 relay outputs are OR-ed in the Arc Detection Chassis into a single active-high permit signal. Any arc event immediately de-asserts it, entering the Interface Chassis AND-gate alongside all other permits. This path is purely hardware with no encoding latency.

Diagnostic identification path: The same 5 relay outputs simultaneously set individual bits in a 5-bit latching register inside the Arc Detection Chassis. All 5 bits are wired as separate status inputs to the Interface Chassis. The Interface Chassis includes these bits in its fault status word, which the Kly MPS PLC reads to determine which sensor fired. The latch holds state until explicitly reset, preserving identity through the fault response sequence.

Routing both paths to the Interface Chassis is cleaner than wiring the diagnostic path directly to the Kly MPS: all arc-related signals are centralized in one place, no extra wiring runs to the MPS PLC are needed, and the Interface Chassis first-fault register can incorporate arc identification alongside all other fault sources.

### 12.5 Permit Logic

• The single arc permit output is active-high (permit present = no arc); any arc event drives it low — consistent with fail-safe convention throughout the Interface Chassis

• The Interface Chassis removes the RF permit immediately upon any arc event

• The 5 identification bits in the Interface Chassis fault status word indicate which sensor(s) fired; the latch holds until reset

• If multiple sensors fire simultaneously, all corresponding bits are set

• Reset of the latch requires an explicit EPICS command via the Kly MPS after the fault is acknowledged

### 12.6 Interfaces

• Arc Detection Chassis → Interface Chassis: 1 permit signal (OR of all 5 sensors, fail-safe active-high) + 5 latched status bits (one per sensor)

• Interface Chassis → Kly MPS PLC: Arc permit state and 5-bit fired-detector ID included in the Interface Chassis fault status word

• Kly MPS / EPICS: Per-sensor trip status PVs, fired-detector identification, event count, latch reset command

• Python Coordinator: Reads arc fault status for operator displays and state machine fault handling

## 13. Subsystem 9: Klystron Cathode Heater

────────────────────────────────────────────────────────────────────────────────

### 13.1 Legacy System

The current klystron heater control system is inherited from the PEP-II era and consists of a motor-driven variac for voltage adjustment, feeding a Kepko 5V/20A power supply (PS-2).

Current System Architecture:

• Input Power: 120VAC, Phase C (from Hoffman box wiring)

• Control Method: Motor-driven variac for voltage adjustment

• Power Supply: Kepko 5V/20A (PS-2) with transformer isolation for HV safety

• Output: 5V/20A maximum (~100W typical operation)

• Control Interface: Manual or slow automatic adjustment

• EPICS Integration: Limited monitoring capabilities

System Limitations:

• Slow Response: Motor-driven variac requires seconds to minutes for adjustment

• Limited Precision: Mechanical variac resolution ~1-2% of full scale

• Aging Components: 25+ year old system with increasing failure rates

• Manual Operation: Requires operator intervention for power adjustments

• Poor EPICS Integration: Limited monitoring and control capabilities

• No Automated Sequences: Manual warm-up and cool-down procedures

• Minimal Diagnostics: No real-time power, voltage, or current monitoring

• Maintenance Issues: Mechanical wear, obsolete parts, calibration drift

### 13.2 Upgraded System — SCR-Based Control

The upgrade replaces the variac/motor system with a solid-state SCR controller:

| Parameter | Legacy | Upgraded |
| --- | --- | --- |
| Control method | Motor-driven variac | SCR zero-crossing switching |
| Response time | Seconds–minutes | <100 ms |
| Voltage regulation | ±1%? | ±0.1% |
| Reliability | Mechanical wear | Solid-state (>50,000 hr MTBF) |
| EPICS integration | Limited | Full (automated sequences) |

Upgraded system architecture:

Figure 5: **Upgraded system architecture**

Key design features:

• Zero-crossing SCR switching minimizes harmonic generation

• LC low-pass filter (fc ≈ 159 Hz) attenuates switching harmonics

• True RMS voltage/current monitoring (AD637 RMS-to-DC converters)

• Automated warm-up, standby, and cool-down sequences

### 13.3 Interfaces

• Python Coordinator (EPICS): Setpoint control, RMS readbacks, status, automated sequences

• HVPS coordination: Heater must be at operating temperature before HVPS enable; HVPS must be off before heater cooldown

• MPS: Heater "ready" status required for RF permit

## 14. Subsystem 10: Control Software

────────────────────────────────────────────────────────────────────────────────

│ Detailed reference: llrf/epicsSequences/legacyLLRF, Docs_JS/LLRFOperation_jims.docx

### 14.1 Legacy Software

The legacy control software consists of SNL (State Notation Language) programs running on VxWorks RTOS in the VXI crate processor:

| Program | Lines | Function |
| --- | --- | --- |
| rf_states.st | 2,227 | Master state machine (OFF/PARK/TUNE/ON_CW), turn-on, fault handling |
| rf_dac_loop.st | 290 | DAC control loop (gap voltage regulation) |
| rf_hvps_loop.st | 343 | HVPS control loop (drive power regulation) |
| rf_tuner_loop.st | 555 | Tuner control (×4 cavities, phase-based, load angle) |
| rf_calib.st | 2,800+ | Calibration (analog offset nulling, coefficient calibration, ~20 min) |
| rf_msgs.st | 352 | Message logging, HVPS faults, TAXI error detection |

### 14.2 Upgraded Software — Python/EPICS Coordinator

The upgrade replaces all SNL code with a Python/PyEPICS coordinator application that operates as a supervisory control layer at ~1 Hz. It is NOT in the fast safety path.

### 14.3 Key Design Principles

• Separation of concerns: Each module has a single responsibility

• Hardware abstraction: All hardware interfaces go through the EPICS layer

• Configuration-driven: All operational parameters in configuration files

• Fault tolerance: Graceful degradation when subsystems are unavailable

• Diagnostics: Structured event logging, LLRF9 + waveform buffer readout, system performance metrics, and logging

• Testability: Mock interfaces for all hardware dependencies

• Safety delegation: Hardware safety handled by Interface Chassis; Python handles sequencing and coordination only

## 15. Control Loop Architecture

────────────────────────────────────────────────────────────────────────────────

This section maps the control loops in the system, identifying which hardware subsystem closes each loop, the loop bandwidth, and the upgrade path.

### 15.1 Fast RF Feedback (Direct Loop)

| Aspect | Legacy | Upgraded |
| --- | --- | --- |
| Implementation | Analog I/Q in RFP module | LLRF9 FPGA: digital proportional + integral |
| Loop delay | ~kHz bandwidth | 270 ns |
| Controlled variable | Cavity field amplitude and phase | Same (vector sum of Cav 1+2 on BRD1) |
| Actuator | DAC driving klystron input | LLRF9 DAC → drive amplifier → klystron |
| Safety | Analog limits | LLRF9 RF interlocks (overvoltage, 9 per board) + baseband window comparators (8 per unit) |

This is the fastest loop in the system and is entirely closed inside the LLRF9 FPGA. The Python coordinator does NOT participate.

### 15.2 HVPS Supervisory Loop (~1 Hz)

| Aspect | Legacy | Upgraded |
| --- | --- | --- |
| Implementation | SNL rf_hvps_loop.st on VxWorks | Python hvps_controller.py + CompactLogix PLC |
| Bandwidth | ~1 Hz | ~1 Hz |
| Measurement | Drive power from analog module | Klystron forward power from LLRF9 Unit 1, BRD3 |
| Setpoint | HVPS voltage via field bus | HVPS voltage via EPICS to CompactLogix PLC |
| Purpose | Maintain drive power within target range by adjusting klystron voltage | Same |

The PLC handles low-level voltage regulation; the Python coordinator provides supervisory setpoint management.

### 15.3 Tuner Control Loops (×4, ~1 Hz)

| Aspect | Legacy | Upgraded |
| --- | --- | --- |
| Implementation | SNL rf_tuner_loop.st | LLRF9 phase data + Python tuner_manager.py + motor controller |
| Measurement | Cavity probe phase from analog module | Cavity probe phase from LLRF9 (10 Hz, sub-degree) |
| Actuator | HSTP1 stepper module + Slo-Syn driver | Modern motion controller + stepper driver |
| Phase setpoint | Fixed per cavity + load angle offset | LLRF9 tuner PVs (OFFSET, GAIN_P, GAIN_I) |

### 15.4 Load Angle Offset Loop (~1 Hz)

| Aspect | Legacy | Upgraded |
| --- | --- | --- |
| Implementation | Part of rf_tuner_loop.st | Python load_angle_controller.py |
| Measurement | 4 cavity probe amplitudes | 4 cavity probe amplitudes from LLRF9 |
| Actuator | Adjusts individual tuner phase setpoints | Adjusts LLRF9 tuner offset PVs |
| Purpose | Balance gap voltage across 4 cavities | Same |

### 15.5 Station State Machine

| Aspect | Legacy | Upgraded |
| --- | --- | --- |
| Implementation | SNL rf_states.st (2,227 lines) | Python state_machine.py |
| States | OFF, STANDBY, PARK, TUNE, ON_CW, ON_FM (unused), FAULT | OFF, PARK, TUNE, ON_CW, FAULT |
| Turn-on sequence | Multi-step with manual fast-on values | LLRF9 setpoint profiles (512 pts, 70 μs–37 ms/step) |
| Safety | Software-based fault handling | Hardware safety delegated to Interface Chassis; software handles sequencing |

### 15.6 Calibration

| Aspect | Legacy | Upgraded |
| --- | --- | --- |
| Implementation | SNL rf_calib.st (2,800+ lines, ~20 min) | Python rf_calib.py + LLRF9 built-in digital calibration |
| Scope | Analog offset nulling, coefficient calibration for RFP module | Factory + installation calibration stored in EEPROM |
| Duration | ~20 minutes | Minutes (digital, no analog drift) |

### 15.7 Eliminated Loops

The following legacy loops are eliminated in the upgrade because the LLRF9's digital feedback inherently provides their function:

• Ripple rejection loop — LLRF9 digital feedback inherently rejects power-line ripple

• Comb filter loop (CFM) — Multi-bunch stabilization handled by LLRF9 FPGA

• Gap voltage feedback (GVF) — Cavity field stabilization handled by LLRF9 vector sum feedback

• 4-way branching (DAC loop) — Eliminated; LLRF9 controls all via single vector sum

## 16. Inter-Subsystem Interface Matrix

────────────────────────────────────────────────────────────────────────────────

This matrix summarizes all physical and logical interfaces between subsystems in the upgraded system.

| From \ To | LLRF9 #1 | LLRF9 #2 | HVPS PLC | Kly MPS PLC | Interface Chassis | Waveform Buffer | Arc Detect | Motor Ctrl | Heater Ctrl | Python Coord |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LLRF9 #1 | — | Interlock daisy (LEMO) | — | — | Status (5V digital) | — | — | — | — | EPICS (Ethernet) |
| LLRF9 #2 | Interlock daisy (LEMO) | — | — | — | Status (5V digital) | — | — | — | — | EPICS (Ethernet) |
| HVPS PLC | — | — | — | — | STATUS (fiber), SCR EN (fiber), CROWBAR (fiber) | — | — | — | — | EPICS (Ethernet) |
| KLY MPS PLC | — | — | — | — | Summary, Heartbeat, Reset (digital) | — | — | — | — | EPICS (Ethernet) |
| Interface Chassis | Enable (3.2V) | Enable (3.2V) | SCR EN, CROWBAR (fiber) | Fault status (digital) | — | — | — | — | — | — |
| Waveform Buffer | — | — | — | — | Comparator trips (digital) | — | — | — | — | EPICS (Ethernet) |
| Arc Detect | — | — | — | — | Permit + Latching | — | — | — | — | — |
| Motor Ctrl | — | — | — | — | — | — | — | — | — | EPICS (Ethernet) |
| Heater Ctrl | — | — | — | — | — | — | — | — | — | EPICS (Ethernet) |
| Python Coord | EPICS (Ethernet) | EPICS (Ethernet) | EPICS (Ethernet) | EPICS (Ethernet) | Monitoring only | EPICS (Ethernet) | — | EPICS (Ethernet) | EPICS (Ethernet) | — |
| External (SPEAR MPS, Orbit) | — | — | — | — | 24V permits | — | — | — | — | — |

### Interface Signal Types Summary

| Signal Type | Examples | Medium | Speed |
| --- | --- | --- | --- |
| EPICS Channel Access | PV reads/writes, setpoints, readbacks | Ethernet (TCP/UDP) | ~1–10 Hz |
| Hardware interlock (digital) | LLRF9 status, Kly MPS summary, comparator trips | Optocoupled wire | <1 μs |
| Fiber-optic interlock | HVPS SCR ENABLE, CROWBAR, STATUS | HFBR fiber | <1 μs |
| RF signals | Cavity probes, forward, reflected | 50 Ω coax (SMA) | 476 MHz analog |
| Motor control | Step/direction pulses, encoder signals | Shielded cable | kHz pulse |
| Interlock daisy-chain | LLRF9 Unit 1 ↔ Unit 2 | LEMO coax | <1 μs |
| Dry contacts | Arc detection relay outputs | Wire | DC / relay |

### 16.2 Open Interface Questions

| # | Question | Subsystems Affected | Impact |
| --- | --- | --- | --- |
| 1 | Does the Heater Controller require a fast interlock connection to the Kly MPS PLC or Interface Chassis? The current design routes Heater Ctrl only through EPICS (slow, ~1 Hz). If a heater overcurrent or overvoltage fault requires sub-second trip action on the klystron, a hardware interlock path would be needed. | Heater Ctrl, Interface Chassis, Kly MPS PLC | If yes: add Heater Ctrl permit input to Interface Chassis and/or digital I/O line to Kly MPS PLC; update interface matrix row accordingly. |

## 17. Protection Chain and Interlock Architecture

────────────────────────────────────────────────────────────────────────────────

### 17.1 Protection Philosophy

The upgraded system implements a layered protection architecture:

1. Layer 1 — LLRF9 FPGA interlocks (<1 μs): RF overvoltage, baseband window comparators, internal housekeeping. These are the fastest protection and act within the LLRF9 itself to disable the DAC output.

2. Layer 2 — Interface Chassis hardware (<1 μs from input change): Aggregates all permit signals and coordinates system-wide protection. All signals are optocoupler-isolated or fiber-optic. First-fault detection identifies the initiating event.

3. Layer 3 — KLY MPS PLC (~ms): Aggregates RF interlock status with external safety systems (SPEAR MPS, orbit interlock). Provides reset signal to Interface Chassis.

4. Layer 4 — Python Coordinator (~1 s): Supervisory monitoring, logging, fault analysis, and recovery sequencing. NOT in the fast safety path.

### 17.2 Fault Propagation Example — RF Arc Event

Figure 6: 17.2 Fault Propagation Example — RF Arc Event

### 17.3 Fail-Safe Design

All critical subsystems are designed to fail safe:

| Subsystem | Failure Mode | Safe State |
| --- | --- | --- |
| Interface Chassis power loss | All outputs de-energize | LLRF9 disabled, HVPS SCR disabled (K4/Ross controlled by separate PPS interface box) |
| LLRF9 power loss | Interlock status goes low | Interface Chassis removes permits |
| HVPS PLC failure | STATUS signal lost | Interface Chassis removes permits |
| MPS PLC failure | Heartbeat lost | Interface Chassis removes permits |
| Python coordinator crash | No effect on safety | Hardware protection continues; supervisory control paused |
| Ethernet failure | No effect on safety | Hardware protection continues; supervisory control paused |

## 18. Communication Architecture

────────────────────────────────────────────────────────────────────────────────

### 18.1 EPICS Channel Access Network

All supervisory communication in the upgraded system uses EPICS Channel Access over Ethernet:

| Device | IP Address | IOC Type | Key PV Prefix | Update Rate |
| --- | --- | --- | --- | --- |
| LLRF9 Unit 1 | TBD | Built-in Linux IOC | LLRF1: | 10 Hz (scalars) |
| LLRF9 Unit 2 | TBD | Built-in Linux IOC | LLRF2: | 10 Hz (scalars) |
| HVPS CompactLogix | TBD | External EPICS gateway | SRF1:HVPS: | ~1 Hz |
| KLY MPS ControlLogix | TBD | External EPICS gateway | SRF1:MPS: | ~1 Hz |
| Motion Controller | TBD | EPICS motor record IOC | SRF1:MTR: | On demand |
| Waveform Buffer | TBD | Dedicated IOC | SRF1:WFBUF: | ~1 Hz / event |
| Heater Controller | TBD | Dedicated IOC | SRF1:HTR: | 10 Hz |
| Python Coordinator | TBD | caproto / PyEPICS client | Various | ~1 Hz |

### 18.2 Non-EPICS Communication Paths

| Path | Type | Purpose |
| --- | --- | --- |
| Interface Chassis ↔ all hardware | Hardwired digital/fiber | Safety interlocks (no network dependency) |
| LLRF9 Unit 1 ↔ Unit 2 | LEMO interlock daisy-chain | Fast interlock propagation |
| PPS ↔ PPS Interface Box | Hardwired (GOB12-88PNE) | Personnel safety (no network dependency) |
| Arc Detection → Interface Chassis | Dry contact relay | Arc interlock (no network dependency) |

Critical design principle: All safety-critical communication paths are hardwired. No safety function depends on the Ethernet network or EPICS Channel Access. Network loss pauses supervisory control but does not compromise protection.

## 19. Implementation Phases and Risk Summary

────────────────────────────────────────────────────────────────────────────────

### 19.1 Implementation Phases

│ Critical Constraint: The SPEAR3 LLRF upgrade has extremely limited flexibility for integration testing since bringing systems online directly impacts SPEAR operations. This severely constrains the available testing window and requires a fundamentally different approach than typical development projects.

Key Implications:

• Maximum standalone subsystem testing must occur before installation

• Integration testing must be incremental and carefully planned

• Some subsystems can be brought online for limited testing without full operational impact

• Full system integration testing window is minimal

#### Phase 1: Maximum Standalone Development

• Software Framework: Develop with simulated interfaces where live hardware unavailable

• Tuner Testing: Test Galil controller with Booster RF cavity

• LLRF9 Installation: Install both units, connect RF signals, and bring them online

• Kly MPS Online: Bring Kly MPS online for EPICS interface development

• HVPS PLC Online: Enables EPICS software development

• Waveform Buffer Assembly: Complete assembly and standalone testing

• Interface Chassis Design/Fab: Complete design and fabrication

• HVPS Controller: Design, fabrication, and installation

• Arc Detector: Design and fabrication

• PPS Interface Box: Design, fabrication, and commissioning

• Test Stand 18: Upgrade to work with the upgraded HVPS controller

#### Phase 2: Incremental Subsystem Integration

• Tuner Test with SPEAR Cavity

• HVPS Integration: Integrate HVPS PLC with Python/EPICS (test at test stand T18)

• Kly MPS Integration: Integrate Kly MPS with LLRF9, waveform buffer, arc detector, HVPS PLC, interface chassis, and software coordinator with existing RF signals

#### Phase 3: Critical Path Integration

• LLRF9 Integration — Basic RF Processing: Verify vector sum, feedback loops, online calibration

• EPICS Integration: Configure IOC, verify all PV interfaces

• End-to-End Interlock Testing: Verify complete interlock chain (limited RF power)

#### Phase 4: Full Power Commissioning

• Incremental Power Ramp: Gradually increase power while monitoring all systems

• Performance Validation: Verify all success criteria are met

• Operator Training: Train operators on new system

• Documentation: Complete commissioning report

### 19.2 Status

| Item | Status | Notes |
| --- | --- | --- |
| LLRF9 (4 units) | Complete | — |
| Kly MPS PLC modules | Complete | — |
| HVPS PLC modules | Complete (HVPS1, HVPS2, B44 Test Stand) | — |
| Enerpro FCOG1200 boards | Needed | 5 boards |
| Arc detection (Microstep-MIS) | Needed | 10 sensors and 5 process, + 1 sensor & 1 process for spare |
| Waveform Buffer System | Needed | Designed |
| Interface Chassis | Needed | - |
| Heater SCR controller | Needed | - |
| PPS Interface Box | Needed |  |
| Software | Needed |  |

### 19.3 Key Technical Risks

| Risk | Severity | Status | Mitigation |
| --- | --- | --- | --- |
| Tuner motor controller reliability | High | In progress | Test on booster tuners first; evaluate multiple candidates |
| HVPS PLC code migration (SLC-500 → CompactLogix) | High | In progress | Reverse-engineer legacy code; build Test Stand 18 (B44) |
| PPS Interface approval delays | High | In progress | Engage protection managers early; provide complete documentation |
| Interface Chassis logic design (LLRF9/HVPS feedback loop) | High | Not started | Design and simulate before fabrication; careful sequencing logic |
| Waveform Buffer System (new custom hardware) | Medium | Not started | Staged development: PCB design → assembly → testing → integration |
| Python/EPICS Coordinator software | Critical | Not started | Largest untouched software scope: state machine, fault handling, auto-recovery sequences, operator interface, and integration with all subsystem IOCs. Must be developed in parallel with hardware integration; delays directly block Phase 3 and Phase 4. Mitigation: begin framework and simulated-interface development immediately; define PV naming conventions and state machine design before hardware is ready. |
| Kly MPS PLC software integration | High | Not started | MPS hardware is assembled and tested standalone, but EPICS IOC development and integration with Interface Chassis fault status, first-fault logic, and reset sequencing has not begun. Mitigation: start IOC development using simulated Interface Chassis I/O; define fault status word bit assignments early. |
| Arc Detection Chassis firmware/logic | Low | Not started | OR-gate and 5-bit latch logic for Arc Detection Chassis not yet designed. Chassis is a prerequisite for arc interlock integration. Mitigation: design is straightforward (combinational logic + latches); prioritize as part of Interface Chassis design phase. |
| Communication latency (Ethernet/EPICS for ~1 Hz control) | Low | Validated | Proven in LLRF9 prototype commissioning |

### 19.4 Success Criteria

| Metric | Legacy Performance | Target |
| --- | --- | --- |
| Amplitude stability | <0.1% | Same or better |
| Phase stability | <0.1 deg | Same or better |
| Tuner resolution | ~0.002–0.003 mm/microstep | Improved (up to 256 microsteps/step) |
| Control loop response | ~1 second | Same or better |
| Uptime | >99%? | Same or better |
| Fault diagnostics | Limited fault file capture | 16k-sample waveform + circular buffer + first-fault |

## 20. Appendix: Source Document Index

────────────────────────────────────────────────────────────────────────────────

### LLRF Source Material

| Path | Content |
| --- | --- |
| llrf/legacyLLRF/rf_states.st | Legacy master state machine (2,227 lines) |
| llrf/legacyLLRF/rf_dac_loop.st | Legacy DAC control loop (290 lines) |
| llrf/legacyLLRF/rf_hvps_loop.st | Legacy HVPS control loop (343 lines) |
| llrf/legacyLLRF/rf_tuner_loop.st | Legacy tuner control loop (555 lines) |
| llrf/legacyLLRF/rf_calib.st | Legacy calibration system (2,800+ lines) |
| llrf/legacyLLRF/rf_msgs.st | Legacy message logging (352 lines) |
| llrf/architecture/llrfInterfaceChassis.docx | Interface Chassis specification |
| llrf/architecture/WaveformBuffersforLLRFUpgrade.docx | Waveform Buffer System design |
| llrf/architecture/arcDetectorHardwareOptions.docx | Arc detection hardware selection |
| llrf/documentation/LLRFOperation_jims.docx | Jim's SPEAR3 RF Station Operation Guide |
| llrf/documentation/LLRFUpgradeTaskListRev3.docx | Full project scope, procurement, costs |
| llrf/tuners/galil/ | Galil DMC-4143 firmware and test notes |
| llrf/driveAmp/KAW2051M12*.pdf | Drive amplifier datasheet |
| llrf/arcDetector/ | Arc detector product sheets and mechanical references |

### HVPS Source Material

| Path | Content |
| --- | --- |
| hvps/architecture/originalDocuments/slac-pub-7591.pdf | Original HVPS design publication |
| hvps/architecture/originalDocuments/ps3413600102.pdf | HVPS power section specifications |
| hvps/controls/enerpro/ | Enerpro SCR gate driver documentation and schematics |
| hvps/documentation/plc/ | SLC-500 PLC code, labels, and analysis |
| hvps/documentation/schematics/ | HVPS electrical schematics |
| hvps/documentation/wiringDiagrams/ | Wiring diagrams (Hoffman Box, HVPS) |
| hvps/documentation/procedures/ | Maintenance and safety procedures |
| hvps/maintenance/ | Stack installation checklists, phase tank maintenance |

### PPS Source Material

| Path | Content |
| --- | --- |
| pps/diagrams/00_SYSTEM_OVERVIEW.md | PPS current vs. upgrade architecture overview |
| pps/diagrams/01-08_*.md | Detailed analysis of each PPS-related schematic |
| pps/MSG from Jim Sebek to Faya about PPS.md | 2022 PPS concerns and upgrade drivers |
| pps/HoffmanBoxPPSWiring.docx | Detailed PPS wiring analysis |
| pps/*.pdf | Original PPS schematics and wiring diagrams |